SA WG2 Meeting #SA2-154AHE	S2-2300702r01

January 16 – 20 2023, Electronic Meeting

Title:	[Draft] Reply LS on mandating the NR Tx Profiles in broadcast mode and groupcast mode communication

Response to:	LS on mandating the NR Tx Profiles in broadcast mode and groupcast mode communication (S2-2300019/ C1-226994)

Release:	Rel-17

Work Item:	5G_ProSe

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Wen Wang

E-mail Address:	wen.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 0174 S2-2300700

- Overall description

SA2 thanks CT1 for their LS on mandating the NR Tx Profiles in broadcast mode and groupcast mode communication (S2-2300019/C1-226994).

SA2 would like to provide the following answers to the questions from CT1.

Q1) Can SA2 confirm that all the ProSe services that the UE is interested in shall have the mapped NR Tx Profiles based on the ProSe identifiers to NR Tx profile mapping rules in broadcast mode and groupcast mode communication?

[SA2 answer] NO, but SA2 confirms that all the ProSe services will have the mapped NR Tx profiles either based on the explicit ProSe identifiers to NR Tx profile mapping rules or the "catch all" NR Tx Profiles. The "catch all" NR Tx Profiles are set in the mapping rules as an entry with the lowest priority which are used for the ProSe services that do not have the explicit ProSe identifiers to NR Tx profile mapping rules.

Besides above mentioned "catch all" NR Tx Profiles, SA2 considers that other mapping rules in the bullet 5) of TS23.304 clause 5.1.3.1 also have the "catch all" entry with the lowest priority used for the ProSe services that do not have explicit mapping policy/parameters in the agreed CR (0174) as attached. SA2 further considers that the design of "catch all" entry is determined by stage 3.

Q2) If the answer to Q1 is no, whether and how does the UE enable the PC5 DRX operation and the ProSe service without the mapped NR Tx Profile?

[SA2 answer] The UE will use the NR Tx profile in "catch all" entry in the NR Tx profile mapping rules as explained above in Q1..

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above feedback from SA2 into account for further work.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#155	20 – 24 February 2023	Athens, GREECE

3GPP TSG SA2#156E	April 17 – 21, 2023	E-meeting