3GPP TSG-WG SA2 Meeting #154AHe  	S2-2300728

# Elbonia, January 16 - 20, 2023											 							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3968 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GS Assistance for Application AI/ML operation: General clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR captures the general architecture principles of the Rel-18 AIMLsys feature which describes how 5G system provides the assistance to support the Application layer AI/ML operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General clause with description of the general architecture principles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | General archirecture principles are not captured in description of the item |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.X (new), 5.X.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

* * * Next Change (All New) * * *

## 5.X	5GS Assistance for Application AI/ML operations

### 5.X.1	General

This clause presents an overview on how the 5G System provides assistance to support application AI/ML operations based on service requirements specified in clause 6.40 of TS 22.261 [2].

AF requests to the 5G System in the context of 5GS assistance to application AI/ML operation may need to be authorized by the 5GC.

NOTE:

In this Release, assistance to application AI/ML operations is not supported to roaming UEs.

UPF may perform traffic detection for AI/ML traffic as defined in clause 5.8.2.4 .

Application AI/ML decisions and their internal operation logic reside at the AF and UE application client and is out of scope of 3GPP.

Based on application logic, it is the application decision whether to request assistance from 5GC e.g. for the purpose of selection of UEs that participate in certain AI/ML operation.

.

In Rel-18, an Application AI/ML operation is conducted within a single slice.

Editor’s note: It is FFS with additional clauses to summarize the other respective 5GC assistance functionalities for the application AI/ML operations and the references to the relevant normative clauses once the normative work becomes stable.  The respective 5GC assistance functionalities are:

- Network resource performance and monitoring

- Information exposure to authorized 3rd party

- External parameter provisioning

- Planned group data transfer support with QoS for application traffic

- QoS and Policy requirements

- Member Selection Assistance functionality

- Network data analytics support

* * * End of Changes * * *