3GPP TSG-SA2 Meeting #154AH-E	S2-2300744

# Elbonia, January 16 –20, 2023																(Revision of S2-xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0651 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Relative Proximity Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following text is extracted from TR 23.700-81 cl. 8.9 and needs to be captured as normative text: “- Analytics ID 'Relative Proximity' provides support for finer granularity of location information at NWDAF via relative proximity information of nearby UEs. The output analytics contain relative proximity and Time to Collision information.” (…) “- The NWDAF may collect data from OAM and DCAF, as in previous Releases, to derive analytics with fine granularity. New input data consists of per UE speed and orientation from OAM and UEs fulfilling a proximity criterion from DCAF, respectively.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative adoption of NWDAF support for Relative Proximity analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation of FS_eNA_Ph3 conclusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X, 6.X.1, 6.X.2, 6.X.3, 6.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[Y1]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[Y2]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[Y3]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

* * * Next Change (all new text) * * *

## 6.X	Relative Proximity Analytics

### 6.X.1	General

Relative Proximity Analytics among UEs provided by NWDAF can be used to assist a consumer NF to more accurately localize a cluster (or a set) of UEs via provisioning statistics or prediction information related to their relative proximity. This analytics type may help the consumer improve the location estimation accuracy of a UE by using proximity information from nearby UEs, or it may help the consumer identify UEs in the vicinity of another UE. Relative proximity information can also be leveraged by NWDAF to provide location information with finer granularity than TA/cell.

The NWDAF provides relativity proximity analytics to an AF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Relative Proximity";

-	Target of Analytics Reporting: a group of UEs or any UE;

-	Analytics Filter Information:

-	optionally, S-NSSAI and/or DNN

-	an optional Area of Interest;

-	an optional individual or set of direction(s) of interest;

-	an optional number of UEs to be accounted for relative proximity (i.e. the number of UEs for which one UE reports proximity information);

- 	an optional set of attributes to be accounted for relative proximity: velocity, average speed, orientation, mobility trajectory (i.e. additional information that can be provided in addition to distance between two UEs);

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.X.3;

-	an optional preferred level of accuracy of the analytics;

-	optionally, "maximum number of objects" indicating the maximum number of UEs

-	an Analytics target period indicating the time period over which the statistics or predictions are requested.

### 6.X.2	Input data

The detailed input data collected by the NWDAF for relative proximity analytics is listed in Table 6.X.2-1, Table 6.X.2-2 and Table 6.X.2-3.

Table 6.X.2-1: OAM input data for relative proximity analytics

| Information | Source | Description |
| --- | --- | --- |
| Per UE information | OAM |  |
| > Speed |  | UE Speed |
| > Orientation |  | UE Orientation |

NOTE 1:	To collect the input data in Table 6.X.2-1, NWDAF can invoke TraceJob as described in clause 4.3.30 of TS 28.622 [Y1] using Generic Provisioning Management Service as described in clause 11.1 of TS 28.532 [6] to get Sensor Information as part of MDT data collection mechanism. The UE sensor information includes Barometric pressure, UE speed and UE orientation as described in clause 5.10.29 of TS 32.422 [Y2].

Table 6.X.2-2: Proximity related input data collected via DCAF/NEF

| Information | Source | Description |
| --- | --- | --- |
| Proximity Attribute | DCAF / NEF | Characterise a set of UEs in relative proximity. |
| > Number of UEs |  | Total number of UEs that fulfil a proximity criterion. |
| > Timestamp |  | A time stamp of time that the proximity attribute derived. |
| > Application ID(s) |  | Identifying the applications(s) providing this information. |
| > List of UE IDs |  | UE IDs that fulfil a proximity criterion. |
| > Other attribute(s) |  | Other attributes the set of UEs in proximity fulfil as a collective attribute (e.g. destination, route, average speed, time of arrival). |
| > Confidence |  | Confidence on relative proximity data. |

NOTE 2:	The list of UE IDs that fulfil a proximity criterion can be determined by DCAF based on the input information collected from the UE application. The UE does not need to determine or provide proximity information related to other UEs.

Table 6.X.2-3: Proximity related input data from 5GC/AF

| Information | Source | Description |
| --- | --- | --- |
| UE ID(s) | AMF | SUPI(s) of individual UE(s). |
| UE location | AMF, GMLC | Absolute location of an UE. |
| UE location accuracy | GMLC | Accuracy of the location estimation. |
| Collision risk distance | AF | Derived from the UE connected physical object dimensions e.g. length, width, height. |
| UE CM state | AMF | UE connection management state (e.g. CM-IDLE). |
| UE heading | AF | UE moving direction. |
| > Absolute heading |  | Heading of the UE movement with respect to the true north. |
| > Relative heading |  | Heading of the UE movement with respect to another UE. |
| UE trajectory | AF | Timestamped UE positions. |
| >UE location |  | Geographical area where the UE is located. |
| >Timestamp |  | Time stamp for the UE location. |

### 6.X.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Relative proximity statistics information is defined in Table 6.X.3-1.

-	Relative proximity predictions information is defined in Table 6.X.3-2.

Table 6.X.3-1: Relative proximity statistics

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies a group of UEs or a set of UEs, e.g. internal group ID, external group ID, list of SUPIs, list of GPSIs or other external UE IDs. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributes (NOTE 1) | Observed proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in cl. 6.X.1 |
| >> relative proximity information | Observed proximity information. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| Time To Collision (TTC) information (NOTE 1) |  |
| > Time To Collision | Time until a collision with another UE happens. |
| > Accuracy | Accuracy of TTC (dependent on the UE location accuracy). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.X.3-2: Relative proximity predictions

| Information | Description |
| --- | --- |
| UE group ID or set of UE IDs | Identifies a group of UEs or a set of UEs, e.g. internal group ID, external group ID, list of SUPIs, list of GPSIs or other external UE IDs. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > UE proximity attributess ((NOTE 1)) | Predicted proximity data. This includes distance between UEs in the group/list and other proximity data as mentioned in cl. 6.X.1. |
| >> relative proximity information | Predicted proximity information. |
| >> Confidence | Confidence of this prediction. |
| >> Sampling Ratio | Percentage of UEs accounted based on proximity criteria. |
| Time To Collision (TTC) information (NOTE 1) |  |
| > Time To Collision | Time until a collision with another UE happens. |
| > Confidence | Confidence of the prediction. |
| > Accuracy | Accuracy of TTC (dependent on both the UE location accuracy and confidence of the prediction). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

### 6.X.4	Procedures

Figure 6.X.4-1: Procedure for NWDAF providing relative proximity analytics

Figure 6.X.4-1 shows the procedure for NWDAF to derive relative proximity analytics. The steps are described as follows:

1.	The Consumer NF/AF sends a request to the NWDAF for analytics related to relative proximity, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service.

The Analytics ID is set to "Relative Proximity". The target for analytics reporting can be a single UE, group of UEs or any UE. Analytic filters may be provided as shown in clause 6.X.1.

The Consumer NF can request statistics or predictions or both for a given Analytics target period.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve relevant information to proximity analytics. The NWDAF may collect MDT input data per individual UE from OAM as defined in Table 6.X.2-1.

6-7.	For relative proximity information, if the request is authorized, and in order to provide the requested analytics, NWDAF may follow the UE Input Data Collection Procedure via the DCAF. DCAF may collect proximity related input data directly from the UE Application, and determine a list of UEs fulfilling certain proximity criterion.

NOTE 1:	The UE data collection procedure should be based on clause 6.2.8.

NOTE 2:	Different Application IDs of the same DCAF or different DCAFs may be selected for different UEs, since each DCAF can only determine the proximity information for the UEs that have PDU session between UE and DCAF.

The NWDAF subscribes to the AF services as above invoking either Nnef_EventExposure_Subscribe for untrusted DCAF or Naf_EventExposure_Subscribe service for trusted DCAF with (Event ID = Relative Proximity, Event Filter information, Target of Event Reporting). The target of event reporting and / or Event Filter information is set according to the target of analytics reporting and / or analytics filters set during the step 1 of the procedure.

Event filters can be defined for relative proximity to indicate to DCAF how to process the data from individual UEs to determine the set of UEs to be accounted for relative proximity.

In the case of a trusted DCAF, the NWDAF may provide the Area of Interest, proximity range, predefined geographical area, or other criteria to the DCAF on the resolution of TAIs or any other finer resolution recognizable by the 5GC. In the case of an untrusted DCAF, NEF translates the requested criteria provided as event filter by the NWDAF into geographic zone identifier(s) or other geographic range identifier(s) or geographic direction identifier(s) that act as event filter(s) for the DCAF.

The DCAF may process the data from individual UEs based on Event Filters indicated by the NWDAF to determine the set of UEs to be accounted for relative proximity before notifying that directly (for trusted DCAF) or via NEF (for untrusted DCAF) to the NWDAF. Input data that may be collected from DCAF can be found in Table 6.X.2-2.

NOTE 3:	Details of DCAF and its data processing can be found in TS 26.531 [32] and TS 26.532 [Y3].

8.	The NWDAF collects input data from AMF as defined in Table 6.X.2-3 via the AMF event exposure service.

NOTE 4:	Step 8 could be performed before step 6 when DCAF cannot determine the set of UEs fulfilling a proximity criterion.

9.	The NWDAF collects input data from GMLC as defined in Table 6.X.2-3 using the Ngmlc_Location service.

10.	The NWDAF collects input data from AF as defined in Table 6.X.2-3 via the AF event exposure service.

11.	The NWDAF derives requested analytics.

12.	The NWDAF provide requested relative proximity and TTC information as defined in Table 6.X.3-1 and Table 6.X.3-2 to the consumer NF or AF along with the corresponding Validity Period or any Validity Area, Validity Direction of interest or ranging distance, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

13-15.	If at step 1 the consumer NF or AF has subscribed to receive continuous reporting of relative proximity information, the NWDAF may generate new analytics and, when relevant according to the Analytics target period and Reporting Threshold, provide them along with the corresponding Validity Period (or any Validity Area, Validity Direction of interest or ranging distance) to the consumer NF or AF.

* * *End of Changes * * *