3GPP TSG-SA2 Meeting #154-AH-e	S2- 2300783

# Electronic Meeting, January 16th - January 20th 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0092 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNAI mapping based on conclusions in TR 23.700-48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-1-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This clause contains the description and procedure of DNAI mapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the detailed description and procedure of mapping table between EAS IP address and DNAI. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Absent of DNAI mapping procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all is new)

6.x	DNAI mapping Information Management

6.x.1	General

NEF/UDR may be configured by OAM with the DNAI mapping information. This information contains mappings between EAS IP address(es)/IP address range(s) and DNAI(s), and/or TA or list(s) of TAs associated with the DNAI. AF may subscribe to the mapping information to NEF. AF may request immediate reporting. If mapping information is stored in UDR, NEF may subscribe to mapping information in UDR, where NEF may request immediate reporting.

The EAS address and DNAI Mapping Information record in UDR is shown Table 6.x.1-1.

Table 6.x.1-1 Description of EAS address and DNAI Mapping Information

| Parameters | Description |
| --- | --- |
| DNN | DNN for the EAS Deployment Information. [Mandatory] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [Mandatory] |
| EAS address(es) | IP address(es) of the EASs or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es)) and/or FQDN(s) where the EAS is deployed for each DNAI. [optional] |
| TAI(s) | List(s) of TAIs and/or range of TAs for each DNAI [optional] |

6.x.2	AF request for DNAI Procedures

Figure 6.x.2-1: AF request for DNAI based on AF request

1.	AF invokes Nnef_DNAIMap subscribe operation to request the EAS address to DNAI mapping. The request may include EAS address (IP address,IP address range or FQDN) and/or geographical area and optionally an AF identifier.

2.	The NEF determines the DNN and, if needed, the S-NSSAI related to the AF identifier. If geographical area is received, NEF translates this to a TA. If mapping information is stored in UDR and NEF has not previously requested the DNAI mapping information from UDR, NEF invokes the Nudr_DM_Query service. NEF may implicitly subscribe to changes of DNAI mapping information.

The NEF may store the mapping information between EAS IP/IP range and DNAI locally. If the NEF receives the AF requests to obtain DNAI and the NEF has already store the mapping information between EAS IP/IP range and DNAI locally, the NEF directly performs step 5 to notify the DNAI to AF and skip step 2 - 4.

3.	The UDR responds with the DNAI mapping information.

4.	NEF determines the DNAI(s) based on the DNAI mapping information.

5.	NEF notifies AF of the DNAI(s).

END of CHANGES