3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300816

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0093 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 23.548 common EAS enforcement for set of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Lenovo, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Edge_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | How AF provides the common EAS/DNAI and where the 5GC selected common EAS/DNAI to be stored need to be clarified. And the coordination procedure among SMF is to be defined. Besides, it needs clarification how the SMF matches FQDN reported by EASDF with PCC rule for determination of UE belonging to a set of UEs for common EAS/DNAI, since FQDN is not included in PCC rule in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update description of common EAS/DNAI discovery procedure. 2) Add procedure for coordinatioin among SMFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution of KI#4 is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.5, 6.2.3.2.6, 6.2.3.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR3987, TS23.502 CR3792, TS23.503 CR0859 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.2.5	Common EAS discovery for a set of UEs

The following is the procedure for discovery common EAS for set of UEs accessing the same application. Different UEs can be served by different SMFs.

The common EAS IP address for the set of UEs can be provided by AF or determined by 5GC. AF can provide the common EAS IP address for the set of UEs as part of group data, or AF can provide the common EAS IP via AF Traffic influence procedure as defined in clause 4.3.6.2 TS23.502[3], AF may determine the common EAS IP address based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 TS 23.502[3]. Also, the common EAS IP address can be determined by SMF using EAS discovery procedure, and SMF stores the common EAS IP address in UDR as part of AF traffic influence request information.

Figure 6.2.3.2.5-1: Common EAS discovery for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common EAS for set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the UEs for correlated selection of EAS.

An EAS Correlation indication can be provided for selecting the same EAS for set of UEs accessing the same application.

A Traffic Correlation ID can be provided for identification of a set of UEs accessing the application identified by the Traffic Description.

A Common EAS can be included in AF request, if it is determined by AF.

Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same EAS.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with Traffic Correlation ID and EAS Correlation indication and/or Common EAS to SMF.

2.	Based on steps 1-19 in figure 6.2.3.2.2-1, with the following updates:

In step 9:

If FQDN in Neasdf_DNSContext_Notify Request is for the application indicated in PCC rule, and if EAS Correlation indication is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accesses the application and determines the UE needs to use the common EAS for the set of UEs. If FQDN is included in the PCC rule, SMF matches the FQDN in Neasdf_DNSContext_Notify Request with FQDN in the PCC rule, otherwise SMF maps the service data flow template information (e.g. application identifier) into FQDN for the matching,e.g. based on local configuration, PFD or EDI.

If the common EAS is not available for the set of UEs:

Steps 10-15 are used for discovering an EAS.

If the common EAS is available for the set of UEs:

Steps 10-15 are skipped.

In step 17:

SMF sends DNS message handling rule with IP address for the common EAS and instructs EASDF to return the Common EAS IP address in a DNS response to UE directly.

In step 19:

EASDF sends DNS response with the IP address of the common EAS instructed in step 17 to UE.

# * * * * Second change * * * *

6.2.3.2.6	EAS discovery corresponding to Common DNAI for a set of UEs

The common DNAI for the set of UEs can be provided by AF or determined by 5GC. AF can provide the common DNAI for the set of UEs via AF Traffic influence procedure as defined in clause 4.3.6.2 TS23.502[3] , AF may determine the common DNAI based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 TS 23.502[3]. Also, the common DNAI can be determined by SMF and SMF stores the common DNAI in UDR as part of AF traffic influence request information.

The following is the procedure for discovery EAS corresponding to a Common DNAI for set of UEs accessing the same application.

Figure 6.2.3.2.6-1: EAS discovery corresponding to Common DNAI for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common DNAI for set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the UEs for correlated selection of DNAI.

An indication of traffic correlation can be provided for indicating selecting the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for set of UEs accessing the same application.

A Traffic Correlation ID can be provided for identification of a set of UEs accessing the application identified by the Traffic Description.

A Common DNAI can be included in AF request, if it is determined by AF.

Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same DNAI.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with Traffic Correlation ID and indication of traffic correlation and Common DNAI to SMF.

2.	Based on steps 1~19 in figure 6.2.3.2.2-1, with the following changes:

In step 10:

If FQDN in Neasdf_DNSContext_Notify Request is for the application indicated in PCC rule, and if indication of traffic correlation is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accessing the application and determines the UE needs to connect to EAS corresponding to the same DNAI for the set of UEs. If FQDN is included in the PCC rule, SMF matches the FQDN in Neasdf_DNSContext_Notify Request with FQDN in the PCC rule, otherwise SMF maps the service data flow template information (e.g. application identifier) into FQDN for the matching, e.g. based on local configuration, PFD or EDI.

If the common DNAI is not available, SMF determines the common DNAI.

Once the common DNAI is available, for Option A, SMF provisions EASDF with the information to build EDNS Client Subnet option that refers to a location that is topologically close to the common DNAI; for Option B, SMF provisions EASDF with Local DNS server related to the common DNAI.

Editor's note:	It is FFS if and how the common DNAI can be changed during Edge relocation procedure.

# * * * * End of changes * * * *