3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300863r01

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0281 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | location service continuity between EPS and 5GS (bi-direction) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The study on enhancement to the 5GC LoCation Services Phase 3 (FS_eLCS_Ph3) has reached conclusions on support of location service continuity between EPS and 5GS (bi-direction) in TR 23.700-71. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new procedure clause to define the Location service continuity between EPS and 5GS (bi-direction) for defferred MT-LR request as per the conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location service continuity for defferred MT-LR request between EPS and 5GS (bi-direction) is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.Y (new), 6.Y.1 (new), 6.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.Y	Location Service Continuity between EPS and 5GS (bi-direction) for deferred MT-LR request

Clause 6.Y.1 presents the location service continuity procedrue from 5GS to EPS. Clause 6.Y.2 presents the location service continuity procedrue from EPS to 5GS. The GMLC in the Figure 6.Y-1 and Figure 6.Y-2 presents collocated EPC-GMLC and 5GC-GMLC or EPC-GMLC and 5GC-GMLC with the non-standardized Lr' interface.

### 6.Y.1 Location Service Continuity from 5GS to EPS

Figure 6.Y.1-1: LCS Continuity Solution for 5GS to EPS Mobility

1.	same as step 1 of deferred 5GC-MT-LR procedure in clause 6.3.1 of TS 23.273.

2.   [Optional] For the multiple QoS class case, the 5GC-GMLC performs the location QoS mapping, obtains and forwards the mapped location QoS  as well as the corresponding unmapped multiple location QoS to LMF via step 4, step 5 and step 14 in clause 6.3.1 of TS 23.273.

3.  Target UE positioning is started in 5GS, e.g, aftrer steps 2-15 of deferred 5GC-MT-LR procedure in clause 6.3.1.

4.	[Optional] In the deferred periodic or triggered 5GC-MT-LR case, if the LCS QoS class in location request is "multiple QoS class" and the access type supported by the UE for event reporting includes "E-UTRAN connected to EPC", the LMF determines ues the corresponding mapped location QoS can be applicable to EPS.

5.   [Conditional] In the deferred periodic or triggered 5GC-MT-LR case, LMF sends the LCS Periodical-Triggered Invoke Request.

If step 4 is performed, the request includes the multiple location QoS corresponding mapped location QoS used for EPS.

6.  UE response to LMF with the LCS Periodical-Triggered Invoke Return Result.

7.	LMF sends the Nlmf_Location_DetermineLocation Response to AMF. LMF indicates AMF to not release resources for the location request in the response, if the step 1 is for deferred periodic or triggered 5GC-MT-LR and LCS continuity between 5GS and EPS is to be supported by the operator.

8. 	UE may stay in 5GC over a period of time and the deferred 5GC-MT-LR procedure can continue in 5GS.

9.	When 5GS to EPS handover happens and the handover succeeds, AMF receives the relocation complete notification message from MME as described in figure 4.11.1.2.1-1 of TS 23.502 [19].

NOTE Y: The handover procedure is decopled with the location service continuity and the subsequent procedures of the handover can be performed in parallel.

10a. AMF sends the Nlmf_Location_CancelLocation Request to LMF to cancle the ongoing positioning session in 5GC. For deferred 5GC-MT-LR for periodic or triggered request, the Notification Correlation ID in the Nlmf_Location_CancelLocation Request is used to identify the location session to be released in AMF and LMF. For immediate 5GC-MT-LR, deferred 5GC-MT-LR for UE available event and 5GC-MO-LR session, the request should contain the UE ID and a handover indication.

10b. [Conditional] For deferred 5GC-MT-LR for periodic or triggered request, steps 6-9 in Figure 6.3.3-1 are performed.

11a. [Conditional] If this is a deferred 5GC-MT-LR session, after AMF receives the relocation complete notification message in step 9, it sends the Namf_LocationEventNotify or Namf_Location_ProvidePositioningInfo response message to 5GC-GMLC, includes the the LDR Reference number, the MME ID/address, the handover to EPS notification.

11b. The LCS session is started on EPS, e.g., steps 2-25 of deferred EPC-MT-LR in Figure 9.1.19.1-1 in TS 23.271 [4].

Specifically, for a deferred or immediate 5GC-MT-LR session:

-	Based on the MME ID/address and handover notificaion in step 11a, the collocated or seperated 5GC-GMLC is aware that the UE has handoverd to EPS and restarts the location session in EPS. To restart the LCS session in EPS with the required location QoS in 5GS:

-	The 5GC-GMLC forwards the location QoS (maybe a mapped location QoS obtained in step 2 for multiple QoS class) to the EPC-GMLC via Lr' interface.

For the deferred periodic or triggered MT-LR case, if UE receiced an LCS Periodical-Triggered Invoke in EPS but the location resource for the origininal 5GS location session is not released (i.e., due to UE access to EPS and the location service cancelation message not reach to UE in time), UE does not create new location resources for the request with the same LDR Reference number and sends an acknowledgement to the MME directly.

12-14.    For the deferred periodic or triggered MT-LR case, if the location resource in UE for the origininal 5GS location session is not released, when the event is detected by UE in EPS, it should send an LCS MO-LR Invoke message for event report to the MME.

If the location QoS is multiple QoS in 5GS, the UE should include the location QoS can be applicable to EPS in the LCS MO-LR Invoke message (i.e., "Best Effort" LCS QoS Class with most stringent QoS requirement) based on the location QoS received in step 5.

For the deferred periodic or triggered MT-LR case, the procedure can continue with step 15-24 as described in Figure 9.1.19.1-1 of TS 23.271 [4].

NOTE XY:	If UE finds handover is not complete but the event is triggered, i.e. step 10 happens before handover complete, the UE need wait handover complete to send the triggered LCS MO-LR Invoke message.

# * * * * Next change * * * *

### 6.Y.2 Location Service Continuity from EPS to 5GS

Figure 6.Y.2-1: LCS Continuity Solution for EPS to 5GS Mobility

1.	Target UE positioning is started in EPS, e.g., after steps 1-16 of deferred EPC-MT-LR procedure for Periodic and Triggered Location procedure in Figure 9.1.19.1-1.

NOTE X: Since all the "LCS QoS class" in "location QoS" supported by EPS can be supported by 5GS, there is no need to perform the location QoS class mapping for a location session started in EPS.

1a. [Conditional] UE may stay in EPC for a period of time and the deferred EPC-MT-LR procedure can continue in EPS.

2.	When EPS to 5GS handover happens and the handover succeeds, MME receives the Forward Relocation Complete Notification message (i.e. step 5in clause 4.11.1.2.2.3 of TS 23.502 [3]) from AMF, if MME finds UE has positioning related information, MME trigger to perform step 3~5.

NOTE Y: The handover procedure is decopled with the location service continuity and the subsequent procedures of the handover can be performed in parallel.

3-4. MME sends an LCS-AP location abort request message to E-SMLC. E-SMLC abort on going positioning session in EPC.

5.   If there is a deferred EPC-MT-LR location session, MME sends Subscriber Location Report to GMLC, includes the AMF ID/address, location QoS, the handover to 5GS notification and the LDR Reference number.

NOTE Z:	Step 3-5 can be performed in parallel.

6.	The LCS session is started on 5GS, e.g., steps 4-31 of deferred 5GC-MT-LR procedure in clause 6.3.1.

Based on the AMF ID/address information, GMLC be aware that the UE handoverd to 5GS and restart the location session in 5GS with the location QoS received in step 5 for the location request.

Specifically, for a deferred EPC-MT-LR session:

-	Based on the AMF ID/address and notificaion information in step 5, the collocated or seperated EPC-GMLC is aware that the UE has handovered to 5GS. To restart the LCS session in 5GS with the required location QoS in EPS:

-	The EPC-GMLC forwards parameters received in step 5 to a 5GC-GMLC via Lr' interface.

For the deferred periodic or triggered MT-LR case, if UE receiced an LCS Periodical-Triggered Invoke in 5GS but the location resource for the origininal EPS location session is not released (i.e., due to UE access to 5GS and the location service cancelation message not reach to UE in time), UE does not create new location resources for the request with the same LDR Reference number and sends an acknowledgement to the AMF directly.

7-8.  For the deferred periodic or triggered MT-LR case, if the location resource in UE for the origininal EPS location session is not released, when the event is detected by UE in 5GS, it should send an Event Report message to the LMF through AMF. If AMF has not received the request from GMLC in step 5, AMF dose not identify the LDR Reference number in the Event Report, thus AMF store the Event Report and wait the LDR Reference number from the GMLC.

For the deferred periodic or triggered MT-LR case, the procedure can continue with step 26-31 as described in clause 6.3.1.

NOTE XX:	If UE finds handover is not complete but the event is triggered, i.e. step 6 happens before handover complete, the UE need wait handover complete to send the event report message.

# * * * * End of changes * * * *