3GPP TSG SA-WG2 Meeting #154AH 	S2-2300871r03

# Online, 16-20 January 2023	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4004 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optimizations for the support of time vality policies for a network slice and graceful network slice PDU sessions release. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | this CR enables the optimized support of temporary slices, i.e. S-NSSAIs that have a time validity |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides the necessary normative text |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support of this feature that has been agreed as part of TR 23.700-41 counlcusions for KI#3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.1, 5.15.x(new), 5.4.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

.

### 5.15.1	General

A Network Slice instance is defined within a PLMN or within an SNPN and shall include:

-	the Core Network Control Plane and User Plane Network Functions, as described in clause 4.2,

and, in the serving PLMN, at least one of the following:

-	the NG-RAN described in TS 38.300 [27];

-	the N3IWF or TNGF functions to the non-3GPP Access Network described in clause 4.2.8.2 or the TWIF functions to the trusted WLAN in the case of support of N5CW devices described in clause 4.2.8.5;

-	the W-AGF function to the Wireline Access Network described in clause 4.2.8.4.

The 5G System deployed in a PLMN shall always support the procedures, information and configurations specified to support Network Slice instance selection in the present document, TS 23.502 [3] and TS 23.503 [45].

Network slicing support for roaming is described in clause 5.15.6.

Network slices may differ for supported features and network functions optimisations, in which case such Network Slices may have e.g. different S-NSSAIs with different Slice/Service Types (see clause 5.15.2.1). The operator can deploy multiple Network Slices delivering exactly the same features but for different groups of UEs, e.g. as they deliver a different committed service and/or because they are dedicated to a customer, in which case such Network Slices may have e.g. different S-NSSAIs with the same Slice/Service Type but different Slice Differentiators (see clause 5.15.2.1).

The network may serve a single UE with one or more Network Slice instances simultaneously via a 5G-AN regardless of the access type(s) over which the UE is registered (i.e. 3GPP Access and/or N3GPP Access). The AMF instance serving the UE logically belongs to each of the Network Slice instances serving the UE, i.e. this AMF instance is common to the Network Slice instances serving a UE.

NOTE 1:	Number of simultaneous connection of Network Slice instances per UE is limited by the number of S-NSSAIs in the Requested/Allowed NSSAI as described in clause 5.15.2.1.

NOTE 2:	In this Release of the specification it is assumed that in any (home or visited) PLMN it is always possible to select an AMF that can serve any combination of S-NSSAIs that will be provided as an Allowed NSSAI.

The selection of the set of Network Slice instances for a UE is triggered by the first contacted AMF in a Registration procedure normally by interacting with the NSSF, and can lead to a change of AMF. This is further described in clause 5.15.5.

A PDU Session belongs to one and only one specific Network Slice instance per PLMN. Different Network Slice instances do not share a PDU Session, though different Network Slice instances may have slice-specific PDU Sessions using the same DNN.

During the Handover procedure the source AMF selects a target AMF by interacting with the NRF as specified in clause 6.3.5.

Network Slice-Specific Authentication and Authorization (NSSAA) enables Network Slice specific authentication as described in clause 5.15.10.

Network Slice Admission Control (NSAC) controls the number of registered UEs per network slice and the number of PDU Sessions per network slice as described in clause 5.15.11.

Support of subscription-based restrictions to simultaneous registration of network slices uses Network Slice Simultaneous Registration Group (NSSRG) information to enable control of which Network Slices that can be registered simultaneously by a UE as described in clause 5.15.12.

Support of data rate limitation per Network Slice for a UE enables enforcement of Maximum Bit Rate per Network Slice for a UE as described in clause 5.15.13.

The selection of N3IWF supporting a set of slice(s) is described in clause 6.3.6.

The Support of Optimized handling of network slices which have temporal validity is described in clause 5.15.x. this also covers aspects related to graceful release of network slices connectivity during slice decommissioning.

## More CHANGEs

#### 5.15.x	Optimized handling of network slices which have temporal validity

A network slice may be available for all UEs or just for some UEs only for a limited time that is known at the network. The limited time duration may be due to, for example, the fact that network slice is only temporarily or periodically active in the deployment, or it is being decommissioned at a known future time, or the slice may be deployed for limited time to serve one event or a UE may be only authorized to access the network slice for a limited amount of time that is known in advance. All of this can be enabled by S-NSSAI validity time policies that the network and the UE can handle in order to more efficiently manage the signalling load associated to the transitions in RM and SM state for the network slice.

The AMF, based on OAM configuration or information received from the UDM or NSSF, may indicate to a UE, which has indicated it supports the temporary network slices optimization feature in the UE MM Core Network Capability (see clause 5.4.4a),  for some S-NSSAIs entry in the Configured NSSAI (in roaming case either because of the VPLMN S-NSSAI or the mapped S-NSSAI of the HPLMN availability) whether a validity time applies to a S-NSSAI, in a Registration Accept or in a UE Configuration Update. If validity time policies applies to a S-NSSAI entry, a supporting UE shall:

-	 If the validity time indicates the network slice associated with the S-NSSAI is available, the UE can request the S-NSSAI in a Requested NSSAI in a Registration request and, if the S-NSSAI is included in the Allowed NSSAI or it is partially allowed, the UE can establish PDU sessions associated with the S-NSSAI.

-	If the validity time indicates the network slice associated with the S-NSSAI is not available, not request the S-NSSAI (with any applicable corresponding S-NSSAI of the HPLMN for which the validity time applies in roaming case) in a Requested NSSAI in a Registration Request (i.e. behave as if the network slice of the S-NSSAI entry was rejected in the PLMN) until new validity time information is provided or until the validity time information indicates the S-NSSAI entry is becoming available.

-	If the validity time indicates the network slice associated with the S-NSSAI is not available, if the S-NSSAI (and the applicable corresponding S-NSSAI(s) of the HPLMN for which the validity time information applies in roaming case) is included in the Allowed NSSAI or it is partially allowed in the registration area, , locally (i.e. without issuing a Mobility Registration update) consider the S-NSSAI  (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case)  no longer included in the Allowed NSSAI or no longer partially allowed in the RA, and consider the S-NSSAI  (with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case)  as if it was Rejected in the PLMN until new validity time information is provided or until the validity time information indicates the S-NSSAI is available again. The UE shall also locally release any PDU sessions associated with the S-NSSAI as per existing functinality.

An AMF supporting the feature, for a S-NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time policies applies in roaming case) that is subject to validity time policies, shall, for a supporting UE:

-	If the validity time indicates the network slice is not available, if the S-NSSAI is provided in a Requested NSSAI in a Registration Request (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case), provide the UE with the validity time policies for the relevant S-NSSAI entries in the Configured NSSAI, if any, or remove the S-NSSAI entry(ies) from the Configured NSSAI if their validity time indicates the network slice(s) associated with some S-NSSAI entry(ies) will not become available again.

-	If the S-NSSAI is in the Allowed NSSAI, or it is partially allowed (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case), and the Validity Time indicates that the corresponding network slice is no longer available, the AMF silently, i.e. without sending any signalling to the UE, if this applies, removes the corresponding S-NSSAI entry in the Allowed NSSAI. or erases the corresponding partially allowed S-NSSAI, (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case), or, if this applies, no longer considers the S-NSSAI partially allowed in the RA (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case). If there is any PDU session established for the network slice, the AMF

requests the SMF to locally release the PDU session within the network.  T

An AMF supporting the feature, for a S-NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time policies applies in roaming case) that is subject to validity time information, shall, for a non-supporting UE:

-	If the validity time available at the AMF for a S-NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case) indicates the network slice is available, the network slice can be allowed or partially allowed, if requested, and PDU sessions can be established in the network slice.

-	If the validity time indicates the network slice is not available, reject the S-NSSAI if provided in a Requested NSSAI in a Registration Request (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case) and remove the S-NSSAI entry(ies) from the Configured NSSAI by providing a new Configured NSSAI in the Registration Accept.

-	If the S-NSSAI is in the Allowed NSSAI, (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case), and the Validity time indicates that the corresponding network slice is no longer available, the AMF removes the corresponding S-NSSAI entry in the Allowed NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case), or no longer considers the S-NSSAI partially allowed in the RA (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case). The AMF then provides the new Allowed NSSAI or Partially Allowed S-NSSAI(s) and a new Configured NSSAI in a UE Configuration Update to the UE. If the network slice is becoming not available, and a large number of UEs are impacted, the AMF can send the updated information to the non supporting UEs while avoiding a surge in signalling.

-	If the AMF detects from the validity time of the S-NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case) that the network slice is available again, the AMF updates the Configured NSSAI to include the S-NSSAI (in association with any applicable corresponding S-NSSAI of the HPLMN for which the validity time information applies in roaming case) in a UE Configuration Update procedure. If a large number of UEs are impacted, the AMF can send the updated information to the non supporting UEs by considering a relative ordering policy among UEs to avoid a surge in signalling.

NOTE:	The AMF, for the case of UE not performing any actions despite of the timing information provided by the network, can terminate PDU Session(s) associated with S-NSSAI subject to be terminated according to this timing information by explicitly releasing the PDU Sessions associated with the S-NSSAI.

## MORE CHANGES

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) see clause 5.4.9..

-	Paging Subgrouping Support Indication (NR) see clause 5.4.12.

-	CAG, see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

-	Support of NSAG (see clause 5.15.14).

-	Minimization of Service Interruption (MINT), as described in clause 5.40.

-	Equivalent SNPNs (see clause 5.30.2.11).

-	Unavailability Period, as described in clause 5.4.1.4.

-	Support temporary network slices optimization (see clause 5.15.x)

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

## END of CHANGES