SA WG2 Meeting #154AHE (e-meeting)	S2-2300889r02

Jan 16 – 20, 2023	(revision of S2-2210458)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0096 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECS Address Configuration Information delivery in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Huawei, Nokia, Nokia Shanghai Bell, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_ph2 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-62[x], in the UDM and UDR, the subscription data of EACI is stored per PLMN ID. The V-SMF (for the LBO case) or the H-SMF via the V-SMF (for the HR case) provides the UE with the EACI as indicated in step 11 in figure 4.3.2.2.1-1 of TS 23.502 [9]. For the HR case, the V-SMF does not modify, but just delivers the EACI provided by the H-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID in ECS Address Configuration Information provided by AF, that the 5GC can identify which PLMN the ECAI associated to. Add a new clause 6.5.2.X to describe the ECS Address Configuration Information provisioning in roaming case. Add a new clause 6.5.2.X to support ECS Address Configuration Information provisioning from VPLMN in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE in HR case cannot receive ECS Address Configuration Information, and H-SMF in HPLMN cannot decide in certain roaming PLMN, which ECS Address Configuration Information should be delivered to UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, 6.5.2.2, 6.5.2.X (new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

#### 6.5.2.1	ECS Address Configuration Information

The ECS Address Configuration Information consists of one or more ECS Configuration Information as defined in clause 8.3.2.1 of TS 23.558 [5]. The ECS Configuration Information may contain Spatial Validity Conditions, which includes one of the following alternatives:

-	a Geographical Service Area (see TS 23.558 [5]);

-	a list of TA(s); or

-	a list of countries (list of MCC).

-	a list of PLMN IDs (see Table 4.15.6.3d-1 of TS 23.502[x]).

A UE may receive multiple instances of ECS Address Configuration Information e.g., corresponding to different ECSPs (e.g., the MNO or a 3rd party service provider).

The ECS Address Configuration Information is sent to the UE on a per PDU Session basis. The same PDU session can be used by multiple ECS providers.

The SMF does not need to be aware of the internal structure of the ECS Address Configuration Information.

* * * Start of Change 2, all new text * * *

#### 6.5.2.X	ECS Address Provisioning in Roaming

For both LBO and HR case, the subscription data of the ECS Address Configuration Information in UDM or UDR is stored per PLMN ID.

For the LBO case, an AF in the visited PLMN may provide the EACI via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502[3] to UDM via H-NEF. This ECS Address Configuration Information is further provided to SMF as part of subscription information.

For the HR case when access to EHE in VPLMN is allowed:

-	The HPLMN has the knowledge of EACI in the VPLMN: For this scenario, the AF is able to interact with NEF in HPLMN. The AF may provide the VPLMN EACI to H-NEF via External Parameter Provisioning procedure as described in clause 4.15.6.3d in TS 23.502[3], and further to the UDM. During the HR PDU session establishment procedure, the H-SMF sends the VPLMN EACI to V-SMF and then to UE. The V-SMF does not modify, but just delivers the EACI provided by the H-SMF.

-	HPLMN does not have the knowledge of EACI in VPLMN: For this scenario, the AF can’t interact with NEF in HPLMN. The AF deployed in the VPLMN provides the VPLMN EACI to the V-SMF via the V-NEF as defined in clause 6.5.2.x. During the HR PDU Session establishment, the V-SMF sends the VPLMN EACI obtained from V-NEF to the H-SMF, and H-SMF decides the VPLMN ECS Address Configuration Information sent to V-SMF and then to UE according to the PLMN ID additionally.

NOTE: It depends on the PLMN operation that the HPLMN can decide the ECAI if both VPLMN and UDM provides the ECAI.

Editor’s note: it is FFS whether a new API is used for V-AF to provide ECAI information for roamers or existing NEF services can be extended to do so.

UDR

NEF

AF

1.Nnef_ECSAddress_Create /Update/Delete Request

2.NEF Handling

3.Nudr_DM_Create/Update/Delete Request

4.Nudr_DM_Create/Update/Delete Response

5.Nnef_ECSAddress_Create /Update/Delete Response

SMF

NEF

3.Nnef_ ECSAddress _Notify Request

4.Nnef_ ECSAddress _Notify Response

1.Nnef_ ECSAddress _Subscribe Request

2.Nnef_ ECSAddress _Subscribe Response

* * * End of Change * * *