SA WG2 Meeting #154AHE (e-meeting)	S2-2300895

Jan 16 – 20, 2023	(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0664 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of NWDAF-assisted URSPs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As the conclusion in TR 23.700-81, the NWDAF assisted URSP rule is agreed to normative. But the procedure NWDAF assisted URSP rule generation is missing, and this should be clarified in a new clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Give a overall procedure of PCF subscribes one/serveral Analytics from NWDAF and decides the related to URSP field (RSCs) according to these analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures of PCF subscrbie analytic from NWDAF to assist URSP rule generation is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 (all new text) * * *

6.4.7	Procedures of NWDAF-assisted URSP rule

This section describes the procedure of NWDAF-assisted URSP rule.

The NF consumers (for example, PCF) may adjust the fields (i.e. RSCs) and the RSD preference in URSP rules based on the several analytics result from NWDAF according to the Table 6.4.7-1.

The NF consumers determines the RSCs in URSP rule according to the analytic from NWDAF that indicates the lower load (e.g.: lower load of a slice or DNN) or indicates the better performance (e.g.: better performance when using certain SSC mode or PDU session type).

For the details of which Analytic that PCF consumes from NWDAF to adjust URSP rule, it can be referred to section 6.1.1.3 of TS 23.503[x].

Table 6.4.7-1: Mapping of fields in URSP and analytics

| URSP field (RSCs) | Analytic ID |
| --- | --- |
| S-NSSAI | "Service Experience", "Slice Load level information", "Dispersion Analytics", "Session Management Congestion Control Experience" |
| DNN | "Service Experience", "UE Communication", "Session Management Congestion Control Experience", "DN Performance" |
| Non-Seamless Offload Indication | "UE Communication", "WLAN performance", "Load level information", "Network Performance", "User Data Congestion" |
| SSC Mode | "Service Experience" |
| PDU Session type | "Service Experience" |
| Access Type Preference | "Service Experience", "WLAN Performance" |
| PDU Session Pair ID | "Redundant Transmission Experience" |
| RSN | "Redundant Transmission Experience" |
| Route Selection Validation Criteria |  |
| Time Window | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation |
| Location Criteria | Based on the validity period and spatial validity provided in the Analytics ID used for RSD generation |

* * * End of Change* * *