SA WG2 Meeting #154AHE (e-meeting)	S2-2300898

Jan 16 – 20, 2023	(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3810 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Information flow for UPF event exposure service to NWDAF for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the section 4.15.11.3, the procedure of information flow for UPF event exposure service to NWDAF for any UE is missing. And another scenario is, if the consumer of NWDAF services that doesn’t provide the DNN or S-NSSAI, and only provide Application server address/FQDN, the NWDAF should recover the target DNAI from NEF/UDR as indicated in the R18 EDGE conclusion of 23.700-48[x]. The |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NWDAF invokes the service to request the DNAI information with EAS IP/IP range and/or FQDN and optionally DNN, S-NSSAI, and geographical area. The NEF/UDR can provide the DNAI to NWDAF. The NWDAF triggers the SMFs/UPFs discovery to NRF by Nnrf_NFDiscovery_Request with DNN, S-NSSAI, DNAI and etc. This procedure is to discover the related SMFs/UPFs associated with any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the procedures for information exposure for any UE is unclear. How to NWDAF to recover the DNAI to determine the target UPFs/SMFs associated with any UE is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.X(all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1(all new text) * * *

4.15.4.X.Y	Information flow for UPF event exposure service to NWDAF for anyUE

When the NWDAF subscription request does not meets the criteria for direct subscription to UPF as described in 4.15.4.X, NWDAF shall subscribe via the SMF. NWDAF send a Nnrf_NFDiscovery_Request to NRF including the related discovery parameters and the NRF returns the SMF(s) which meet(s) the discovery request. NWDAF executes then steps 4-5 in figure 4.15.4.X.Y -1(for single UE) where the target of the event reporting is in this case anyUE.

When the NWDAF subscription request meets the criteria for direct subscription to UPF as described in 4.15.4.X.Y -1, NWDAF queries the NRF including the related discovery parameter. The NRF returns the UPF(s) which meet(s) the discovery request, and NWDAF sends the Nupf_EventExposure_Subscription directly to UPF.

Figure 4.15.4.X.Y -1: UPF Information Exposure to NWDAF of any UE scenario

1.	The analytics consumer sends a request to the NWDAF for analytics on any UE. The consumer provides the any UE in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, APP ID, DNAI and etc.

2.	(Optional) If in the analytic filter information, only application server IP address/FQDN exists and no DNN/S-NSSAI exists, the NWDAF should firstly obtain the DNAI from NEF/UDR by Application server IP address/FQDN. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN and optionally DNN, S-NSSAI, and geographical area. And the NEF invokes the Nudr_DM_Query service to request the suitable DNAI(s) information.

3.	(Optional) The NEF/UDR determines the suitable DNAI(s) based on the EAS IP/IP range and/or FQDN and optionally DNN, S-NSSAI, and geographical area. The UDR notifies the DNAI(s) or the udpated DNAI information to NEF and to NWDAF.

4.	The NWDAF triggers the SMFs/UPFs discovery to NRF by Nnrf_NFDiscovery_Request with DNN, S-NSSAI, DNAI and etc. This procedure is to discover the related SMFs/UPFs associated with any UE.

5.	The NWDAF receives Nnrf_NFDiscovery_Response with several SMFs/UPFs ID or FQDN or IP address(es) from NRF.

6.	(Option 1) If the subscribed UPF events by NWDAF needs the SMFs to do a third-party subscription onto UPF on behalf of NWDAF, the same procedure as Indirect subscription via several SMFs (step 3 – 5 in Figure 4.15.4.X.Y -1(for single UE)).

6.	(Option 2) If the subscribed UPF events by NWDAF allows to directly subscribe to UPF, the NWDAF triggers the several Nupf_EventExposure_Subscribe to UPFs. The information included in the subscription is the same as step 3 in 4.15.4.X.Y -1.

7.	The several UPFs response with the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the NWDAF.

* * * End of Change * * *