SA WG2 Meeting #154AHE (e-meeting)	S2-2300898

Jan 16 – 20, 2023	(revision of S2-23xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3810 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Information flow for UPF event exposure service to NWDAF for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia , Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the section 4.15.11.3, the procedure of information flow for UPF event exposure service to NWDAF for any UE is missing. And another scenario is, if the consumer of NWDAF services that doesn’t provide the DNN or S-NSSAI, and only provide Application server address/FQDN, the NWDAF should recover the target DNAI from NEF/UDR as indicated in the R18 EDGE conclusion of 23.700-48[x]. The |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NWDAF invokes the service to request the DNAI information with EAS IP/IP range and/or FQDN and optionally DNN, S-NSSAI, and geographical area. The NEF/UDR can provide the DNAI to NWDAF. The NWDAF triggers the SMFs/UPFs discovery to NRF by Nnrf_NFDiscovery_Request with DNN, S-NSSAI, DNAI and etc. This procedure is to discover the related SMFs/UPFs associated with any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the procedures for information exposure for any UE is unclear. How to NWDAF to recover the DNAI to determine the target UPFs/SMFs associated with any UE is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.X(all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1(all new text) * * *

#### 4.15.4.X.Y	Information flow for UPF event exposure service for anyUE

When the subscription request to UPF events does not meets the criteria for direct subscription to UPF as described in 4.15.4.X,1 the UPF event consumer (e.g. NWDAF) shall subscribe via the SMF. The UPF event consumer (e.g. NWDAF)send a Nnrf_NFDiscovery_Request to NRF targeting SMF(s) including the related discovery parameters and the NRF returns the SMF(s) which meet(s) the discovery request. The UPF event consumer then subscribes via the SMF(e.g. NWDAF).

When the UPF event consumer (e.g. NWDAF)subscription request meets the criteria for direct subscription to UPF as described in in 4.15.4.X,1, the UPF event consumer (e.g. NWDAF)queries the NRF targetting UPF(s). The NRF returns the UPF(s) which meet(s) the discovery request, and the UPF event consumer (e.g. NWDAF) sends the Nupf_EventExposure_Subscription directly to UPF.

Figure 4.15.4.X.Y -1: UPF Information Exposure to the UPF event consumer (e.g. NWDAF)of any UE scenario

1.	in case of NWDAF The analytics consumer sends a request to the NWDAF for analytics on any UE. The consumer provides the any UE in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, APP ID, DNAI and etc.

2.	(Optional and only when the UPF event consumer is NWDAF) If in the analytic filter information does not conatin DNN/S-NSSAI, but only application server IP address/FQDN, the NWDAF should firstly obtain the DNAI from NEF/UDR. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN.

3.	(if step 2 took place) The NEF/UDR determines the suitable DNAI(s) and answers  to NWDAF.

4.	The the UPF event consumer  triggers the SMFs/UPFs discovery to NRF by Nnrf_NFDiscovery_Request providing the DNN, S-NSSAI, DNAI etc. This procedure is to discover the related SMFs/UPFs associated with any UE. SMF or UPF(s) are discovered depending on whether the subscription request to UPF events meets the criteria for direct subscription to UPF as described in 4.15.4.X,1 (option 1:option 2)

5.	The RF provides Nnrf_NFDiscovery_Response that may refer to several SMFs/UPFs

6.	(Option 1) If the subscribed UPF events needs the SMFs to do a third-party subscription onto UPF, the same procedure as Indirect subscription via several SMFs (step 3 – 5 in Figure 4.15.4.X.Y -1(for single UE)) takes place via each discoivered SMF.

6.	(Option 2) If the subscribed UPF events allows to directly subscribe to UPF, the UPF event consumer (e;g; NWDAF) triggers the Nupf_EventExposure_Subscribe to all discovered UPFs. The information included in the subscription is the same as step 3 in 4.15.4.X.Y -1.

7.	each UPFs invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (NWDAF).

* * * End of Change * * *