3GPP TSG-SA WG2 Meeting #154-AH-e	S2-2300938r05

# January 16 – 20, 2023, electronic meeting	(revision of S2-221xyz

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0665 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF analytics to detect URSP enforcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions in KI#2 of clause 8.2 of TR 23.700-85 agreed that a new analytic ID is required to be supported by the NWDAF to monitor the traffic in one or multiple PDU sessions for statistics of URSP enforcement. The analytics request includes the expected traffic according to provisioned URSP rules via the S-NSSAI/DNN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description and procedure of new analytics to be supported by the NWDAF to monitor known traffic for statistics of URSP enforcement. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI#2 as part of the UEPO study will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** START OF CHANGE (ALL NEW TEXT)****************************

## 6.X	Statistics for traffic monitoring of known traffic according to provisioned URSP rule(s)

### 6.X.1	General

This clause specifies the procedure on how NWDAF to monitor the traffic in one or multiple PDU sessions based on provisioned URSP rules to the UE. It is assumed that there are associated Packet Detection Rule(s) for the expected traffic listed in the analytics request, i.e. the expected traffic is known and that a URSP rule always includes the S-NSSAI and DNN.

The NWDAF receives expected UE traffic via one or more PDU sessions (based on S-NSSAI/DNN) according to the Traffic Descriptor of provisioned URSP rules and provides information of traffic matching or not matching the expected traffic according to provisioned URSP rules.

NOTE: How traffic from different applications/URSP rules over the same S-NSSAI/DNN and/or PDU session is discerned is implementation-specific

The service consumer may be a PCF. The service consumer uses such analytics to determine if a traffic is routed according to a provisioned URSP rule. The service consumer includes in the analytics request information corresponding to a URSP rule that needs to be monitored.

The consumer of these analytics may indicate in the request:

-	Analytics ID = " URSP enforcement statistics";

-	Target of Analytics Reporting: UE, group of UE or any UE;

-	information corresponding to the expected traffic to be routed via a PDU session based on URSP traffic descriptor list (e.g. Flow Descriptors in URSP rule) of the provisioned URSP rule ;

-	Analytics Filter Information per URSP traffic descriptor list containing:

-	UE ID

-	configured URSP rule to be monitored

-	Area of Interest (i.e. the location of UEs where URSP rule enforcement is monitored) ;

-	S-NSSAI corresponding to a Route Selection Descriptor of the URSP rule ;

-	DNN corresponding to a Route Selection Descriptor of the URSP rule ;

-	An Analytics target period indicates the time period over which the statistics are requested.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.X.2	Input Data

NWDAF collects input data from the UPF. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1: Input data to detect traffic based on a URSP rule

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that established PDU sessions to an S-NSSAI/DNN |
| S-NSSAI | SMF, UPF | The S-NSSAI for which the PDU session is established |
| DNN | SMF, UPF | The DNN for which the PDU session is established |
| IP Flow Information (1..max) | UPF | IP flow information that matches/does not match installed PDRs at the UPF |
| > IP 5-tuple | UPF | IP 5-tuple of the detected traffic |
| > Duration | UPF | Duration of traffic detection. |

Editor’s Note: Additional input data from AF and other Data Sources for URSP rule enforcement are FFS.

### 6.X.3	Output Analytics

The output analytics include statistics of traffic information that was not expected to be routed over a PDU session, is shown in Table 6.X.3-1.

Table 6.X.3-1: Statistics of URSP enforcement

| Information | Description |
| --- | --- |
| UE ID | UE that has established a PDU session over a specific S-NSSAI and/or DNN |
| Statistics of URSP rule enforcement | Identifies traffic that matches installed PDR for application traffic in the analytics request |
| > Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |
| > S-NSSAI |  |
| > DNN |  |
| Statistics of traffic not corresponding to a URSP rule | Identifies traffic that do not correspond to installed PDR associated for the expected application traffic in the analytics request |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol). |

### 6.X.4	Procedures

The procedure for deriving statistics of URSP enforcement is shown below.

Figure 6.X.4-1: NWDAF providing URSP enforcement statistics

1.	The Consumer NF (i.e. PCF) requests or subscribes to the NWDAF to request statistics of URSP enforcement. The consumer includes within analytics filter the expected traffic via a PDU session according to the Traffic Descriptors within provisioned URSP rules to the UE and the S-NSSAI/DNN of the PDU session. It is assumed that there are associated Packet Detection Rule(s) for the expected traffic listed in the analytics request, i.e. the expected traffic is known.

2.	The NWDAF determines the UPF where the UE has established a PDU session to the specified DNN/S-NSSAI.

3.	The NWDAF collects session related information from the UPF of packets corresponding to service data flows matching or not matching installed PDRs for the expected traffic.

Editor’s Note: NWDAF data collection from UPF is specified in UPEAS work.

4.	The NWDAF derives statistics of URSP enforcement indicating traffic matching or not matching traffic according to a URSP rule..

5.	The NWDAF notifies URSP enforcement statistics.

************************************ END OF CHANGE ****************************************