3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300957r03

# Elbonia, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0286 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | introduce new feature: NWDAF assisted positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR conclusion agrees the following principles: - NWDAF provides new analytics for Location Estimation Accuracy. The location accuracy analytics include horizontal or vertical accuracy, indoor/outdoor indication. - Solutions 11 and 2012 will be used as a basis to derive location estimation accuracy analytics. Further details on input data for the new location accuracy analytics will be determined during the normative phase in coordination with eNA_Ph3. - LMF as a NWDAF consumer of such analytics uses Location Estimation Accuracy analytics to determine Position Method in the area where a UE is located. - Location client as a consumer of such analytics uses Location Estimation Accuracy to determine a requested LCS QoS class or adjust application specific parameters (out of scope of 3GPP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new feature NWDAF assisted positioning by proposing general description how LMF can leverage NWDAF analytics for UE location estimate |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description of TR conclusion for NWDAF assisted positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (ALL NEW TEXTS)* * * *

## 5.x	Location estimate assisted by NWDAF

NWDAF may provide analytics for Location Accuracy as specified in TS 23.288 [37].

When LMF receive analytics for Location Accuracy from the NWDAF, as a service consumer, it may use the analytics in determination of positioning methods.

# * * * * End of changes * * * *