SA WG2 Meeting #154AH	S2-2300970

# January 16th – 20th, 2023; Elbonia               			(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-88: Conclusions update for KI#4

Document for: 	Approval

Agenda Item: 	9.3.1

Work Item / Release:	FS_PIN / Rel-18

Abstract of the contribution: This pCR proposes conclusions updates for KI#4

1 Discussion

The TR conclusions for KI#4 currently has an open point regarding whether a single PDU session can serve more than one PIN.

Editor's note:	It is FFS whether one PDU session can serve more than one PIN.

Proposal 1: For PIN traffic isolation, it is proposed to have a separate PDU session for each PIN that is served by a PEGC. The above EN is proposed to be removed without any further action.

In addition, it is not necessary to restrict PIN traffic to a single PDU session. For a single PIN, traffic coming from different PINE may be routed via different PDU session in the PEGC depending on the traffic characteristics. For example, in a PIN a user may have different types of devices like home appliances and smart home devices, XR devices, streaming devices etc. For each type of device, the traffic may be routed through different PDU Session.

Proposal 2: It is proposed to update the conclusions to allow traffic from a single PIN to be routed via more than one PDU Session in the PEGC.

The TR conclusions for KI#4 currently has below conclusion:

17)	If AF for PIN is used, the 5GC authorizes the number of PIN that the AF requests to create, which results in the number of PDU Sessions per PEGC/PEMC for PIN, according to user's PIN service subscription, which reflect the agreement between user and operator for using PIN service.

It is not clear why there shall be any restriction on how many PIN an AF can create. The AF may be serving thousands/millions of PIN service users and that number would grow as the business grows for the PIN service provider. This restriction does not seem to serve any purpose.

Additionally, it is not clear how the number of PIN created by an AF would result into the number of PDU sessions for the PEGC (or PEMC). The AF may create PIN for different subscribers using different PEGC UEs and so there is no relation between the number of PIN create by an AF and number of PDU sessions of a PEGC UE. Moreover, a PEGC may serve multiple PINs and for each PIN there could be multiple PDU Sessions. The number of PDU sessions from a PEGC will be driven by the URSP (or PIN routing policies) provided to the PEGC UE.

KI#4 is about "Communication of PIN" and conclusion 17) does not relate to communication. It is more about authorization of AF request which probably is not suitable to be concluded in KI#4.

Proposal 3: For above reasons it is proposed to remove conclusion 17) as the conclusion seems wrong and not serving any purpose.

1.1	KI #4: whether more parameters are needed in non-3GPP QoS assistance information.

Current conclusion proposes:

2)	Non-3GPP QoS assistance information (including QoS characteristics, GFBR/MFBR UL/DL, MPLR UL/DL) that contains the same parameters as the Additional QoS Information specified in table 9.3.1.1-2 of TS 24.502 [9] may be sent to PEGC from SMF to assist the deriving of N3GPP QoS parameters for PIN.

a)	Whether and how PEGC performs the deriving of QoS parameters and mapping procedure to be applied between the PINE and the PEGC is implementation specific and therefore it is not specified by 3GPP.

The “Additional QoS Information" specified in clause 9.3.1.1 of TS 24.502, are related to parameters send in IKEv2 5G_QOS_INFO and they are related to the QoS to be applied in the N3GPP from the UE to the TNAP and NOT from the UE to the network behind.

- The 5G_QOS_INFO payload is used to indicate:

- a)	the PDU session identity;

- b)	zero or more QFIs;

- c)	optionally a DSCP value associated with the child SA;

- d)	whether the child SA is the default child SA; and

- e)	if trusted non-3GPP access, Additional QoS Information or if untrusted non-3GPP access, optionally Additional QoS Information.

The PIN conclusion states that

- Additional QoS Information specified in table 9.3.1.1-2 of TS 24.502 [9] may be sent to PEGC from SMF to assist the deriving of N3GPP QoS parameters for PIN

- Whether and how PEGC performs the deriving of QoS parameters and mapping procedure to be applied between the PINE and the PEGC is implementation specific and therefore it is not specified by 3GPP

Figure 1: PIN network

The PINE to PEGC is N3GPP connection and it can be WLAN, BT or any other N3GPP access technologies.

Now the table 1 below indicates whether the current specified as “Addition QoS Information" specified in clause 9.3.1.1 of TS 24.502 are applicable to the PIN scenario for devices connected behind the PEGC

| Additional QoS Information | Applicable to PIN, i.e. between PINE and PEGC? | Consideration |
| --- | --- | --- |
| A) the PDU session identity; | Applicable | It is the PDU session terminated to PEGC |
| b) zero or more QFIs | Applicable, but not useful | In this scenario refers to the QFI used in PDU session from PEGC to 5GS. The PEGC gets the QoS to be applied in PDU session, and since it map the QOS in PDU with the N3GPP access in a proprietary manner, this is a duplication of information and not needed |
| c) optionally a DSCP value associated with the child SA; | Not applicable | The child SA applies only to Untrusted/Trusted N3GPP |
| d) whether the child SA is the default child SA; and | Not applicable | There is not IKEv2 between PINE and PEGC. If the PINE support IKEv2 it means that it is a UE and it is connected directly to 3GPP via N3IWF, hence it is not a PINE |
| e)if trusted non-3GPP access, Additional QoS Information or if untrusted non-3GPP access, optionally Additional QoS Information | Not applicable | This is not a PIN scenario |

The mechanism to map the QoS related to the traffic in the PDU session with the QoS in the “transport layer” from the PEGC to the PINE is media specific and the PEGC can use specific implementation mechanism or procedure defined by the Specific N3GPP media to be applied for the mapping of traffic. The PCF is not aware of the specific N3GPP transport layer used between the PINE and the PEGC, hence it cannot provide any reasonable N3GPP QoS assistance information.

For pursuing the analysis consider which are the current parameters available in the UE are pre-defined or signaled QoS rule containing:

- a QoS flow identifier (QFI);

- optionally, a set of packet filter for UL and/or DL

The network can also provide the UE with one or more QoS flow descriptions associated with a PDU session at the PDU session establishment or at the PDU session modification, where each QoS flow description contains:

a)	a QoS flow identifier (QFI);

b)	if the flow is a GBR QoS flow:

1)	Guaranteed flow bit rate (GFBR) for UL;

2)	Guaranteed flow bit rate (GFBR) for DL;

3)	Maximum flow bit rate (MFBR) for UL;

4)	Maximum flow bit rate (MFBR) for DL; and

5)	optionally averaging window, applicable for both UL and DL;

c)	5QI, if the QFI is not the same as the 5QI of the QoS flow identified by the QFI;

The network provides also the Session-AMBR parameter.

The packet filters defined are :

- IP filters

  - IPv4 remote address type, IPv4 local address type

  - IPv6 remote address/prefix length type , IPv6 local address/prefix length type

  - Protocol identifier/Next header type

  - Ports - Single local port type, Local port range type, Single remote port type, Remote port range type

  - IKEv2 SPI Security parameter index type

  - Type of service/Traffic class type  (DSCP)

- Ethertype type filter

  - Destination MAC address range type

  - Source MAC address range type

  - VLAN Tag ( 802.1Q C-TAG VID type, 802.1Q S-TAG VID type)

  - Ethernet Priority marking (802.1Q C-TAG PCP/DEI type, 802.1Q S-TAG PCP/DEI type)

  - Flow label type

The UE/PEGC has the following information

- For each PINEs which is the associated PIN and the traffic that is received from/send to

- The PDU session and the QoS flow associated to the traffic map to/from PDU session

- Which is the actual Non-3GPP access that it is used and therefore which is the QoS capability for such Non-3GPP access supported by PEGC/UE and PINE, for example in case of WLAN whether WFA QoS profile is supported or not

Therefore the PEGC/UE based on its supported features can perform the mapping and translation between the DPU session Qos flow and the Non-3GPP access specific flow.

Proposal #2: Considering that:

- None of the current “Additional QoS Information" specified in clause 9.3.1.1 of TS 24.502 is applicable to PIN scenario

- Whether and how PEGC performs the deriving of QoS parameters and mapping procedure to be applied between the PINE and the PEGC is implementation specific and therefore it is not specified by 3GPP

- The UE has all information available for the generation of QoS flow in PDU session and so depending by the N3GPP access and UE capabilities supported each of the N3GP access which are outside 3GPP scope

It is concluded that:

- There is not need to add parameters in Additional QoS parameters

- The usage of Additional QoS parameters is technically not correct , hence it shall to be removed from the conclusion.

1.4	KI #4 and KI#5:	whether PEMC UE needs a specific 5G subscription for providing PIN service

Current conclusion proposes:

9)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs but one PIN will be served by one PDU session. (See PIN Session models as described in Annex A).

Editor's note:	It is FFS whether one PDU session can serve more than one PIN.

The PIN is designed for the personal IoT network, which means the different PIN may have different QoS requirement as concluded in subclause 8.4 bullet 4) for KI#4. Additionally, except the different QoS requirement, the different PIN may also have the different security level. For example, the user may configure one PIN for entertainment, the other PIN for the work which may have higher level security than the entertainment PIN. So considering the different QoS and security requirement for the different PIN, it is proposed to establish the different PDU session.

13)	PIN is a service that needs user subscribing from operator, the user's PIN service subscription is used by operator for policy configuration to PEGCs/PEMCs.

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

As concluded that the PIN is a service that needs user subscription from operator, PEMC as a member of PIN takes responsibility to manage the PIN, so it should be authorized by the operator. In order to authorize the PIN service, the normal solution is to extend the UE subscription data, such as the PIN ID added to the PEMC subscription data to indicate this PEMC belongs to this PIN. Additionally, PEMC will communication with PEGC via PIN related PDU session to transport PIN configuration policy, such as which PINE belongs to this PIN. Therefore the PIN subscription data is needed to authorize the PEMC to establish the PIN related PDU session.

11)	If AF for PIN is used, the AF may provide necessary parameters to 5GC which may be considered by PCF to generate the PIN Route Selection Policy for PDU Session selection by PEGC(s) and to generate the URSP accordingly for PEMC(s).

It is proposed to apply URSP for PEGC, because firstly the PEGC is also a UE as the PEMC, secondly the policy is to select the PDU session as the same policy as URSP. Therefore, no need to define another new policy, the URSP can be reused by the PEGC.

Proposed to remove the second editor’s note in the conclusion for KI#4.

It is concluded in conclusion of KI#4 that PEGC can map the PIN traffic/signalling data to the right PDU session (i.e. existing PDU session or new PDU session) based on existing mechanism or implementation. Everything works well with further enhancement.

Generally, PEGC can initiate any PIN services (e.g., video service, IMS service, voice service, etc.) which requires different PDU sessions based on existing mechanism. But there is no reason to limite that one PIN will be served by one PDU session. Whether all the services provided by one PIN can map into one PDU session, or multiple PDU sessions is up to the service requirements.

5) PDU session management functionality can be used by the PEGC. a) When the PEGC detects new packets (PIN signalling or PIN traffic or creation of PIN) from a device in the PIN, it may map the PIN or PIN packets to an existing PDU session or establish a new PDU session. The criteria for taking the decision can be based on existing mechanism or implementation. 9）A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs. but one PIN will be served by one PDU session (See PIN Session models as described in Annex A). Editor's note: It is FFS whether one PDU session can serve more than one PIN.

Proposal: remove the sentence “but one PIN will be served by one PDU session” in bullet 9) and remove the EN after bullet 9).

The TR conclusions currently have an open EN related to whether the PEMC UE needs a PIN specific 5G subscription to provide PIN Service.

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

Observation:

1. 	As part of the conclusions for KI#1 (Architecture enhancements) it has been agreed that legacy UE acting as PEMC needs to be considered. If a new subscription information is introduced for network to allow a UE to act as PEMC, this will have a UE impact also, as the UE has to understand whether the network has authorized the UE to act as a PEMC.

2.	As part of the conclusions for KI#3 (Management of PIN and PIN Elements), it has been agreed that "PEMC is responsible for PIN Element management (e.g. add/remove the PINE/PEGCs, etc.), which is out of SA2 scope.". Which means, although PEMC is performing management of the PIN elements, SA2 will not define any new functionality for PEMC UE to be able to perform PINE management. Since there is no new UE functionality defined, the ability for a device to manage of PIN or PINE does not need to be authorized by 5GC based on a new PIN specific subscription information.

Observation:

It is proposed to remove the EN and update the conclusions to remove any PIN specific subscription and authorization requirement from PEMC UE.

There is one EN under the conclusions for KI#5

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

For this EN, it seems clear based on the text below in clause 4.2 of the TR 23.700-68, i.e. PEMC is a UE with 5G subscription.

- Only a 3GPP UE can act as PEGC and/or PEMC.

In addition, as conclusions for KI#4, the subscription data for PEMC may be enhanced with PIN related information from AF for PIN.

Based on above, it’s proposed to support the conclusion updates below for KI#3.

In this paper, we introduce an use case related with the remaining technical issue for key issue #4 and propose to update the conclusion.

In order for PEGC to map the XR traffic from XR device (PINE), PEGC needs the traffic identification information of the XR device. The existing traffic identification information within the QoS rule is 5 or 6 tuples for IPv4, IPv6 or IPv4v6 PDU Session Type may not identify a specific PINE traffic when NAT is applied in PEGC. Because the QoS Rule is checked by the 3GPP modem after NAT is applied in PEGC at IP layer. In such a case, the PEMC informs the mapping of PINE identification and PINE traffic to the PEGC by the application layer between the PEMC and PEGC. To authorize such a QoS request, the SMF eventually should update QoS Rule including the PINE traffic identification to the PEMC.

ECN marking for L4S is the capability of the gateway. Since PEGC has the gateway capability, the PEGC need to support L4S if is agreed to be supported considering PIN architecture. Since L4S is supported in 5GC per QoS Flow-based, based on the current conclusion of FS_XRM KI#3, PEGC supporting L4S capability also support its related operation should support the feature per QoS Flow-based. If supported, the capability of PEGC support for L4S should be informed to the SMF and based on the capability, the SMF should send the L4S applicability of QoS Flow in QoS rule to the PEGC. This can be further discussed in line with the XRM normative progress.

Regarding the PIN Session model, one PDU Session support one PIN for simplicity in this release.

According to our understanding, the role of PEMC is solved by application layer, which is out of SA2 scope, therefore, no specified policy for PEMC is not required.

With the above reasons and understading, it is proposed to update the conclusion for the KI#4.

There are three ENs under the conclusions for KI#4

Editor's note:	It is FFS whether more parameters are needed in non-3GPP QoS assistance information.

For this EN, there is no further non-3GPP QoS assistance information discussion so far besides the information defined in 24.502. This EN can be removed from the TR.

Editor's note:	It is FFS whether one PDU session can serve more than one PIN.

For this EN, it shall be allowed to use one PDU session for different PINs. However, this shall be allowed in the pre-condition that there is no need of differentiation between PINs for the QoS control, routing steering in the 5GC.

Editor's note:	It is FFS whether PEMC UE needs a specific 5G subscription for providing PIN service.

For this EN, it’s addressed in conclusion for KI#5 and it can be removed in this KI#4.

Based on above, it’s proposed to support the conclusion updates below for KI#3.

2 Proposal

It is proposed to update 23.700-88 as follows:

FIRST CHANGE

8.4	Conclusion on Key Issue #4

The normative work is based on the following principles

1)	When the communication between a PEMC and a PINE behind a PEGC takes place via 5GC, or when the communication between PINEs requires multiple PEGCs and 5GC, the existing traffic forwarding functionalities in 5GS via UPF(s) or N6 can be applied if available.

2a)	PIN related QoS assistance information (including QoS characteristics, GFBR/MFBR UL/DL, MPLR UL/DL) that contains the same parameters as the Additional QoS Information specified in table 9.3.1.1-2 of TS 24.502 [9] may be sent to PEGC from SMF to assist the deriving of PIN related QoS parameters for PIN.

a)	How PEGC performs the deriving of QoS parameters and mapping procedure from 3GPP QoS rule(s) and QoS flow(s) to be applied to Non-3GPP access(es) between the PINE and the PEGC is implementation specific and therefore it is not specified by 3GPP.

b) 	PINE traffic identification assistant information (e.g. PINE identifier, IP address or Ethernet MAC address) may be included in the PINE related QoS assistant information.

c) Whether to indicate the PEGC support for L4S will be decided in normative phase in coordination with XRM.

3)	For traffic generated from PINE(s) behind a PEGC, differentiated traffic routing and QoS control may be supported by a PEGC with the PIN related QoS assistance information.

4)	If AF for PIN is used, the AF can request the 5GC to exposes capabilities in order for the AF to provision parameters for resources configuration/deconfiguration for a PIN e.g. QoS authorization for a PIN, QoS control for the PIN traffic, and routing control for the PIN traffic. The mechanism and criteria used by the AF to determine the need for a QoS modification for the PIN traffic are out of 3GPP scope.

5)	PDU session management functionality can be used by the PEGC.

a)	When the PEGC detects new packets from a device in the PIN, it may map the packets to an existing PDU session or establish a new PDU session. The criteria for taking the decision can be based on existing mechanism or implementation.

NOTE 1:	The procedure is the same used when application generating the traffic resides directly on the UE.

b)	The PEGC initiates PDU Session Establishment/Modification Request with necessary information:

i)	To enable 5GC to manage system resources related to the PDU Session .

ii)	To enable 5GC to provide differentiated QoS control for PIN traffic.

NOTE 2:	The AF relies on PIN signalling between the PINE/PEGC/PEMC and AF, which is transferred via UP transparently to the 5G system, to determine the need for a QoS modification.

6)	The procedure for supporting one PINE connected to multiple PEGCs in the same PIN and PINE to move between PEGCs is outside the 3GPP scope.

7)	PIN direct communication is not specified since it is implementation specific.

8)	PIN indirect communication via PEGC is managed within the PIN, which may be supported by 5GS.

9a)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs and in this case the PEGC shall have at least one PDU Session for each PIN. One PIN may be served by more than one PDU sessions in the PEGC. (See PIN Session models as described in Annex A).

9b)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs but one PIN will be served by one PDU session, and the different PIN is served by the different PDU session. (See PIN Session models as described in Annex A).

9c)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs. (See PIN Session models as described in Annex A).

9d)	A PEGC may establish a single or multiple PDU Sessions used for PIN communication. One PEGC may serve more than one PINs but one PIN will be served by single PDU session as default. (See PIN Session models as described in Annex A). Single PDU session may serve multiple PINs if there is no differentiation handling needs among different PIN traffics.

10)	IPv6 Prefix Delegation as described in clause 4.6.2.3 of TS 23.316 [5], or DHCP proxy by PEGC, or Framed Routing as described in clause 5.6.14 of TS 23.501 [2] are applied for IP address allocation of PINEs connected to PEGC.

NOTE 3:	Framed Route support will be further considered during normative work.

11)	If AF for PIN is used, the AF may provide necessary parameters to 5GC which may be considered by PCF to generate the URSP for PDU Session selection by PEGC(s) and to generate the URSP accordingly for PEMC(s).

12)	Routing of traffic from/to PDU session and the PIN elements is left to implementation

13)	PIN is a service that needs user subscribing from operator, the user's PIN service subscription is used by operator for policy configuration to PEGCs.

14)	UDR is enhanced to support the storage and retrieval of PIN related policy and QoS parameters.

15)	The N3GPP network delay between PINE and PEGC may be signalled from PEGC to PCF, and be taken into account when PCF derives the PDB value of QoS flow for PEGC.

16)	The 5G system support for anchoring PDU Sessions of PEGCs and PEMCs at same SMF based on a combination of DNN, S-NSSAI.

NOTE 4:	Other possibility without anchoring at same SMF may be determined in normative phase.

SECOND CHANGE

## 8.5	Conclusion on Key Issue #5

The following conclusions are agreed for Authorization for PIN:

1)	PIN application-level Authentication and Authorization of PIN and PIN Elements are not specified by SA WG2.

2a)	PEGC may be authorized by 5GC to provides PIN service via subscription in the UDM.

2b)	PEMC and PEGC are authorized by 5GC to provides PIN service via subscription in the UDM. The subscription data for PEMC and PEGC can be enhanced by authorization information for PIN from AF for PIN service.

3)	PEGC is subject to operator policies for any PIN services that operate over 5GC.

4)	A PINE is authorized by PEMC to join a PIN, which is outside the scope SA2.

5)	A PINE is allowed or disallowed to connect to a PEGC by the PEGC based on the provisioned information.

END OF CHANGES