3GPP TSG-SA2 Meeting #154AH	S2-2300972

# 16-20 January 2023, Elbonia               		  									(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4028 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

s

| Title: | Informative Annex on PIN Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PIN Architecture needs to be specified in TS 23.501 as per the conclusions in the TR 23.700-88. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative annex is added explaining the PIN Architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete PIN specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all text are new)

Annex Y (Informative):
Personal IoT Networks

# Y.1	PIN Architecture

## Y.1.1	PIN reference architecture

Figure Y.1.1-1: PIN reference architecture

Figure Y.1.1-1 shows the logical PIN reference architecture.

NOTE:	The PEMC may be collocated with PEGC.

## Y.1.2	Reference points

The 5G System Architecture for PIN contains below reference points in addition to the reference points defined in TS 23.501 [x].

P1:	Reference point between the PINE and the PEGC. This reference point is based on non-3GPP access (e.g. WIFI, Bluetooth) and is not specified by 3GPP.

P2:	Reference point between the PINE and the PEMC. This reference point is based on non-3GPP access (e.g. WIFI, Bluetooth) or relay path via the PEGC and is not specified by 3GPP.

P3:	Reference point between the PEMC and the PEGC. This reference point is based on non-3GPP access (e.g. WIFI, Bluetooth) or 5G ProSe Direct Communication or using user plane path over 5GC. This reference point is not specified by 3GPP.

P4:	Reference point between the PEMC and the PIN Application Server. This reference point can be based on the direct user plane path over the 5GS between PEMC and the PIN Application Server or relay path via the PEGC, or other communication path via Internet. This reference point is not specified by 3GPP.

P5:	Reference point between the PEGC and the PIN Application Server. This reference point is over the user plane path over 5GS between the PEGC and the PIN Application Server and is not specified by 3GPP.

END OF CHANGES