3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2300993r03

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0201 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on UE-to-UE Relay reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution proposes to update the UE-to-UE Relay reselection procedure. - It is not clear how the initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays and how the responding 5G ProSe End UE determines the new selected 5G ProSe UE-to-UE Relay. So this paper tries to clarify these issues. - On the EN, Editor's note: How the Link Modification procedure is aligned with the connection establishment procedures and IP address discovery is to be checked. The initating 5G ProSe End UE needs to set up PC5 connection with the new Relay to get the IP address via the new Relay, before responsing the Link Modification Ack message. This makes the procedure more complex. This paper proposes that the PC5 connection establishment and IP address discovery reuses the procedure defined in clause 6.7.1, which is independent and after the Relay reselection procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarify how the initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays and how the responding 5G ProSe End UE determines the new selected 5G ProSe UE-to-UE Relay. - To delete the EN, and proposes to reuse PC5 connection establishment and IP address discovery after the relay reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling on candidate/new 5G ProSe UE-to-UE Relay is not clear, and EN is not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.2, 6.7.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.7.4.2	5G ProSe Layer-2 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.2-1 is the procedure for the negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection.

Figure 6.7.4.2-1: 5G ProSe Layer-2 UE-to-UE Relay reselection

1.	A PC5 unicast link is established between the 5G ProSe End UEs via a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.2.

2.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays. The initiating 5G ProSe End UE may reveive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength, RSC within the UE-to-UE Relay Discovery Announcement message.

3.	The initating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe End UE which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s) and security information.

4.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message, and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 3, as the Destination Layer-2 ID to carry the discovery message. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

5.	The responding 5G ProSe End UE sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay and security information.

6.	5G ProSe End UEs set up PC5 unicast links, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.2, and the PC5 unicast is link established between 5G ProSe End UEs via the new 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs switch the data traffic via the new 5G ProSe UE-to-UE Relay. The security information is used to verify that the new link has been set up successfully.

NOTE 1:	The security information contents and usage will be defined by SA WG3.

NOTE 2:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

# * * * * Second change * * * *

#### 6.7.4.3	5G ProSe Layer-3 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.3-1 is the procedure for the negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection.

Figure 6.7.4.3-1: 5G ProSe Layer-3 UE-to-UE Relay reselection

1.	5G ProSe End UEs have set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

2.	The 5G ProSe End UEs have additionally set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

3.	The 5G ProSe End UEs are transferring traffic via the 5G ProSe UE-to-UE Relay.

4.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength, RSC within the UE-to-UE Relay Discovery Announcement message.

5.	The initating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe UE-to-UE Relay, which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), the IP addresses of the responding 5G ProSe End UEs and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

Multiple 5G ProSe End UEs IP addresses may be included when the initiating 5G ProSe End UE is communicating with multiple 5G ProSe End UEs via the 5G ProSe UE-to-UE Relay.

6.	5G ProSe UE-to-UE Relay determines the responding 5G ProSe End UE based on the IP address received from the initiating 5G ProSe End UE and sends a Link Modification Request message to the responding 5G ProSe End UE. The Link Modification Request message includes a Relay re-selection indication, User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), IP address of the initiating 5G ProSe End UE, and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

If multiple 5G ProSe End UEs are specified in the Link Modification Request message received from the initiating 5G ProSe End UE, the 5G ProSe UE-to-UE Relay sends a PC5 Link Modification Request to each of the 5G ProSe End UEs.

7.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message, and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 4, as the Destination Layer-2 ID to carry the discovery message. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

The responding 5G ProSe End UE set up PC5 unicast link, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing steps 3-11 of the 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay procedure defined in clause 6.7.1.

8.	The responding 5G ProSe End UE sends a Link Modification Accept message to the 5G ProSe UE-to-UE Relay, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay, and Relay re-selection indication.

9.	5G ProSe UE-to-UE Relay sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay and Relay re-selection indication.

The initiating 5G ProSe End UE set up PC5 unicast link, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.1.

10-11.	Link Modification Ack is sent from the initating 5G ProSe End UE to the responding the 5G ProSe End UE via the 5G ProSE UE-to-UE Relay, including the IP address of the initiating 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay and Relay re-selection indication.

12.	The 5G ProSe End UEs transfer traffic via the newly selected 5G ProSe UE-to-UE Relay.

NOTE:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

# * * * * End of changes * * * *