3GPP TSG-SA WG2 Meeting #154AH-e	S2-2301011

# E-Meeting, 16th – 20th January 2023

Title:	[draft] LS on partially allowed/rejected S-NSSAI and network slices which have Area of Service not matching Tracking Areas

Response to:	None

Release:	Rel-18

Work Item:	eNS_Ph3

Source:	SA2

To:	RAN3

Cc:	None

Contact Person:

Name:	alessio casati

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2, as part of eNS_Ph3 in rel-18, is introducing the concepts of partially allowed S-NSSAI and Partially rejected s-NSSAI in order to support Registration areas where S-NSSAIs are not uniformly supported. This is their definition.

Partially Allowed S-NSSAI: S-NSSAIs value identifying a network slice that is not uniformly supported in a RA. The UE can consider the S-NSSAI allowed inside an area smaller than the Registration area, where this S-NSSAI is indicated to be supported to the UE. The UE needs not perform a mobility registration update when entering and exiting this area of support inside the RA.

Partially Rejected S-NSSAI: S-NSSAIs value of a network slice that is not uniformly supported in a RA and not in the cell where the UE receives this S-NSSAI indication. The UE, upon mobility, can request this S-NSSAI if it camps in a cell inside an area smaller than the Registration area, where this S-NSSAI is indicated to be supported to the UE.

SA2 intends to provide the RAN with visibility on these S-NSSAIs so the RAN can Optimise RRM (e.g. steer the UE to cells where the rejected or partially allowed S-NSSAI is also supported).

Also, for the partially allowed S-NSSAI, to trigger PDU session handling policies for the sessions of the network slice (e.g. locally avoid data transmission outside the area of support of the slice or e.g. deactivate the PDU session, or trigger reporting of entering or exiting a S-NSSAI area of support to AMF). the exact mechanism is for further study but RAN opinion on feasibility of various mechanisms is required.

Separately, SA2 is also working on specifying the handling of network slices that have Area of Service (AoS) not matching deployed Tracking Areas. it is discussed that it is possible that these are indicated as supported uniformly over the TA to CN so legacy UEs can be supported, but actually in some cells of the TA zero resources may be allocated. RAN feedback is welcome then on the expected handover behaviour when zero resources are allocated but the network slice is indicated as supported in the cell:

-  	Should the PDU sessions be handed over anyhow to a cell where its network slice has zero resources configured? i.e. can the PDU session context and DRBs that require no admission control be retained upon connected mode mobility? Would it be possible to then not schedule the PDU session in UL and drop DL packets and report UL the dropped data to CN (conceptually similar reporting to reporting NR data used to CN in EPS NR Option 3)

- 	if Area of Interest reporting is configured to let the CN know when the UE is outside the area where S-NSSAI AoS  can the AoI be identified by the S-NSSAI instead of a AoI ID? (i.e. can the AoI be enhanced to use S-NSSAI as AoI identifier?). This could avoid a tedious task of mapping S-NSSAIs to AoI IDs

- 	Can the RAN trigger, if configured to do so, release or deactivation of PDU sessions according to policy as the UE is moving to an area where 0 resources are allocated to its network slice?

-

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks RAN3 to provide feedback on theabove

3. Date of Next SA2 Meetings:

Next SA2 Meetings dates