SA WG2 Meeting #154AHE	S2-2301014

Elbonia, January 16 – 20, 2023	(revision of S2-230xxxx)

Source:	Samsung

Title:	KI#3: Update on Conclusion

Document for:	Approval

Agenda Item:	9.25.1

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: This contribution proposes to update the conclusion for KI#3 to address the first Editor’s Note.

1.	Discussion

This contribution proposes to resolve the EN in the conclusion for KI#3 in TR 23.700-85 in below:

Editor's note: It is FFS how to handle the EPS mobility for a UE that is dual registered (i.e. with a registration in 5GS via N3IWF and attached to EPC/E-UTRAN) and thus whether UE policy association can be maintained in both EPS and 5GS or only in 5GS or EPS.

For the UE first attached to MME and established PDN Connection in EPS, the UE Policy Association is established between the SM-PCF and the UE-PCF. After then, if the UE requests 5GS registration to AMF and the AMF will perform PCF selection for AM Policy Association and UE Policy Association. The selected PCF for AMF may be a different with the UE-PCF which was selected and established the UE Policy Association in EPS. This situation can cause confusions to the UDR that which UE-PCF should be notified when the policy related subscription data changes, but also can cause conflicts when each UE-PCF invokes UE Policy Association Modification/Release procedure with different content in each side.

In order to avoid those confusions and conflicts, the following option is proposed which can resolve this EN with minimum impacts on both UE and network as below:

During 5GS Registration, the selected PCF for AMF (UE-PCF) queries BSF to discover the UE-PCF which serves the UE Policy Association in EPS if any. If there is existing serving UE-PCF in EPS, then the selected PCF for AMF (UE-PCF) provides the existing UE-PCF ID received from BSF to the AMF and requests to relocate the AM Policy Association to the existing UE-PCF and establish the UE Policy Association with the existing UE-PCF.

The above option does not require AMF to support BSF querying, nor does not require UE to signal again to EPC to complete the registration procedure.

2.	Proposal

It is proposed to capture the following changes into TS 23.700-85.

# * * * * First change * * * *

8.3	Conclusion on KI#3:

-	The ePCO based URSP delivery to UE in EPS is selected as baseline mechanism for normative work.

-	In order to avoid unnecessary signalling overload for the purpose of delivery URSP to pre-Rel-18 UEs in EPS, the UE needs to indicate the URSP Provisioning Support Indication in EPS in the UE Policy Container to UE-PCF in 5GS.

-	During 5GS to EPS mobility with N26, the AMF doesn't initiate the UE Policy Association termination procedure, instead the AMF will request the UE-PCF to delay the termination of UE Policy Association for a configured period. The UE-PCF can initiate the UE Policy Association Termination procedure either after it receives the UE Policy Association Establishment from SM-PCF or after the configured period. The mobility procedure from 5GS to EPC described in clause 6.16.2.3.3 is selected as the baseline procedure for 5GS to EPS mobility with N26.

-	AMF doesn't provide any new information to MME during 5GS to EPS mobility with N26, the SM-PCF determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The SM-PCF discovers the address of UE-PCF serving the UE by querying BSF. The UE-PCF recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS handover").

-	When the UE is attached to EPS, the SM-PCF can retrieve the PCRTs for UE Policy from UE-PCF and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

-	When the UE returns from EPS to 5GS, the old UE policy association in EPS should be terminated after the new UE policy association established in 5GS.

-	When the UE does Initial Attach to EPC, in the default PDN connection is requested to established, the UE includes the UE Policy Container ePCO in PDN Connectivity Request encapsulated in Attach Request; otherwise, the UE will include the UE Policy Container ePCO in the PDN Connectivity Request during the first request for PDN connectivity as described (i.e. as described in Sol#16). The PDN Connection used by UE and SMF/PGW-C to convey UE Policy Container ePCO shall be kept when the UE is in the CONNECTED mode.

-	The PCF triggers the re-evaluation of applicable URSPs for the UE and determines an update of URSP is needed for the UE when an event as described in clause 6.16.11 happens in PCF. The PCF selects one of the PDN connection/PDU sessions associated to EPC for the delivery of the URSP update. Then the PCF generates the corresponding UPDP message MANAGE UE POLICY COMMAND in a similar way than it is done in 5GC and then includes the message into a new IE for sending it to the SMF over N7, then further to UE as described in clause 6.16.2.2.

-	During 5GS Registration, the selected PCF for AMF (UE-PCF) may decide to re-use the UE Policy Association established in EPS when it receives URSP Provisioning Support Indication in EPS included in UE Policy Container from the UE (at step 17a of Figure 4.11.1.3.3-1 in TS 23.502). If the selected PCF for AMF (UE-PCF) decided to re-use the existing UE Policy Association then it queries BSF and get the UE-PCF ID selected in EPS. The selected PCF for AMF (UE-PCF) provides the existing UE-PCF ID received from BSF to the AMF and requests to relocate the AM Policy Association to the existing UE-PCF and establish the UE Policy Association with the existing UE-PCF. If the selected PCF for AMF (UE-PCF) decided not to re-use the existing UE Policy Association, then the UE-PCF selected in EPS can the UE Policy Association Termination procedure when it is notified to deregistered from the BSF.

Editor's note:	It is FFS whether URSP mapping in EPS requires any dynamic configuration mechanism (e.g. as proposed in Sol #17) to address the aspects identified in the 2nd bullet of the KI#3 description.

# * * * * End of changes * * * *