3GPP TSG-SA2 Meeting #154AH	S2-2301065r01

# 16-20 January 2023, Elbonia               		  									(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3834 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

s

| Title: | Delayed indication of RRC-Inactive with long eDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NG-RAN option to hold back the indication of RRC-Inactive with long eDRX to the CN until the UE is (surprisingly) paged is documented, but there is no solution to synchronising the NG-RAN and the AMF timing in that case since the eDRX parameters only express the full eDRX cycle length. If the NG-RAN delays the indication of the ongoing eDRX cycle to the AMF, then it needs to include sufficient information to synchronise the RAN and CN understanding of the ongoing eDRX cycle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If NG-RAN chooses to not indicate the RRC-Inactive with long eDRX to the AMF immediately when moving the UE to RRC-Inactive state, then the NG-RAN shall include the remining eDRX cycle parameter in addition to the full eDRX cycle length. This additions solves one of the editor's notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_RedCap_Ph2 feature is incomplete and the delayed indication of the RRC-Inactive with long eDRX does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE – removed from the CR to avoid collision with S2-2300221

NEXT CHANGE

#### 4.8.2.2b	Network Triggered Connection Resume in RRC Inactive with CN based MT communication handling

When the UE is in CM-CONNECTED with RRC_INACTIVE state with CN based mobile terminating (MT) communication handling, high latency communication as described in clause 5.31.8 of TS 23.501 [2] is applied.

This procedure may be triggered by MT data, or a N1 procedure from SMF and UPF as shown in Figure 4.8.2.2b-1. When the procedure is triggered by other NFs (e.g. SMSF, LMF, GMLC), the UPF (or SMF) in the following figure should be replaced by the respective NF (the corresponding service operations used by other NFs when they communicate with AMF may also be different from the service operations used by SMF/UPF).

During the procedure, the NG-RAN (i.e. gNB) performs RAN paging towards the UE based on the N2 message from the AMF in order to trigger the UE triggered Connection Resume procedure in clause 4.8.2.2.

Figure 4.8.2.2b-1: Network Triggered Connection Resume for UE in RRC Inactive with CN based MT communication handling

1a.	When downlink data is received and the SMF/UPF is requested to perform buffering as specified in clause 4.8.1.1a, the UPF/SMF checks with AMF for the possibility of data delivery, similar to step 2 of clause 4.24.2 with the following differences:

-	The AMF determines if the UE is reachable based on the stored eDRX cycle value and the remaining time in current eDRX cycle, if applicable and not already expired, for RRC_INACTIVE state provided by NG-RAN in clause 4.8.1.1a. If the UE is unreachable, the AMF provides the Estimated Maximum Wait time in the response message based on the eDRX cycle value and the remaining time in current eDRX cycle, if applicable and not already expired, for RRC INACTIVE in AMF (steps 2-5 are skipped). If the UE is considered reachable, step 2 is triggered.

NOTE:	This handling is similar to CM-IDLE with eDRX. When the AMF provides the Estimated Maximum Wait time, it can consider the time needed for RRC level procedures (e.g. RRC RNA update procedure) when UE wakes up from the eDRX cycle.

2.	The AMF sends an N2 message to NG-RAN with the request for the UE to be transitioned to RRC_CONNECTED.

Editor's note:	Whether the N2 message to be used is a new message or an existing message, e.g. N2 notification, needs coordination with RAN WGs.

3.	NG-RAN performs RAN paging towards the UE.

4.	When the UE receives RAN paging, it initiates the UE triggered Connection Resume procedure and NG-RAN notifies CN as specified in clause 4.8.2.2 including the N2 Notification in step 3b.

5.	The UPF triggers downlink data delivery if there is any. The AMF sends downlink NAS messages if there is any.