3GPP TSG SA WG2 Meeting #154AHE	S2-2301075r03

# January 16 - 20, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1263 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture of IMS supporting data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the conclusion of FS_NG_RTC, IMS network architecture and procedures need to be enhanced to support data channel services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new annex is proposed to document IMS network enhancement to support data channel services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex XX (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex XX (normative):
Support of Data Channel services in IMS

# XX.1	General

This annex describes IMS architecture enhancements to support data channel services. This annex is applicable to E-UTRAN and NG-RAN as IP-Connectivity Access Network to access IMS.

Data channels are established in the context of an IMS MMTel session. Standalone data channels without accompanying media in an IMS MMTel session are not supported in this Release of the specification.

Service based interfaces are specified in Annex AA.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A new service based function, Data Channel Signalling Function (DCSF), is introduced. There DC media function can be provided in two ways:

-	by enhancing the existing IMS MRF (called enhanced MRF) to perform media functions related to DC, which interacts with the TAS via enhanced Mr'/Cr interfaces; and

-	by introducing a new service based function called Data Channel Media Function (DCMF), which interacts with the TAS via the service based interface DC2.

# XX.2	Architecture and functions

### XX.2.1	Architecture

Figure XX.2-1 shows the data channel architecture when using the service based media function DCMF.

Figure XX.2-1: Architecture option of IMS supporting DC usage with DCMF

Figure XX.2-2 shows the data channel architecture when using the enhanced IMS MRF.

Figure XX.2-2: Architecture option of IMS supporting DC usage with enhanced MRF

NOTE 1:	As a deployment option, the DCAR can be part of the DCSF.

NOTE 2:	The DC Application Server is out of scope of 3GPP.

NOTE 4:	MDC2 may connect different IMS security domains.

NOTE 5:	DC4 and DC5 reference points are not specified in 3GPP.

NOTE 6:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 7:	DC3 reference point is not specified in this release.

NOTE 8:	Enhancements to the N33 reference point for support of data channel services are not specified in this release.

Editor's note:	Enhancements to other interfaces (e.g., N71/Sh, N70/Cx, Gm, Mw, ISC, Iq, Mb) for support of IMS data channel service are FFS.

### XX.2.2	Functional entities

#### XX.2.2.1	Data Channel Signalling Function (DCSF)

The DCSF is the signalling control function to provide data channel control logic. The DCSF is not involved in SIP/SDP negotiation. The DCSF provides the following functionalities:

-	The DCSF receives event reports from the TAS and decides whether data channel service is allowed to be provided during the IMS session;

-	The DCSF manages bootstrap data channel and (if applicable) application data channel resources at the DCMF or enhanced MRF via the TAS;

-	The DCSF supports HTTP web server functionality to download data channel applications (bootstrapping) via DCMF and/or enhanced MRF to the UE based on UE subscription.

-	The DCSF may download data channel applications from the Data Channel Application Repository;

NOTE 1:	Procedures to download data channel applications to the UE are not specified by 3GPP.

-	The DCSF interacts with NEF for data channel capability exposure via N33 and MDC3;

NOTE 2:	Interaction with NEF is not specified in this release of the specification.

-	The DCSF interacts with the DC Application Server for DC resource control and traffic forwarding via MDC3.

NOTE 3:	Interaction with the DC Application Server is not specified by 3GPP.

#### XX.2.2.2	Data Channel Application Repository (DCAR)

The Data Channel Application Repository stores data channel applications which are retrieved by the DCSF when required.

NOTE:	The Data Channel Application Repository is out of scope of 3GPP.

#### XX.2.2.3	Data Channel Media Function (DCMF) / enhanced MRF

The DCMF and enhanced MRF provide the media resource management and forwarding of data channel media traffic. The DCMF and enhanced MRF have the following functionalities:

-	The DCMF and enhanced MRF manage the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the TAS;

-	The DCMF and enhanced MRF terminate the bootstrap data channel from the UE and forward HTTP traffic between the UE and DSCF via MDC1;

-	The DCMF and enhanced MRF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs;

-	The DCMF and enhanced MRF relays traffic on the A2P/P2A application data channel between the UE and theDC Application Server via either MDC2 or MDC1, if the DC Application Server is collocated with the DCSF.

#### XX.2.2.4	TAS

The TAS is enhanced to support the following functionalities:

-	The TAS interacts with the DCSF via DC1 for event notifications;

-	The TAS receives the data channel control instructions from the DCSF and accordingly interacts with the DCMF via DC2 or with enhanced MRF via Mr'/Cr for data channel media resource management;

-	The TAS proceeds IMS procedures by correlating the data channel management procedures.

### XX.2.3	Reference points

The following service based reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the TAS.

-	DC2:	Reference point between the TAS and DCMF.

-	DC3:	Reference point between the DCSF and NEF.

NOTE 1:	DC3 reference point is not specified in this release.

The following reference points are usedfor data channel media handling:

-	Mb:	Existing reference point used for transport of data channel media between DCMF/enhanced MRF and IMS-AGW.

-	Mr'/Cr:	SIP based reference point between TAS and enhanced MRF.

-	MDC1:	Reference point for transport of data channel media between data channel media function (either DCMF or enhanced MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function (either DCMF or enhanced MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

NOTE 2:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

**** End of Changes ****