3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301099r04

# Janurary 16 – 20, 2023, Elbonia		(revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4061 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove ENs for QoS monitoring on dynamic satellite backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSATB |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is a EN left in the spec: “Editor's note: In this case, whether PCF can request QoS monitoring for the backhaul only, i.e. without receiving delay values including RAN part delay, is FFS.” According to the discussion, there are two opinions on QoS monitoring for dynamic satellite backhaul: 1. When the PCF receives the satellite backhaul category indicating dynamic satellite backhaul, it triggers per QoS Flow per UE QoS Monitoring to monitor E2E delay; 2. When the PCF receives the satellite backhaul category indicating dynamic satellite backhaul, it triggers the SMF to monitor N3 delay only, i.e., GTP-U path monitoring. This CR revision documents the first approach by re-using the already existing procedure by reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description to let the PCF request monitoring of E2E delay. and remove the EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved issues exist, and QoS monitoring for dynamic Satellite Backhaul can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.43.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.43.5	QoS monitoring when dynamic Satellite Backhaul is used

If dynamic satellite backhaul is used, QoS monitoring can be used to measure packet delay as specified in clause 5.33.3.

If the Satellite backhaul category received from SMF indicates dynamic satellite backhaul is used, the PCF may request QoS monitoring for the packet delay between UE and PSA UPF as specified in clause 5.33.3

.

# * * * * End of changes * * * *