3GPP TSG-SA2 Meeting # 154AHE (e-meeting)	S2-2301117r03

# Elbonia,16th – 20th January 2023													(new CR)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4069 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Service Function Chaining in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Nokia, Nokia Shanghai Bell, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SFC |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion in TR 23.700-18, the changes listed below are proposed- - Introduce new sub-clause 5.6.X to describe high level summary on Service Function Chaining support in 5GS. - Update Table 5.6.7-1, Metadata in clause 5.6.7.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new clause for Service Function Chaining describing how how to enble AF to influence 5G traffic steering to support SFC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SFC feature not supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.6.X	Support for Service Function Chaining

The content of this clause applies to non-roaming and to Home Routed roaming scenario i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.  In the non-roaming scenario, Application Function influence on Service Function Chain and Application Function influence on traffic routing (as defined in clause 5.6.7.1) can be applicable to the same traffic simultaneously.

It is assumed that a service level agreement exists between the operator and a third party that includes a list of authorized predefined Service Function Paths (SFPs), each SFP being identified based on the Service Function Chaining policy identifier (SFC ID). The AF may request the selected traffic flows to be steered towards a specific SFP, either at PDU Session establishment or any time after PDU Session establishment.

The AF requests are sent to the PCF via the NEF. When SFC ID is included in the AF request, the following parameters only are included in the AF request –, Traffic Description, Target UE (a UE, a group of UE or any UE) identifier(s), Spatial Validity Condition, AF transaction identifier and Metadata. The PCF based on the SFC ID received from the AF evaluates whether the SFC ID correspond to an authorized SFC policy for the AF. Based on the SFC ID received from the AF, the PCF derives the TSP IDs that can be different for uplink and downlink directions that apply to PDU Session and sends the TSP IDs and optionally Metadata (as provided by the AF) to the SMF as part of the PCC rules. The SMF provides the TSP IDs and optionally Metadata to the UPF via N4 in a FAR.

The UPF applies traffic steering mechanism based on TSP IDs. The UPF serving as PSA uses the TSP ID to steer traffic over N6 and the TSP ID identifies a specific Service Function Path in the SFC.

NOTE 1: It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

NOTE 2: The existing user plane mechanisms (e.g., VXLAN, NSH, GENEVE, GRE, VLAN, etc.) defined at IETF are reused as applicable by the PSA UPF to support SFC.

-

-