3GPP TSG-SA2 Meeting # 154AHE (e-meeting)	S2-2301117r04

# Elbonia,16th – 20th January 2023													(new CR)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4069 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Service Function Chaining in 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Nokia, Nokia Shanghai Bell, Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SFC |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion in TR 23.700-18, the changes listed below are proposed- - Introduce new sub-clause 5.6.X to describe high level summary on Service Function Chaining support in 5GS. - Update Table 5.6.7-1, Metadata in clause 5.6.7.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new clause for Service Function Chaining describing how how to enble AF to influence 5G traffic steering to support SFC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SFC feature not supported in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.6.X	Support for Service Function Chaining

#### 5.6.X.1 General

Traffic Steering Control can be used to activate/deactivate traffic steering policies for the purpose of service function chaining, i.e. steering subscriber's traffic flows to appropriate operator or 3rd party service functions (e.g. NAT, antimalware, parental control, DDoS protection) in the N6-LAN.

The content of this clause applies to non-roaming and to Home Routed roaming scenario i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.

The PCF controls traffic steering by provisioning and modifying traffic steering control information in PCC rules as described in TS 23.503 [45], e.g. clause 6.1.3.14. In the case of service function chaining, the traffic steering control information consists of a traffic description and, a reference to a traffic steering policy that is configured in the SMF/UPF.

The SMF provides instructions to UPF for traffic steering as further detailed in clause 5.8.2.11. The UPF applies traffic steering mechanism based on TSP IDs. The UPF serving as PSA performs deployment specific actions as configured for that the TSP ID. Based on the configuration, the UPF may for example perform encapsulation with packet marking where, for the traffic identified by the PDR, the UPF steers traffic over N6. .

NOTE 1: It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

For enforcing the traffic steering policy, the UPF may support traffic steering related functions and user plane encapsulation protocols that are out of 3GPP scope (e.g. as defined by other standards organizations).

NOTE 2: The existing user plane mechanisms (e.g., VXLAN, NSH, GENEVE, GRE, VLAN, etc.) defined at IETF are reused as applicable by the PSA UPF to support SFC.

The mechanism used for forwarding the traffic between the service functions within the N6-LAN is out of 3GPP scope.

An AF may request the use of a preconfigured service function chain.

#### 5.6.X.2 Application Function influence on Service Function Chaining

An AF may request the steering of user plane traffic to a pre-configured chain of service functions on N6-LAN.

In the non-roaming scenario, Application Function influence on Service Function Chain and Application Function influence on traffic routing (as defined in clause 5.6.7.1) can be applicable to the same traffic simultaneously.

It is assumed that a service level agreement exists between the operator and a third party that includes a list of authorized predefined Service Function Chains (SFCs), each SFC being identified based on the Service Function Chaining policy identifier (SFC ID). The AF may request the selected traffic flows to be steered towards a specific SFC, either at PDU Session establishment or any time after PDU Session establishment.

The AF requests may be sent to the PCF via the NEF. When SFC ID is included in the AF request, the parameters listed in Table 5.6.X.2-1 may be included in the AF request.

Table 5.6.X.2-1: Information element contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic to be influenced, represented by the combination of DNN and optionally S-NSSAI, and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier, instead of combination of DNN and optionally S-NSSAI. | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier (NOTE 2), or any UE accessing the combination of DNN and S-NSSAI. | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UE. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| SFC identifier(s) | Indicates the pre-defined Service Function Chain(s) in downlink and/or uplink. | N/A | Mandatory |
| Metadata | Contains information that is passed to UPF and provided by UPF to the service functions in N6-LAN. |  | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. |  |  |  |

The PCF based on the SFC ID received from the AF evaluates whether the SFC ID correspond to an authorized SFC policy. Based on the SFC ID received from the AF, the PCF derives the TSP IDs that can be different for uplink and downlink directions that apply to PDU Session and sends the TSP IDs and optionally Metadata (as provided by the AF) to the SMF as part of the PCC rules.

When the PCC rule is activated or updated with the traffic steering policy identifier(s) for AF influenced Traffic Steering Enforcement Control information, N6-LAN Traffic Steering Enforcement Control information or both, the SMF

derives the forwarding policy identifier (uplink and/or downlink) within the FAR(s) based on the authorized traffic steering policy identifier(s) in the PCC rule.

The SMF provides the Forwarding Policy and optionally Metadata to the UPF via N4 in a FAR as described in clause 5.8.2.11.

The UPF applies traffic steering mechanism based on TSP IDs. The UPF serving as PSA uses the TSP ID to steer traffic over N6 and the TSP ID identifies a specific Service Function Path in the SFC.

NOTE 1: It is assumed that all UPFs in the operator network serving as PSA for the DNN/S-NSSAI/DNAI subject to SFC control need to be configured with the same traffic steering information for SFC processing.

NOTE 2: The existing user plane mechanisms (e.g., VXLAN, NSH, GENEVE, GRE, VLAN, etc.) defined at IETF are reused as applicable by the PSA UPF to support SFC.

* * * End of Changes * * *