3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301129

# 16 – 20 January, 2023, Electronic Meeting					                   (revision of S2-23XXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0202 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-to-UE Relay operation during ProSe Direct Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support Source UE and Target UE to indicate support of UE-to-UE Relay operation that in the conclusion of KI#1 in TR 23.700-33 as following NOTE. NOTE 3: Whether and how a Source UE and a Target UE indicate support of UE-to-UE Relay operation will be determined in normative phase. As in 5G ProSe UE-to-UE Relay Model A discovery, the 5G ProSe UE-to-UE Relay needs to broadcast list of the User Info ID of other UEs in proximity (e.g. obtained via a previous direct discovery or direct communication procedures). However there is no clear confirmation from the other UEs before broadcasting the User Info ID of other UEs. So there would be provicy issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To add “support of UE-to-UE Relay operation” in the Annoucement message of 5G ProSe direct discovery with Model A - To add “support of UE-to-UE Relay operation” in the solicitation and response message of 5G ProSe direct discovery with Model B - Add description that clear indication that support of UE-to-UE Relay operation from the other UEs when 5G ProSe UE-to-UE Relay obtains the User Info ID of other UEs in proximity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would be the provicy issue when broadcasting other UE’s user info ID during UE-to-UE Relay Model A discovery without clear indication from other UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1; 6.3.2.4.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

#### 6.3.2.1	General

A PC5 communication channel is used to carry the discovery message over PC5 and the discovery message over PC5 is differentiated from other PC5 messages by AS layer.

Both Model A and Model B discovery as defined in TS 23.303 [3] are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Depicted in figure 6.3.2.1-1 is the procedure for 5G ProSe Direct Discovery with Model A.

Figure 6.3.2.1-1: 5G ProSe direct discovery with Model A

1.	The Announcing UE sends an Announcement message. The Announcement message may include the Type of Discovery Message, ProSe Application Code or ProSe Restricted Code, Relay_Indication, , security protection element, [metadata information]. The Application layer metadata information may be included as metadata in the Announcement message.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Announcement message are specified in clause 5.8.1.2 and clause 5.8.1.3.

Announcing UE includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery.

The Monitoring UE determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.8.1.2.

Depicted in figure 6.3.2.1-2 is the procedure for 5G ProSe Direct Discovery with Model B.

Figure 6.3.2.1-2: 5G ProSe direct discovery with Model B

1.	The Discoverer UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message, ProSe Query Code, relay_Indication, security protection element.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Solicitation message are specified in clause 5.8.1.2 and clause 5.8.1.3.

How the Discoveree UE determines the Destination Layer-2 ID for signalling reception is specified in clause 5.8.1.2.

Discoverer UE includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery.

2.	The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message, ProSe Response Code, Relay_Indication, security protection element, [metadata information]. The Application layer metadata information may be included as metadata in the Response message.

Discoveree UE includes relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery.

The Source Layer-2 ID used to send the Response message is specified in clause 5.8.1.3. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

NOTE:	Details of security protection element will be defined by SA WG3.

**** NEXT CHANGE ****

##### 6.3.2.4.2	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model A

Depicted in Figure 6.3.2.4.2-1 is the procedure for 5G ProSe UE-to-UE Discovery with Model A.

Figure 6.3.2.4.2-1: 5G ProSe UE-to-UE Relay Discovery with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered other UEs in proximity (e.g. via a previous direct discovery). The 5G ProSe UE-to-UE Relay obtains the User Info ID of other UEs in proximity.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, list of User Info ID of the 5G ProSe End UEs and RSCs, and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

The 5G ProSe UE-to-UE Relay shall only announce User Info IDs of other UEs in proximity which provided relay_indication when they were previously discovered.

A 5G ProSe End UE monitors announcement messages from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1

****LAST CHANGE ****