3GPP TSG-SA WG2 Meeting #154AHE	S2-2301164r03

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0179 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the conclusion for Key Issue #1 "MBS session reception in RRC Inactive" in TR 23.700-47: - When an RRC_INACTIVE UE is in the process of receiving ongoing MBS session data and moves to a new cell within the RNA, the UE shall be able to receive MBS session data. - When the UE moves outside the RNA, the UE performs UE Triggered Connection Resume in RRC Inactive procedure to the target RAN node as per existing procedures in TS 23.502 [3]. - When the UE receives the MBS data in RRC Inactive state and move out of the RNA area but Connection resume fails, it follows existing procedures and transition to CM-IDLE. When the UE transition to CM-IDLE since is not able to receive the MBS multicast data at the new cell, the UE initiates the mobility registration update or Service Request procedure and activates the associated PDU session, so that the shared tunnel (if not already established) or the individual delivery can be established towards the NG-RAN node for multicast data delivery to the UE. - When the UE moves outside the RA, the UE performs mobility registration procedure and as per the existing procedure in clause 5.3.3.2.5 of TS 23.501 [2] for "Mobile initiated NAS signalling procedure" for UEs in CM-CONNECTED with RRC Inactive state, the UE will resume the RRC Connection. - During the handover procedure, the SMF includes the "MBS assistance information for RRC Inactive" if any in N2 SM Info and sent to NG-RAN via AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility procedures for UEs receiving multicast MBS session data in RRC Inactive state is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.Y (new), 7.2.3.Y.1 (new), 7.2.3.Y.2 (new), 7.2.3.Y.3 (new), 7.2.3.Y.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 7.2.3.Y	Mobility of UE in RRC_INACTIVE state receiving multicast MBS session data

##### 7.2.3.Y.1	General

For a UE that is receiving multicast MBS data in CM-CONNECTED with RRC_INACTIVE state, the UE may move as follows:

- 	UE moves to new cell within the RAN Notification Area

- 	UE moves outside the current RAN Notification Area but within the current Registration Area

-	UE moves out of the current Registration Area

##### 7.2.3.Y.2	UE mobility within RNA

When a UE in RRC_INACTIVE is receiving ongoing MBS session data and moves to a new cell within the RNA, the UE shall be able to continue to receive the MBS session data.

Editor's note:	The procedure of UE mobility within RNA is is to be specified RAN WGs and alignment with RAN will be done later.

##### 7.2.3.Y.3	UE mobility out of RNA and within RA

Whena UE in RRC_INACTIVE state is receiving multicast MBS session data, if the UE moves out of the RNA and within the RA, the NG-RAN resumes the connection in the same way as specified in clause 7.2.3.7.

##### 7.2.3.Y.4	UE mobility out of RA

When the UE in RRC_INACTIVE receiving multicast MBS session data,  if the UE moves out of the RA, the UE initiates the Mobility Registration Update procedure as specified in clause 4.2.2.2.2 of TS 23.502 [6]. The Registration Request message contains the PDU session ID(s) of the previously established PDU Session(s) which is/are associated with the ongoing multicast MBS session(s). The AMF forwards the PDU session ID(s) of the associated PDU Session(s) towards the SMF(s) serving the associated PDU Session(s) by invoking the Nsmf_PDUSession_UpdateSMContext service operation. Based on the PDU session ID(s) of the associated PDU Session(s), the SMF performs step 7 to step 12 of clause 7.2.1.3 to establish the multicast MBS session(s) towards the target NG-RAN..

* * * End of Changes * * * *