3GPP TSG-SA WG2 Meeting #154AHE	S2-2301169r04

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0683 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for network data analytics in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures for network data analytics in roaming need to defined to support data and analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add general descriptions for network data analytics in roaming. - Add procedures for analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network data analytics in roaming case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.1.1.A (new), 6.1.1.B (new) , 6.1.2.X (new), 6.1.2.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 6.x	Network data analytics for roaming

Based on operator’s policy and local regulations (e.g. privacy), data or analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In this case, an NWDAF is used as entry point to exchange analytics and to collect input data for analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent, and filters the information exposed in reply to the request.

Exposure of input data for analytics is allowed from VPLMN to HPLMN and vice versa. It may be restricted based on operator policy and user consent.

The H-NWDAF may provide analytics to the V-NWDAF. The V-NWDAF may provide analytics to the H-NWDAF.

Editor's note: It is FFS whether an Entry NWDAF is to be defined as entry point to exchange analytics and to collect data in roaming scenario.

NOTE 1:	Analytics that rely on input data from the HPLMN are preferbly not provided from V-NWDAF to H-NWDAF, but generated by the H-NWDAF. Analytics that rely on input data from the VPLMN are preferbly not provided from H-NWDAF to V-NWDAF, but generated by the V-NWDAF.

Editor's note:	Data and analytics exchange between HPLMN and VPLMN may require updates based on feedback from GSMA.

Editor´s Note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

V-NWDAF may request or subscribe to HPLMN analytics generated by the H-NWDAF, as described in Clauses 6.1.2.X and 6.1.1.A, respectively, or request or subscribe to input data of the HPLMN from the H-NWDAF as described Claue 6.2.A and generate analytics on its own, and then the analytics can be leveraged by the 5GC NF in the VPLMN:

Editor's note: It is FFS whether an analytics profile is required in addition to analytics.

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for network slice selection and SMF selection for PDU Session management.

-	UE-related analytics (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

H-NWDAF may request or subscribe to VPLMN analytics generated by the V-NWDAF, as described in Clauses 6.1.2.Y and 6.1.1.B, respectively, or request or subscribe to input data of the VPLMN from the V-NWDAF as described Claue 6.2.B and generate analytics on its own, and then the analytics can be leveraged by the 5GC NF in the HPLMN:

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

The consumer 5GC NF may indicate parameters as defined in the analytics related clauses for non-roaming case, in the request or subscription of analytics information in a different PLMN (i.e. HPLMN or VPLMN) towards the NWDAF (which is in the same PLMN as the consumer 5GC NF).

Editor's note: It is FFS whether there is a need to restrict the parameters compared to the non-roaming case. It is ffs whether NSSAIs of the VPLMN and/or the HPLMN can be provided as analytics filters and how they are derived. It is ffs if additional analytics filter information to identify the requesting HPLMN or VPLMN is required, or this can be derived from the origin of related request. It is ffs whether an indication about inbound or outbound roaming scenario is required or this can be derived from service operation names.

2nd change

#### 6.1.1.A	Analytics subscription by H-NWDAF from V-NWDAF for outbound roaming users

This procedure may be used by the NWDAF in the H-PLMN as service consumer to subscribe/unsubscibe to notifications about analytics from the related V-PLMN for outbound roaming users (from the HPMN perspective)

Figure 6.1.1.A-1: Analytics subscription by H-NWDAF from V-NWDAF for outbound roaming users

1.	For subscription, the H-NWDAF checks the user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

2.	The H-NWDAF of HPLMN discovers V-NWDAF of VPLMN that supports the Nwdaf_InboundRoamerAnalytics service using the NRF.

NOTE:	The access to the Nwdaf_AnalyticsSubscription service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor´s note: Parameters and how to discover V-NWDAF from NRF is FFS.

3.	The H-NWDAF subsribes/unsubscribes to notifications about analytics information by invoking the Nnwdaf_RoamerAnalytics_Subscribe/ Nnwdaf_RoamerAnalytics_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_RoamerAnalytics Subscribe service operation are ffs.

4.	V-NWDAF checks if the HPLMN is authorised to request the analytics based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted analytics and related parameters).

5.	The V-NWDAF triggers new data collection if needed and monitores the requested analytics.

6.	The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices (that may depend on the HPLMN)

7.	The V-NWDAF notifies analytics information to the H-NWDAF.

3rd change

#### 6.1.1.B	Analytics subscription by V-NWDAF from H-NWDAF for inbound roaming users.

This procedure may be used by an NWDAF in the V-PLMN to subscribe to notifications about analytics from the related H-PLMN for inbound roaming users (from the VPLMN perspective)

Figure 6.1.1.B-1: Analytics subscription by V-NWDAF from H-NWDAF for inbound roaming users

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the Nwdaf_RoamingAnalytics service using the NRF.

NOTE:	The access to the Nwdaf_AnalyticsSubscription service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor´s note: Parameters and how to discover H-NWDAF from NRF is FFS.

2.	The V-NWDAF subscribes/unsubscribes to analytics information by invoking Nnwdaf_RoamingAnalytics_Subscribe/ Nnwdaf_RoamingAnalytics_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_RoamingAnalytics_Subscribe service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to subscribe to the indicated analytics based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted analytics and related parameters) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF triggers new data collection if needed and monitors the requested analytics.

5.	The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF notifies analytics information to the V-NWDAF.

4th change

#### 6.1.2.X	Analytics request by H-NWDAF from V-NWDAF for outbound roaming users

This procedure may be used by the NWDAF in the H-PLMN as service consumer to retrieve analytics from the related V-PLMN for outbound roaming users (from the HPMN perspective)

Figure 6.1.2.X-1: Analytics request by H-NWDAF from V-NWDAF for outbound roaming users

1.	The H-NWDAF checks the user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

2.	The H-NWDAF of HPLMN discovers V-NWDAF of VPLMN that supports the Nnwdaf_RoamingAnalytics service

NOTE:	The access to the Nwdaf_AnalyticsInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor´s note: Parameters and how to discover V-NWDAF from NRF is FFS.

3.	The H-NWDAF requests analytics information by invoking Nnwdaf_RoamingAnalytics_Request service operation.

Editor´s note: Parameters for the Nnwdaf_InboundRoamerAnalyticsInfo_Request service operation are ffs.

4.	V-NWDAF checks if the HPLMN is authorised to request the analytics based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted analytics and related parameters).

5.	The V-NWDAF triggers new data collection if needed and determines the requested analytics.

6.	The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices (that may depend on the HPLMN)

7.	The V-NWDAF responds with analytics information to the H-NWDAF.

5th change

#### 6.1.2.Y	Analytics request by V-NWDAF from H-NWDAF for inbound roaming users.

This procedure may be used by an NWDAF in the V-PLMN to retrieve analytics from the related H-PLMN for inbound roaming users (from the VPLMN perspective)

Figure 6.1.2.Y-1: Analytics request by V-NWDAF from H-NWDAF for inbound roaming users

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the Nwdaf_RoamingAnalytics service

NOTE:	The access to the Nwdaf_AnalyticsInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Editor´s note: Parameters and how to discover H-NWDAF from NRF is FFS.

2.	The V-NWDAF requests analytics information by invoking Nnwdaf_RoamingAnalytics_Request service operation.

Editor´s note: Parameters for the Nnwdaf_RoamingAnalytics_Request service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to request the analytics based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted analytics and related parameters) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF triggers new data collection if needed and determines the requested analytics.

5.	The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF responds with analytics information and/or analytics profile to the V-NWDAF.

* * * End of Changes * * * *