3GPP TSG-SA WG2 Meeting #154AHE	S2-2301169r04

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0683 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for network data analytics in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures for network data analytics in roaming need to defined to support data and analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add general descriptions for network data analytics in roaming. - Add procedures for analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network data analytics in roaming case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 6.x	Network data analytics for roaming

Based on operator’s policy and local regulations (e.g. privacy), data or analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In this case, an NWDAF is used as entry point to exchange analytics and to collect input data for analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent, and filters the information exposed in reply to the request.

The NWDAF provides the Nnwdaf_RoamingAnalytics and Nnwdaf_RoamingData services for that purpose.

NOTE:	The access to the Nwdaf_AnalyticsSubscription service, and the Nwdaf_AnalyticsInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

Exposure of input data for analytics is allowed from VPLMN to HPLMN and vice versa. It may be restricted based on operator policy and user consent.

The H-NWDAF may provide analytics to the V-NWDAF. The V-NWDAF may provide analytics to the H-NWDAF.

Editor's note: It is FFS whether an Entry NWDAF is to be defined as entry point to exchange analytics and to collect data in roaming scenario.

NOTE 1:	Analytics that rely on input data from the HPLMN are preferbly not provided from V-NWDAF to H-NWDAF, but generated by the H-NWDAF. Analytics that rely on input data from the VPLMN are preferbly not provided from H-NWDAF to V-NWDAF, but generated by the V-NWDAF.

Editor's note:	Data and analytics exchange between HPLMN and VPLMN may require updates based on feedback from GSMA.

Editor´s Note:	The user consent check procedure and security aspects will align with SA WG3's conclusion.

V-NWDAF may request or subscribe to HPLMN analytics generated by the H-NWDAF, , or request or subscribe to input data of the HPLMN from the H-NWDAF as described Claue 6.2.A and generate analytics on its own, and then the analytics can be leveraged by the 5GC NF in the VPLMN:

Editor's note: It is FFS whether an analytics profile is required in addition to analytics.

-	UE-related analytics (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

H-NWDAF may request or subscribe to VPLMN analytics generated by the V-NWDAF, or request or subscribe to input data of the VPLMN from the V-NWDAF as described Claue 6.2.B and generate analytics on its own, and then the analytics can be leveraged by the 5GC NF in the HPLMN:

The consumer 5GC NF may indicate parameters as defined in the analytics related clauses for non-roaming case, in the request or subscription of analytics information in a different PLMN (i.e. HPLMN or VPLMN) towards the NWDAF (which is in the same PLMN as the consumer 5GC NF).

Editor's note: It is FFS whether there is a need to restrict the parameters compared to the non-roaming case. It is ffs whether NSSAIs of the VPLMN and/or the HPLMN can be provided as analytics filters and how they are derived. It is ffs if additional analytics filter information to identify the requesting HPLMN or VPLMN is required, or this can be derived from the origin of related request. It is ffs whether an indication about inbound or outbound roaming scenario is required or this can be derived from service operation names.

Editor's note: Call flows for analytics subscription and analytics request in inbound and outbound roaming case need to be added. It is ffs whether .to add them as subclauses to clause 6.1.1 and 6.1.2 or within the present clause

* * * End of Changes * * * *