3GPP TSG-SA2 Meeting # 154AH	S2-2301221r02

# E-Meeting, Jan 16 - Jan 20, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4095 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN broadcast system information and manual network selection for localized service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_eNPN_Ph2 KI#4 conclusion in TR 23.700-08 v1.4, It was agreed that existing HRNN of SNPN can provide localized service information to assist manual selection of hosting network, without normative impact (i.e. without extending the SIB). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Note 3 of 5.30.2.2, additional text is added to clarify that HRNN may also contain localized service information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether HRNN can be used for manual selection of hosting network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.30.2.2	Broadcast system information

NG-RAN nodes or Trusted non-3GPP access networks which provide access to SNPNs broadcast the following information:

-	One or multiple PLMN IDs

-	List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to

NOTE 1:	It is assumed that an NG-RAN node supports broadcasting a total of twelve NIDs. Further details are defined in TS 38.331 [28].

NOTE°2:	The presence of a list of NIDs for a PLMN ID indicates that the related PLMN ID and NIDs identify SNPNs.

-	Optionally:

-	A human-readable network name per SNPN;

NOTE 3:	The human-readable network name per SNPN is only used for manual SNPN selection. If the SNPN supports localized service, the human-readable network name of the SNPN can be information associated to the localized service. The mechanism how human-readable network name is provided (i.e. whether it is broadcasted or unicasted) to the UE is specified in TS 38.331 [28].

-	Information, as described in TS 38.300 [27], TS 38.331 [28] and in TS 38.304 [50], to prevent Ues not supporting SNPNs from accessing the cell, e.g. if the cell only provides access to non-public networks;

-	An indication per SNPN of whether access using credentials from a Credentials Holder is supported;

-	List of supported Group IDs for Network Selection (GINs) per SNPN;

-	An indication per SNPN of whether the SNPN allows registration attempts from Ues that are not explicitly configured to select the SNPN, i.e. Ues that do not have any PLMN ID and NID nor GIN broadcast by the SNPN in the Credentials Holder controlled prioritized lists of preferred SNPNs/GINs.

NOTE 4:	Further details (including number of supported GINs per SNPN) are defined in TS 38.331 [28].

NEXT CHANGE

END OF CHANGE