SA WG2 #154-AH-e e-meeting	S2-2301234

# 16-20 January 2023, Elbonia

Source: 	Interdigital

Title: 	Update on conclusion of KI#5

Document for: 	Approval

Agenda Item: 	9.5.1

Work Item / Release:	FS_Ranging_SL/Rel-18

Abstract of the contribution: This contribution updates the conclusion of KI#5.

# Discussion

According to the conclusion, when target UE is out of coverage, if it can setup the connection via U2N Relay, LCS request may be handled. But, if there is no condition is specified when the target UE needs to setup U2N Relay, the service availability for SL positioning will be different per UE’s implementation and it will decrease overall quality of service.

Therefore, the following text is proposed to be added:

- When the target UE has pending LCS request e.g., per deferred MT-LR and if there is no direct connection available, the target UE shall try to discover U2N Relay and setup indirect connection for handling the pending LCS request, e.g., location report.

- 5GC shall be abled to configure whether the target UE needs to discover and connect U2N Relay for future SL positioning service request.

There are several E.N. remaining in conclusion of KI#5. And the following resolutions are proposed.

- The following E.N. is proposed to be deleted as it is already described that the protocol used between UE and LMF will be decided by RAN under KI#4.

Editor's note: RAN will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network assisted SL positioning including an MT-LR and MO-LR.

- The following E.N. needs further discussion. Therefore, it is proposed to change it to NOTE for further discussion during normative phase.

Editor's note: It is FFS what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF.

- As concluded in KI#3, Discovery for SL positioning UE is based on ProSe Discovery which is the operation of UE. Therefore, discovery of Located UE shall be performed by target UE. And as selection of SL positioning UE is relating to the determination of SL positioning method which is belonging to SLPP/RSPP operation between target UE and LMF. The following E.N. shall be clarified accordingly.

Editor's note: It is FFS how and whether the 5GC can discover and select a Located UE to participate in the network assisted SL positioning.

- LMF is responsible for calculation of SL positioning measurement results. When SL positioning measurement data needs to be collected and reported to the LMF, it is efficient to minimize the connections between UE and LMF. Therefore, it is proposed that target UE collect the measurement data and report the collated data to the LMF.

Editor's note: It is FFS whether and how the Located UE reports the ranging/SL positioning measurement result to the LMF. Is this done together with the absolute positioning result of the Located UE from e.g. the GMLC.

- When target UE does not have a NAS connection, there are many issues to resolve for handling of new MT-LR received from AF. Therefore, it is proposed not to pursue to support MT-LR, deferred MT-LR when a Target UE does not have a NAS connection in this Release.

- But, when the target UE has pending MT-LR request to report or target UE is originating MO-LR request for LCS client or report to the AF, it can be supported with small update to enable the location report from target UE via trusted located UE to the 5GC as UE only operation is possible for SL positioning measurement and calculation of result. Therefore, it is proposed to support location report from target UE via located UE for pending MT-LR request e.g., deferred MT-LR, or for MO-LR with a NOTE saying that supporting location report via the located UE to the LMF will be confirmed by RAN.

Editor's note: It is FFS whether and how to support MT-LR, Deferred MT-LR and MO-LR when a Target UE that does not have a NAS connection. Particularly, LPP impact when the Located UE forwards the Target UEs location or measurement reports to the LMF to locate the Target UE location needs RAN feedback.

# 2 Proposal

The following changes are proposed for TR 23.700-86 V1.2.0

FIRST CHANGE

## 8.5	Key Issue #5: Network assisted SL Positioning

The Normative work will be based on the following principles:

-	Legacy functionality specified in TS 23.273 [11] for location services shall be reused including MO-LR, MT-LR for both regulatory and commercial use, when the Target UE has NAS connection with AMF (directly or indirectly via a ProSe UE-to-Network Relay).

-	The target UE reports its support of Network assisted SL Positioning during the registration.

-	When the target UE has pending LCS request e.g., per deferred MT-LR and if there is no direct connection available, the target UE shall try to discover U2N Relay and setup indirect connection for handling the pending LCS request, e.g., location report.

-	5GC shall be able to configure whether the target UE needs to discover and connect U2N Relay for future SL positioning service request.

-	The Location Service is triggered via the AMF that serves the Target UE, the request comes either from a 5G NF, or an application server via the GMLC, or the Target UE.

-	The Target UE can initiate MO-LR or the LMF can initiate MT-LR based on legacy procedures.

-	Positioning protocol LPP endpoints are the Target UE and the LMF.

NOTE 1:	RAN WGs will determine whether and what enhancements and the subset functionalities of LPP are needed to support Network assisted SL positioning including an MT-LR and MO-LR.

-	Either the Target UE or LMF determines if network assisted SL positioning will be applied. When LMF determines that network assisted SL positioning is used, LMF may trigger the Target UE to perform the discovery of Located UE(s).

-	The Target UE and Located UE(s) perform ranging/SL positioning, as defined in KI#4.

-	The Target UE reports the identity of the Located UE(s) to the LMF.

NOTE 2:	It will be determined during Normative phase what UE identity of the Located UE that the Target UE provides to the LMF. Depending on which UE identity, the LMF or the GMLC may need to resolve this by query another NF. The security impact will be coordinated with SA3.

-	Target UE discover Located UE(s) for network assisted SL positioning and LMF may provide candidate list of Located UE to participate in the network assisted SL positioning for the target UE. The provisioning of candidate list of Located UE assumes that the capability and location information of the Located UE in the list are maintained by 5GC.

-	When candidate list of Located UE is provided by LMF, target UE needs to discover and select the located UEs in the list with priority.

-	Selection of Located UE for network assisted SL positioning is done for determination of SL positioning method between LMF and target UE and it belongs to SLPP/RSPP, extension of LPP, or new protocol to be determined by RAN.

-	The LMF uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result reported by Target UE and optionally also by Located UEs to estimate the location of the Target UE. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same time (per TS 23.273 [11]) to improve the Target UE positioning accuracy.

-	Target UE collects the ranging/SL positioning measurement data and reports the collated data to the LMF.

NOTE 3:	The LMF may need to compensate the time difference of the positioning of Target UE and Located UE with additional information.

NOTE 4:	The Ranging/Sidelink positioning supports the scheduled location time feature.

-	The LMF provides the location estimate to the AMF which forwards it either to the Target UE over NAS,  to the GMLC, or to the 5GC NF according to legacy functionality.

-	If Target UE is not in network coverage and does not have NAS connection with AMF, for MO-LR, the Target UE requests for the Location of the Located UE:

-	If the Located UE(s) needs network assistance to retrieve its location, the Located UE(s) may trigger a MO-LR. This assumes that the Located UE(s) has a NAS connection to an AMF.

-	The Target UE uses the location returned by Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location.

NOTE 4:	Security aspects require confirmation from SA WG3.

-	When Target UE does not have NAS connection, for MO-LR or pending MT-LR e.g., deferred MT-LR, Target UE may report the ranging/SL positioning measurement result to the LMF via Located UE.

NOTE 5:	Supporting of location report via the Located UE to the LMF will be confirmed by RAN.