3GPP TSG-WG SA2 Meeting #154AH E e-meeting 	S2-2301279

Elbonia, January 16 – 20, 2022	(revision of S2-220xxxx)

Source:	Intel

Title:	TR 23.700-86: KI#3 Conclusion update

Document for:	Approval

Agenda Item:	9.5.1

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract:  This paper proposes the conclusion update for KI#3.

# 1. Introduction/Discussion

5G ProSe Direct Discovery procedures defined in TS 23.304 include 5G ProSe Direct Discovery with 5G DDNMF, Group Member Discovery and 5G ProSe UE-to-Network Relay Discovery. All of them support Model A and Model B with different parameters. In current conclusion, it’s not clear which procedure will be used for Ranging/Sidelink Positioning device discovery. On the other hand, ProSe identifier is not explicitly used in 5G ProSe Direct Discovery procedures.

For Restricted ProSe Discovery, there is not clear requirement and clear solutions yet. For example, to support restricted ProSe Discovery, core network need to interacts with the ProSe Application server for the authorization of the discovery request. However, it’s not decided whether there will be a ProSe Application server for Ranging/Sidelink Positioning device discovery. So we propose not to conclude support on Restricted ProSe Discovery at this stage.

V2X communication defined in TS 23.304 includes unicast, groupcast and broadcast mode. In unicast mode, Service Type is included during Layer-2 link establishment procedure, which can be enhanced for Ranging/Sidelink Positioning device discovery. For broadcast mode or groupcast mode, Service Type is involved during Policy/Parameter provisioning procedure for configuration, there is no need to introduce any enhancement for broadcast mode or groupcast mode V2X communication procedure.

According to above, this contribution proposes to update the conclusion of KI#3.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-86.

# * * * * First change * * * *

## 8.3	Key Issue #3: Ranging/Sidelink Positioning device discovery

For Key Issue #3 "Ranging/Sidelink Positioning device discovery", the followings are taken as conclusions:

-	For Ranging/Sidelink Positioning device discovery, Model A and Model B Direct Discovery as defined in TS 23.304 [4] are reused for the 5G ProSe capable UE (including commercial and public safety use cases); and procedures for V2X communication over PC5 reference point as defined in TS 23.287 [3] are reused for the V2X capable UEs, with the following enhancements:

-	When 5G ProSe Direct Discovery procedures as defined in 6.3.1 of TS 23.304 [4] are used, the ProSe identifier  indicates "Ranging/Sidelink Positioning";

-	When Group Member Discovery procedures as defined in 6.3.2.2 of TS 23.304 [4] are used, the Application Layer Group ID indicates an Ranging/Sidelink Positioning group that the UE belongs to;

-	When unicast mode V2X communication procedure is used, the Service Type in Layer-2 link establishment procedure indicates "Ranging/Sidelink Positioning";

-	When broadcast mode or groupcast mode V2X communication procedure is used, there is no procedure enhancements. The Service Type is to be set to "Ranging/Sidelink Positioning";

-	The expected roles of the UEs (e.g. Target UE, SL Reference UE) can be reflected in discovery procedure.

NOTE 1:	The parameters used in Ranging/Sidelink Positioning device discovery procedures can be decided in normative phase.

NOTE 2:	The Ranging/Sidelink Positioning device discovery procedures and the parameters used in the procedures should be aligned with RAN WGs.

NOTE 3:	Whether and how to support Restricted ProSe Discovery defined in TS 23.304 [4] for the 5G ProSe capable UE will be decided in normative phase.

# * * * * End of changes * * * *