3GPP TSG-SA2 Meeting #154AH E e-meeting	S2-2301282r05

# Elbonia, January 16 – 20, 2022										  	 	      (revision of S2-230XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4103 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBSR authorization update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBSR uses the IAB architecture and operates as an IAB node as specified in subclasue 5.35A.1. Therefore, it is reasonable to use same mechanism for IAB-indication and mobile IAB-indication, i.e. MBSR indicates its support on MBSR in AN signalling and NG-RAN provided this indication to core network. However, considering that the MBSR is authorized based on location and time periods, existing IAB-Operation allowed is not sufficient for MBSR authorization. Therefore, a dedicated MBSR- Operation allowed indication is introduced for MBSR authorization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing corresponding Editor’s notes and clarify MBSR-Operation allowed Indication is used for MBSR authorization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G systems procedure does not support MBSR authorization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

### 5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

Editor's note:	The subscription information for the MBSR will be further specified.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the 5GC can make use of it for authorization of MBSR in VPLMN. MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

Editor's note:	Configuration mechanism will be further described based on development of clause 5.35A.2.1.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR-Operation allowed indication to the UE and to NG-RAN. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the AMF can make use of subscription data for authorization of MBSR in VPLMN. The MBSR IAB-DU can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

Editor's note:	 The detailed support of inter-IAB-donor gNB mobility is FFS and needs coordination with RAN WGs.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

The AMF of the MBSR can indicate to the MBSR  that it is not allowed to act as an IAB node as part of registration procedure, and in this case does not include MBSR authorization indication to donor-gNB.

Editor's note:	Which NAS message(s) (registration accept, registration reject or/and deregistration request) is used by the AMF of the MBSR for indicating the MBSR is not allowed to act as MBSR is FFS. Whether the NAS layer includes indication of the MBSR (IAB-UE) supports specific feature is FFS.

NOTE 2:	The mechanism applies to both roaming and non-roaming MBSR operations.

***** END OF CHANGES *****