3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301297r02

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4108 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuraiton of the MBSR node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clasue 5.35A.2, the details on the mechanism to configure the MBSR was left unspecified. The plan was to update the contents based on the conlusion of TR 23.700-05 clause 8.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions on configuration of the MBSR based on TR 23.700-05 conclusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of the MBSR node is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.35A.2		Configuration of the MBSR

In order for a MBSR to operate as a relay, it receives configuration from the OAM system of serving PLMN as specified in TS 38.401 [42].

NOTE: SA3 and SA5 are responsible for the secure access to OAM, including in roaming case.

In addition, the MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation.

The MBSR can be pre-configured with UE policy or provisioned uisng existing UE Policy mechanism as defined in TS 23.503 [45]. The UE Policy for the MBSR contains the following:

-	the PDU session parameters to use for the OAM server access, for each of the PLMNs it is authorized to operate;

-	the Allowed PLMN list and Forbiddent PLMN list for PLMN selection of MBSR.

# * * * * End of changes * * * *