3GPP TSG-SA WG2 Meeting #154AH-E (e-meeting)	S2-2301306

# January 16 – 20, 2023, Elbonia												 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0078 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architectural enhancements for support of Detect And Avoid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-58 conclusion for DAA include the following: - a mechanism for DAA leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution #5 […] - Additionally, may support network-assisted (ground based) DAA solution #7. It is applicable for a specific area, such as a stadium or arena where drones are used. This CR addresses the related changes to TS 23.256 specifically for DAA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR introduces section 5.X describing the support of DAA based on PC5 mechanisms.The only difference from the TR is that PC5-U is assumed for broadcast and unicast application layer exchange, to re-use V2X PC5 mechanisms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR conclusions are not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 5.X Detect and Avoid Mechanisms

### 5.X.1	Detect and Avoid based on PC5

Procedures and mechanisms in TS 23.287 [9] apply to A2X with the following differences:

-	the content of the messages for DAA are defined according to the regional regulations for DAA and is out of scope of this specification.

The detection and resolution of collisions between UAVs is locally performed between UAVs using direct UAV to UAV communication over PC5. The USS may be informed of the collision situation.

NOTE 1:	The UAV can inform the USS of the collision situation over application layer communication that is out of scope of this specification.

The UAVs are assumed to be provisioned with the A2X Policy which includes a DAA deconflicting policy indicating unicast or broadcast communication for deconflicting.

Figure 5.X.1-1: DAA procedure based on PC5.

1.	UAV1 receives broadcast messages from UAV2 in PC5-U messages, that may include application layer DAA payload, e.g. CAA-level UAV ID, velocity, heading direction, position.

2.	UAV1 passes the DAA payload to the upper layer. The application layer detects a conflict, based on the broadcast messages received from UAV2, e.g. by comparing it with its own trajectory and location. If the application layer in the UAV1 detects a collision, it initiates a collision avoidance/conflict resolution procedure with UAV2.

3.	Optionally, UAV1 may inform its own USS about the detected collision by including peer UAV 2s' ID(s).

4.	UAV1 selects a communication mode (broadcast or unicast) for DAA deconfliction based on the input received from the application layer and A2X Policy. If unicast deconfliction mode is selected, then UAV1 triggers the establishment of a unicast link with UAV2 based on procedures in TS 23.287 [9] clause 6.3.3. Otherwise, messages in steps 5 and 6 are exchanged using broadcast mode as defined in TS 23.287 [9] clause 6.3.1.

5.	UAV1 sends to UAV2 a DAA deconfliction message, e.g. deconfliction request message which may include collision detection alert, its CAA-level UAV IDs and the one(s) from other detected conflicting UAV(s), and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

NOTE 2:	The deconflicting specific parameters are application layer content and is out of scope of this specification.

6.	UAV2 replies to provide agreed DAA deconflicting policy, its updated trajectory and other info, e.g. message deconfliction status response, conflict resolved alert, CAA-level UAV IDs of participating UAVs from the receiving UAV.

Subsequent messages can be exchanged between UAVs until traffic conflict resolution is reached (e.g. for mutual position/trajectory monitoring) based on application layer mechanisms.

* * * * End of Changes * * * *