3GPP TSG-SA WG2 Meeting #154AH-E (e-meeting)	S2-2301306

# January 16 – 20, 2023, Elbonia												 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0078 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architectural enhancements for support of Detect And Avoid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-58 conclusion for DAA include the following: - a mechanism for DAA leveraging U2X (UAV-to-everything), with the architectural enhancements identified in solution #5 […] - Additionally, may support network-assisted (ground based) DAA solution #7. It is applicable for a specific area, such as a stadium or arena where drones are used. This CR addresses the related changes to TS 23.256 specifically for DAA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR introduces section 5.X describing the support of DAA based on PC5 mechanisms.The only difference from the TR is that PC5-U is assumed for broadcast and unicast application layer exchange, to re-use V2X PC5 mechanisms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR conclusions are not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[X]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

* * * * Start of Next Change * * * *

5.X Detect and Avoid Mechanisms

5.X.1	Detect and Avoid based on PC5

Procedures and mechanisms in TS 23.287 [X] apply to A2X with the following differences:

-	the content of the messages for DAA are defined according to the regional regulations for DAA and is out of scope of this specification.

The detection and resolution of collisions between UAVs is locally performed between UAVs using direct UAV to UAV communication over PC5. The USS may be informed of the collision situation.

NOTE 1:	The UAV can inform the USS of the collision situation over application layer communication that is out of scope of this specification.

The UAVs are assumed to be provisioned with the A2X Policy which includes a DAA deconflicting policy indicating unicast or broadcast communication for deconflicting.

Figure 5.X.1-1: DAA procedure based on PC5.

1.	UAV1 receives broadcast messages from UAV2 in PC5-U messages, that may include application layer DAA payload, e.g. CAA-level UAV ID, velocity, heading direction, position.

2.	UAV1 passes the DAA payload to the upper layer. The application layer detects a conflict, based on the broadcast messages received from UAV2, e.g. by comparing it with its own trajectory and location. If the application layer in the UAV1 detects a collision, it initiates a collision avoidance/conflict resolution procedure with UAV2.

3.	Optionally, UAV1 may inform its own USS about the detected collision by including peer UAV2 ID(s).

4.	UAV1 selects a communication mode (broadcast or unicast) for DAA deconfliction based on the input received from the application layer and A2X Policy. If unicast deconfliction mode is selected, then UAV1 triggers the establishment of a unicast link with UAV2 based on procedures in TS 23.287 [X] clause 6.3.3. Otherwise, messages in steps 5 and 6 are exchanged using broadcast mode as defined in TS 23.287 [X] clause 6.3.1.

5.	UAV1 sends to UAV2 a DAA deconfliction message, e.g. deconfliction request message which may include collision detection alert, its CAA-level UAV IDs and the one(s) from other detected conflicting UAV(s), and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

NOTE 2:	The deconflicting specific parameters are application layer content and is out of scope of this specification.

6.	UAV2 replies to provide agreed DAA deconflicting policy, its updated trajectory and other info, e.g. message deconfliction status response, conflict resolved alert, CAA-level UAV IDs of participating UAVs from the receiving UAV.

NOTE 3:	It is assumed that all UAVs in the area can be involved for DAA, anyhow, the procedure shows only two UAVs for simplicity.

Subsequent messages can be exchanged between UAVs until traffic conflict resolution is reached (e.g. for mutual position/trajectory monitoring) based on application layer mechanisms.

* * * * End of Changes * * * *