3GPP TSG-SA2 Meeting #154AH-e	S2-2301312r06

# emeeting, January 16 - 20, 2023											 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0079 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ground based DAA for an area (Sol#7) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inclusion of solution #7 from TR 23.700.58 into TS 23.256 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of soln#7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution #7 will not be covered in TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, new clause (with subclauses) 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 23.501 [2].

Area Airspace Manager (AAM): A ground-based entity that is responsible for managing the airspace for a specific area/arena so that correspondingly residing UAVs are prevented to collide with each other and with other physical objects. For this purpose, AAM is able to detect UAVs residing in the specific airspace and provide  policies for collision avoidance. to the corresponding UASes. The AAM acts as a TPAE having specific abilities for direct communication with UAVs relevant for the applicable airspace and it includes one or more UEs enabled for use of PC5.

3GPP UAV ID: Identifier assigned by the 3GPP system and used by external AF (e.g. USS) to identify the UAV. GPSI is used as the 3GPP UAV ID.

Broadcast Remote ID: The capability of providing Remote Identification and Tracking over broadcast radio links.

NOTE 1:	In the scope of this release, the radio link for Broadcast Remote ID is assumed to utilize radio technologies outside the scope of 3GPP.

CAA (Civil Aviation Administration)-Level UAV Identity: a UAV identity assigned by USS/UTM, and uniquely identifies a UAV at least within the scope of a USS.

Command and Control (C2) Communication: the user plane link to deliver messages with information of command and control for UAV operation from a UAV controller or a UTM to a UAV or to report telemetry data from a UAV to its UAV controller or a UTM.

C2 Aviation Payload: Contains application layer information sent by the UAS to the USS containing UAV pairing information and/or flight authorization information that is transparent to the 3GPP System.

C2 Authorization Payload: Contains application layer information sent by the USS to the UAV containing e.g. C2 pairing information and/or C2 security information that is transparent to the 3GPP System.

C2 Pairing Information: Contains UAV-C Addressing Information which may e.g. include the UAV-C IP Address.

Networked UAV Controller: a UAV Controller connected to the 3GPP network and connected to the UAV via a 3GPP network.

Non-Networked UAV Controller: a UAV Controller not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP, e.g. internet connectivity or direct wireless communication over a technology outside the scope of 3GPP.

Networked Remote ID: The capability of providing Remote Identification and Tracking to a USS over 3GPP network.

Remote Identification (Remote ID) of UAS: The ability of a UAS in flight to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly. The Remote ID information payload may include Serial Number or Session ID assigned to the UAV, location of the ground-station controller, emergency status indication, etc.

Third Party Authorized Entity: is either a privileged Networked UAV Controller, or a privileged Non-Networked UAV Controller, or another entity which gets information on sets of UAV controllers and UAVs from the 3GPP network, and may be connected to the UAV via the Internet; it may be authorized by the UTM to interface with sets of UAV(s).

UAS NF: a 3GPP UAS Network Function for support of aerial functionality related to UAV identification, authentication/authorization and tracking, and to support Remote Identification.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g. UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAS Traffic Management (UTM): a system that can safely and efficiently integrate the flying UAV along with other airspace users. It provides a set of functions and services for managing a range of autonomous vehicle operations (e.g. authenticating UAV, authorizing UAS services, managing UAS policies, and controlling UAV traffics in the airspace).

UAV controller: The UAV controller of a UAS enables a drone pilot to control an UAV.

UAV operator: the entity owning and operating a UAV.

UAS Container: A container to the 3GPP system that includes UUAA Aviation/Authorization Payload and/or C2 Aviation/Authorization Payload. The internal content of the individual payloads is transparent to the 3GPP system.

UAS Services: refers to establishment of connectivity for a UAS for communication with USS, for C2, for remote identification, and for UAV location and tracking.

USS communication: A communication between a UAV and a USS other than C2 communication, by means of user plane data transmission for some UAS Services.

NOTE 2:	The PDU session/PDN connection for C2 communication and the PDU session/PDN connection for USS communication can be common or separate.

UUAA Authorization Payload: Contains application layer information optionally including UUAA result for UAV consumption provided by the USS to the UAS which is transparent to the 3GPP System.

UUAA Aviation Payload: Contains application layer information provided by the UAS to USS and is transparent to the 3GPP System

Uncrewed Aerial System (UAS): Composed of Uncrewed Aerial Vehicle (UAV) and related functionality, including command and control (C2) links between the UAV and the control station, the UAV and the network, and for remote identification. An UAS may comprise of a UAV and a UAV controller.

Unknown UAVs: A list of the UAVs to be identified in the target area and served by the PLMN as the result of the UAV tracking requested by USS/UTM.

UUAA: UAV USS authentication and authorization procedure of the UAV to ensure that the UAV has successfully registered with a USS and has therefore been authorized for operations by the USS. An UAV is authenticated and authorized by USS via a UUAA procedure with the support of the 3GPP system before connectivity for UAS services is enabled.

UUAA-MM: the UUAA procedure optionally performed during registration to a 5GS.

UUAA-SM: the UUAA procedure performed during the establishment of a PDU session and performed during the establishment of a PDN connection.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAM	Area Airspace Manager

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

C2	Command and Control

NRID	Networked Remote Identification

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

USS	UAS Service Supplier

UTM	Uncrewed Aerial System Traffic Management

UUAA	USS UAV Authorization/Authentication

UUID	Universal Unique Identifier

* * * Next Change  All New* * *

5.x	Ground-based DAA for an Area

Editor’s Note: This clause is work in progress, based on updates to other KI solutions, needs to be further aligned.

5.x.1	Functional Description

This clause provides a network-assisted (ground based) DAA mechanism. It is applicable for a specific area, such as a stadium or arena where UAVs are used e.g. for filming an event. It is based on a ground-based entity, Area Airspace Manager (AAM), that is able to detect UAVs in the specific areaand provide local steering policies to the individual UASes in order to avoid the UAVs crashing into each other or different structures etc. The policy may e.g. include allowed flying zones and positions allowed for the specific UAV. The policy may also apply to a specific outdoor area, e.g. an event, where specific measures to avoid collision between drones are established locally.

NOTE 1:	The policies provided by the AAM is application specific steering/DAA policies that are out of scope of 3GPP. The same or different local policies can be provided to the UAVs in the arena. UAVs can, for example, be given different fly zones in the arena just like people have different seating in an arena.

The high-level principles of the network-assisted (ground based) DAA are:

-	The arena/area has a ground-based entity Area Airspace Manager (AAM). The AAM includes one or more UEs enabled for use of PC5. The AAM may also have a direct connection to the Data Network.

-	For the applicable airspace of the area/arena the AAM defines individually adapted local collision avoidance rules for correspondingly located UAVs.

NOTE 2:	How the AAM can determine the local steering policy for collision avoidance is out of scope of 3GPP. It may e.g. be based on maps of the area/arena  and detailed information about the facilities together with an awareness of the current usage need for the airspace.

-		Provisioning of AAM local collision avoidance rules to a UAV/UE must comply with the policies for PC5 operations received from the 5GC or being preconfigured in the UE.

-	Detected UAVs are identified by their coordinates and by the Remote ID as retrieved by Broadcast Remote ID (BRID) or Network Remote ID (NRID) mechanisms dependent on the method used by the UAV.

Editor’s Note: How the USS determines which AAM to provide the NRID and of which UAVs is FFS.

NOTE 3:	How the AAM can scan the UAVs in specific area is out of scope of 3GPP.

NOTE 4:	How the AAM retrieves the Remote ID for detected UAVs is out of scope of 3GPP. For example, in case of BRID different UAVs can use different  methods, such as WLAN, Blutooth or PC5 direct communication, for broadcating the Remote ID and in case of NRID the AAM will retrieve the Remote ID from the USS at application layer, if the AAM is enabled to do so.

Editor’s Note: How the USS determines which AAM to provide the NRID and of which UAVs is FFS.

-	Based on the retrieved Remote ID, the AAM activates PC5 communication with each detected UAV

by triggering establishment of a unicast link based on procedures in 23.287 [X] clause 6.3.3 using the UAV's Remote-ID as Application-Layer-ID (i.e. Target User Info) in the Direct Communication Request message.

For Direct Communication over PC5 the UAV and AAM shall comply with the authorization and provisioning principles described in clause 4.2.1.2.2 including the following considerations:

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communication over PC5 reference point.

-	The default destination Layer-2 ID to be used for initial signalling to establish a unicast connection for the A2X  service.

-	Parameters for Groupcast are not applicable.

-	NR PC5 is supported

-	The AAM uses PC5 unicast link to provide each UAV present in the arena/area with local DAA policies as user traffic.

-	A UAV that receives local collision avoidance related policies from the AAM over PC5 may  forward the policies to its UAV-C so that the UAV-C can steer the UAV accordingly by use of C2 communication (e.g. over Uu, PC5, or other means) in order to enforce the local policies and avoid collisions.

NOTE 5:	Whether and how to forward the policies from UAV to its UAV-C and how UAV-C uses the policies is out of scope of 3GPP.

NOTE 6:	A UAV-C that receives local collision avoidance policies from an AAM, can inform the AAM about its network address (e.g. IP address or URL) in order to enable direct AAM to UAV-C communication via Data Network. How a UAV-C communicates with the AAM is out of scope of 3GPP.

Figure 5.x.1-1: Logical architecture for ground based DAA

5.x.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the procedure.

* * * End of Changes * * *