SA WG2 Meeting #154AHe	S2-2301314r01

# January 16th – 20th, 2023; Elbonia               	  	       		 (revision of S2-230xxxx)

Title:	[draft] Reply LS on Security architecture for 5G multicast/broadcast services

Response to:	S2-2300071/ S3-223919

Release:	Rel-17

Work Item:	5MBS

Source:	Nokia, Will be SA2

To:	SA3

Cc:	CT3, CT4, SA4

Contact Person:

#### Name:	Thomas Belling

E-mail Address:	 Thomas (dot) Belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA2 thanks SA3 for their LS, where SA3 informs that SA2 about an agreed CR introducing a new Multicast/Broadcast Service Security Function (MBSSF). According to the CR, "the MBSSF is a logical function that includes the security functionalities defined in TS 33.246. MBSSF can be collocated with either MBSF or MBSTF and the according interfaces are up to the implementation of the deployment options."

SA2 would like to comment that the MBSSF, if it is defined as a new NF, is not reflected in the stage 2 architecture and corresponding procedures in TS 23.247, and the fact that Rel-17 is frozen prevents substantial architectural changes. The MBSSF needs to be inserted into the user plane path for MBS data distribution. While procedures for the insertion of the MBSF/MBSTF into the user plane path via the provisioning of appropriate address information are defined clause 7.1.1.2 of TS 23.247, related procedures are lacking for the MBSSF.

Anyway, SA2 recommends that SA3 clarifies that the MBSSF needs to be (not only "can be") collocated with MBSF or MBSTF.

SA2 would also like to comment that the work item code included in the CR cover sheet (i.e. TEI 17) is not correct as there is ongoing Work Item 5MBS in Rel-17.

2. Actions:

To SA3:

ACTION: 	SA2 kindly asks SA3 to take the above information into account and provide further clarification.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#155 | February 20-24th, 2023 | Athens | Greece |
| SA2#156 | April 17-21th, 2023 | E-meeting | Any |