3GPP TSG-SA2 Meeting #154AH	S2-2301320

# Elbonia, January 16 –20, 2023																(Revision of S2-xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0215 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of out-of-coverage operation for U2U Relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the study TR 23.700-33, it is possible to operate UE-to-UE relay whereby the end UEs as well as the UE-to-UE relay itself is out-of-coverage. This needs to be clarified in the technical specification TS 23.304. Clause 5.14 currenly only describes Layer-3 UE-to-UE relay communication. Also Layer-2 UE-to-UE relay operation needs to be described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify out-of-coverage operation for 5G ProSe UE-to-UE Relay in clause 5.14. Added clause 5.14.2 to describe Layer-2 UE-to-UE relay communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may not be clear that UE-to-UE relay can operate also in out-of-coverage to implementers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.14	5G ProSe UE-to-UE Relay Communication

### 5.14.1	5G ProSe Layer-3 UE-to-UE Relay Communication

The 5G ProSe Layer-3 UE-to-UE Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic.

The 5G ProSe Layer-3 UE-to-UE Relay provides the functionality to support connectivity between 5G ProSe Layer-3 End UEs. A 5G ProSe Layer-3 UE-to-UE Relay and 5G ProSe Layer-3 End UEs can be located within NG-RAN coverage or outside of NG-RAN coverage.

The type of traffic supported over PC5 reference point is indicated by the 5G ProSe Layer-3 UE-to-UE Relay e.g. using the corresponding RSC.

### 5.14.2	5G ProSe Layer-2 UE-to-UE Relay Communication

The 5G ProSe Layer-2 UE-to-UE Relay provides forwarding functionality that can relay any type of traffic over the PC5 link.

The 5G ProSe Layer-2 UE-to-UE Relay provides the functionality to support connectivity between 5G ProSe Layer-2 End UEs. A 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-2 End UEs can be located within NG-RAN coverage or outside of NG-RAN coverage.

* * *End of Changes * * *