3GPP TSG-WG SA2 Meeting #154 	S2-2210812

Toulouse, France, Nov 14 – 18, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0590 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eNA_Ph3 DCCF relocation in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The process of DCCF relocation needs to be reflect in TS23.288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Clause 6.2.6.3.1 - Introduce general information about DCCF relocation 2.Clause 6.2.6.3.X - Introduce DCCF relocation initiated by DCCF 3.Clause 6.2.6.3.Y - Introduce DCCF relocation initiated by consumer |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCCF relocation is not updated to reflect conclusions in TR23.700-81 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.1, 6.2.6.3.X (new), 6.2.6.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.2.6.3.1	General

This clause specifies procedures for data collection using the DCCF described in clause 5A for cases other than obtaining analytics from an NWDAF (which is specified in clause 6.1.2). Two options are supported: data delivered via the DCCF, according to clauses 6.2.6.3.2 and 6.2.6.3.3 and data delivered via a messaging framework according to clauses 6.2.6.3.4 and 6.2.6.3.5. Which option to be used is determined by DCCF configuration.

In scenarios where multiple DCCF instances are deployed in the same 5G network, there are some cases that the serving DCCF may need to be changed during the operation due to, e.g. UE mobility, change in DCCF profile, new addition or removal of DCCF instances, etc. Two options are supported: DCCF relocation initiated by DCCF according to clauses 6.2.6.3.X and DCCF relocation initiated by consumer according to clauses 6.2.6.3.Y. Which option to be used is determined by operator.

* * * Second Change (all new text) * * *

6.2.6.3.X	DCCF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.X-1 is used by a data consumer (e.g. NWDAFor central DCCF) to obtain UE(s) data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation and when  DCCF cannot serve the UE anymore, DCCF will execute the relocation.

Figure 6.2.6.3.X-1: Procedure for DCCF relocation initiated by DCCF

0.	The data consumer subscribes source DCCF. The data consumer sends UE(s) ID, enabling DCCF to determine whether the UE(s) data subscription transfer procedure is applicable.

1.	Source DCCF subscribes UE(s) mobility events from AMF.

2.	If UE(s) moves out of the service area of the source DCCF, source DCCF determines UE(s) data subscription to be transferred to target DCCF(s), e.g. triggered by a UE mobility event notification from AMF.

3.	Source DCCF may query the NRF to perform a DCCF discovers and selects the target DCCF(s), e.g. based on the UE location information received from AMF.

4.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation, a transfer of UE(s) data subscription to the target DCCF(s).

5.	Target DCCF(s) accepts the data subscription(s) transfer.

6.	Target DCCF informs the data consumer about the successful UE(s) data subscription transfer using a Ndccf_DataManagement_Notify message.

7.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription and repeat step 1 to subscribes UE(s) mobility events from AMF.

8.	Target DCCF confirms UE(s) data subscription transfer to the source DCCF.

9.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE(s) data subscriptions. And target DCCF may subscribe to relevant data source(s), if not yet subscribed.

NOTE:	At this point, UE(s) data subscription transfer is deemed completed.

10-11.	Target DCCF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify message.

* * * Third Change (all new text) * * *

6.2.6.3.Y	DCCF relocation initiated by consumer

The procedure depicted in Figure 6.2.6.3.Y-1 is used by a data consumer (e.g. NWDAFor central DCCF) to obtain UE(s) data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation and when DCCF cannot serve the UE anymore, data consumer will reselect DCCF.

Figure 6.2.6.3.Y-1: Procedure for DCCF relocation initiated by consumer

1.	Source DCCF notifies the service consumer that, e.g. location of UE falls outside the serving area of the DCCF, so it cannot serve the UE anymore. A cause code is also added with the notification (e.g. UE moved outside DCCF serving area, if data source is added in subscription - UE moved outside data source serving area).

2.	Service consumer for the source DCCF determines to select a new instance of DCCF and discovers and selects the target DCCF.

3.	The service consumer sends a subscription request to the target DCCF using Ndccf_DataManagement_Subscribe request.

4.	[OPTIONAL] The service consumer unsubscribes from the source DCCF since it will be served by the target DCCF from now on.

5.	[OPTIONAL] Target DCCF may subscribe to relevant data source(s), if not yet subscribed.

* * *End of Changes * * *