SA WG2 Meeting #S2-154	S2-2210870

14 - 18 November, 2022, Toulouse, France

Title:	[DRAFT] LS on PDU Set Importance and in-order delivery over Uu

Response to:	-

Release:	Release 18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	SA2

To:	RAN2, RAN3

Cc:	-

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to bring to the attention of RAN2 and RAN3 that SA2 has reached the following agreements:

-	From SA2 perspective the PDU Set Importance is used primarily for PDU Set-level packet discarding in presence of congestion. How the PDU Set Importance translates on Uu (e.g. in terms of the number of DRBs for the same QoS Flow) is up to RAN WGs to decide.

-	PDU Sets with different PDU Set Importance belonging to the same media stream are bound by the UPF on the same QoS Flow.

-	PDU Set Importance, together with other information listed in TR 23.700-60 clause 8.4.2.3, is conveyed from UPF to NG-RAN via the GTP-U header of user plane packets.

-	Regarding the need for in-order delivery of PDU Sets with different importance belonging to the same media stream over the radio, SA2 concluded that the application client in the receiver can cope with re-ordering at the upper layers (e.g. using RTP sequence numbers).

2. Actions:

To RAN2 and RAN3 groups.

ACTION: 	SA2 respectfully asks RAN2 and RAN3 to take into account the feedback above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#154-AH-E			16 - 20 January 2023			Electronic