3GPP TSG-WG SA2 Meeting #154 	S2-2210677

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Lenovo

Title:	KI#1: Conclusions update

Document for:	Approval

Agenda Item:	9.14.1

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: this paper proposes to update the conclusion of KI#1 and to resolve the Editor's Notes.

# 1. Introduction/Discussion

This paper attempts to resolve the Editor's Notes in clause 8.1 "Conclusions for KI#1".

### 1.1 Regarding conclusion 0)

With regard to the conclusion 0) on the trigger to replace a currently used S-NSSAI with an alternative S-NSSAI, it is still not clear which entity and which procedure is used to trigger the replacement of the S-NSSAI in case of roaming case with Home route PDU Session. In such case, it is possible that the HPLMN wishes to temporary replace the HPLMN S-NSSAI. Therefore, it is proposed to keep the EN regarding bullet 0).

Proposal 1: The EN regarding bullet 0) is kept in as a place-holder that there is an open issue. Alternatively, the EN can be formulated as NOTE and the issue can be solved during the normative phase.

### 1.2 Regarding conclusion 4-a)

With regard to conclusion 4-a) on the UE configuration for the replacement of the old S-NSSAI an alternative S-NSSAI deployed on a different NSIs, it is proposed to agree on the following points:

- Regarding the use of IE to send the alternative S-NSSAI replacing the old S-NSSAI, it is proposed to further discuss the resolution during the normative phase. The issue is that further discussions are needed how to signal the replacement of the HPLMN S-NSSAI by an alternative S-NSSAI, and also how to signal the mapping of the VPLMN S-NSSAI to an HPLMN S-NSSAI. It may be required that two different mapping information are sent to the UE.

- In roaming case with Home routed traffic, it is required that the AMF, optionally with V-NSSF, determines the following three elements:

  - VPLMN S-NSSAI;

  - the mapping of the VPLMN S-NSSAI to the alternative HPLMN S-NSSAI; and

  - the mapping/replacement of the alternative HPLMN S-NSSAI to the HPLMN S-NSSAI.

The AMF should be able to send all three elements to the UE.

- When the UE receives the updated Allowed NSSAI and/or in the Configured NSSAI including the alternative S-NSSAI and if the UE has an active PDU Session to the old S-NSSAI, the UE does not immediately trigger a release of the PDU Session to the old S-NSSAI.

Proposal 2: It is proposed to document all bullet points A, B and C from above in the conclusion 4-a).

### 1.3 Regarding conclusion 4-b)

Regarding conclusion 4-b), the question is which of both mechanisms is more efficient:

- When the AMF receives the request for the new PDU Session establishment, the AMF updates the UE configuration according 4-a), and the AMF rejects the UE's request for PDU Session establishment with an indication to use an alternative S-NSSAI.

- When the AMF receives the request for the new PDU Session establishment, the AMF exchanges the old S-NSSAI with the alternative S-NSSAI, selects an SMF using the alternative S-NSSAI and forwards the PDU Session establishment request to the selected SMF. The SMF proceeds with the establishes of the PDU Session on the alternative S-NSSAI and the SMF sends PDU Session establishment accept to the UE with the alternative S-NSSAI. Afterwards the AMF performs UCU procedure to update the Allowed NSSAI to remove the old S-NSSAI and to include the alternative S-NSSAI.

The drawback of mechanism B is that during the PDU Session establishment, the SMF sends N2 SM information to the RAN to request the UP establishment on the alternative S-NSSAI although the Allowed NSSAI in the UE context does not include the alternative S-NSSAI. The RAN would establish the corresponding DRB(s) on the resources of the alternative S-NSSAI which is not yet allowed for the UE. It is expected that this would require RAN impact. Therefore it is proposed to move forward with the mechanism A.

Proposal 3: It is proposed to conclude mechanism A for the conclusion 4-b).

# 2. Text Proposal

Based on the analysis above, it is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change * * * *

## 8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM system towards a NF like AMF/NSSF) or based on notification from AM-PCF (e.g. the AM-PCF obtains analytics from NWDAF in roaming case with Home routed PDU Sessions).

Editor's note:	It is FFS how the AM-PCF is used to trigger the S-NSSAI replacement on per slice-level signalling and what SBI service is used for the PCF notification. Further, trigger events in case of roaming with Home routed PDU Session are FFS.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the AM-PCF and/or with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1.

2)	In case of a same S-NSSAI associated with multiple NSIs and the change of the NSI is required, when an existing PDU session is decided as to be migrated to another NSI, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. When a PDU session establishment request from the UE arrives at the AMF, the AMF performs NSI re-selection i.e. removes from the UE context the old NSI and the AMF selects another NSI, e.g. as described in solution #15.

3)	In cases where the old S-NSSAI and the alternative S-NSSAI are associated with the same NSI, and the IP address is retained at same UPF and SMF does not change, SA5 mechanism may be sufficient to repartition resources between the old and alternative S-NSSAIs and the PDU session associated with the old S-NSSAI does not need to be transferred on the alternative S-NSSAI.

4)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet, and to remove the old S-NSSAI from the Allowed NSSAI. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case. The replacement/mapping information is provided to the UE independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

-	The IE to be used to send to the UE the mapping of the alternative S-NSSAI to old S-NSSAI (e.g. whether to re-use the existing Mapping Of Allowed NSSAI or specify a new IE e.g. "replaced NSSAI") will be determined during the normative phase.

-	In Home routed roaming case, if the HPLMN S-NSSAI has to be replaced by alternative HPLMN S-NSSAI (e.g. AMF is notified by the AM-PCF), the AMF, optionally together with NSSF, determines: 1) the VPLMN S-NSSAI, 2) the mapping of the VPLMN S-NSSAI to the alternative HPLMN S-NSSAI and 3) the mapping/replacement of the alternative HPLMN S-NSSAI to the HPLMN S-NSSAI. The AMF sends both mapping information to the UE.

-	When the UE receives the updated Allowed NSSAI and/or in the Configured NSSAI including the alternative S-NSSAI and if the UE has an active PDU Session to the old S-NSSAI, the UE does not immediately trigger a release of the PDU Session to the old S-NSSAI.

-	The UE continues to use the existing URSP rules to establish new PDU Sessions, i.e. the new PDU Session would be associated to the HPLMN S-NSSAI, which in non-roaming case is the old S-NSSAI.

-

b)	For a new PDU Session establishment request from the UE on the old S-NSSAI (e.g. applicable when the UE is not yet updated as per 4a):

-	When the AMF receives the request for the new PDU Session establishment, the AMF updates the UE configuration according 4a). Afterwards the AMF rejects the UE's request for PDU Session establishment with an indication to use an alternative S-NSSAI. The UE triggers new PDU Session establishment using the alternative S-NSSAI according the MM configuration as per 4a).

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	The AMF first performs the UCU procedure according to bullet 4a). Then the AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and optionally indicates the alternative S-NSSAI.

-	In Home routed roaming case, if the HPLMN S-NSSAI has to be replaced by the alternative HPLMN S-NSSAI, the AMF notifies the H-SMF via the V-SMF that the PDU Session is to be relocated to the alternative HPLMN S-NSSAI.

-	The SMF sends to the UE either PDU Session Modification Command (if the PDU Session is of SSC mode 3) or PDU Session Release/Modification Command (if the PDU Session is of SSC mode 1 or 2) and indicates to the UE that the PDU Session can be re-established on another S-NSSAI. If AMF indicated to the SMF the alternative S-NSSAI, the SMF provides the alternative S-NSSAI to the UE.

-	the UE triggers a new PDU Session Establishment procedure to establish a PDU Session on the alternative S-NSSAI by evaluating the URSP rules, using the new MM configuration from 4a) and the SM signalling from 4c).

NOTE X:	For PDU Session of SSC mode 1, the IP address preservation is not guaranteed and the session continuity can be preserved based on the application layer capability to deal with IP address change.

-	If the alternative S-NSSAI for the HPLMN is part of alternative RSD of the same URSP rule, the PDU Session establishment request includes only the alternative S-NSSAI or the alternative S-NSSAI and the mapping to VPLMN S-NSSAI; otherwise.

-	The PDU Session establishment request includes the alternative S-NSSAI together with the old S-NSSAI (similar as to use the mapped S-NSSAI value of the VPLMN when applying the RSD matched to the HPLMN S-NSSAI).

# * * * * End of changes * * * *