# 3GPP SA WG2 Meeting #154	S2-2210684r05

# Toulouse, France, November 14 – 18, 2022	(Revision of S2-220xxxx)

# Source:	ZTE, LG Electronics

# Title:	New WID: Enhancement of Network Slicing Phase 3

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Stage 2 of Network Slicing Phase 3

Acronym: eNS_Ph3

Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x | x |  |
| No |  |  |  |  |  |
| Don't know | x |  |  |  | x |

2	Classification of the Work Item and linked work items

2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_eNS_Ph3 | SA2 | 940063 | Study on Network Slicing Phase 3 |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 910032 | Enhanced Access to and Support of Network Slice | Stage 1 requirements |

3	Justification

This work item aims at specifying system enhancements on network slicing as concluded within TR 23.700-41.

4	Objective

The objective is to specify the following enhancements to 5GS as per conclusions reached within TR 23.700-41 for the following aspects:

For key issue #1

-	Support the AMF determining alternative S-NSSAI and triggering the S-NSSAI replacement by interaction with OAM / AM PCF / NSSF.

-	Allow the AMF in early binding to perform NSI re-selection when the UE establish a new PDU session.

-	Enhance the system to provide alternative S-NSSAI to the supporting UE via Registration Procedure or UE Configuration Update Procedure.

-	Support S-NSSAI replacement for a new PDU Session by supporting the SMF to provide alternative S-NSSAI to UE upon receiving alternative S-NSSAI from AMF.

-	Support S-NSSAI replacement for an existing PDU Session without PDU session re-establishment by supporting the SMF to provide alternative S-NSSAI to UE, RAN, and UPF upon receiving alternative S-NSSAI from AMF.

-	Support S-NSSAI replacement for an existing PDU Session using PDU session re-establishment by supporting the SMF to provide alternative S-NSSAI to UE upon receiving alternative S-NSSAI from AMF.

For key issue #2

-	Support of slice based SoR mechanism by referencing to TS 23.122.

For key issue #3

-	To support network slices which have Area of Service (see NG.116) that is not matching existing TAs: Enhance the system to provide network slice validity policy to the supporting UE which includes location information, via Registration Procedure or UE Configuration Update Procedure.

-	Clarify that RRM policies for S-NSSAIs can be configured per cell basis as defined in TS 28.541 while the CN support the slice per TA level.

-	To support temporary network slices: Enhance the system to provide network slice validity policy to the supporting UE which includes timing information, via Registration Procedure or UE Configuration Update Procedure.

-	AMF enforcement of the S-NSSAI availability policies e.g. when the UE does not support the policies based on AMF subscribing to AoI.

-	To support of graceful/gradual termination of PDU sessions during network slice decommissioning for UEs supporting timing information and non-supporting UEs.

For key issue #4

-	Enhance the Number of UEs per network slice availability check and update procedure to convey the Service Area and PLMN ID to NSACF

-	Define a Primary NSACF to handle overall NSAC for an S-NSSAI at the global level, and enhance the NSACF to support interaction with Primary NSACF for UE admission quota-based control, or UE admission threshold based control.

-	Define a Centralized architecture with a single centralized NSAC node to handle a PLMN with multiple service areas.

-	Support of NSAC (HPLMN control, VPLMN control and HPLMN delegated control) for roaming UE

For key issue #5

-	Enhance the system to provide information related to the supported or not supported TA(s) regarding some S-NSSAIs which are partially rejected in the RA to the UE via Registration Procedure or UE Configuration Update Procedure.

-	Enhance the system to provide information related to the supported or not supported TA(s) regarding some S-NSSAIs which are partially/conditionally allowed in the RA to the UE via Registration procedure or UE Configuration Update procedure.

-	Enhance the system to provide partially/conditionally allowed NSSAI to the NG-RAN.

For key issue #6

-	Define the slice specific policy related to S-NSSAIs subject to network control and the network control the S-NSSAI deregistration and PDU session release

-	Enhance the system to configure the supporting UE and enable the UE to handle policies for network slice registration and deregistration based on configuration information inform the PCF or local AMF configuration.

-	Enhance the system to enable the network to control the PDU session release and network slice deregistration.

-	Support transfer of all PDU sessions from one S-NSSAI to a replacement S-NSSAI leveraging the procedures of key issue #1.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A | N/A | N/A | N/A | N/A | N/A |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Architectural enhancements to support the stated objectives |  |  |
| 23.502 | Procedure enhancements to support the stated objectives |  |  |
| 23.503 | Procedure enhancements to support the stated objectives |  |  |

6	Work item Rapporteur(s)

ZHU Jinguo, ZTE, zhu.jinguo@zte.com.cn, Primary Rapporteur (KI#1, KI#3, KI#4)

Myungjune Youn, LG Electronics, m.youn@lge.com, Secondary Rapporteur (KI#2, KI#5, KI#6)

7	Work item leadership

SA2

8	Aspects that involve other WGs

CT1 for specify SoR enhancements

SA5 for management and charging aspects. Cell based support requires OAM configuration of AMFs and requires SA5 alignment to provide this information to AMF. Coordinate with SA5 with regards to interaction for decommissioning of a network slice, including a gradual PDU session release.

SA3 for security aspects

9	Supporting Individual Members

Supporting IM name

Alibaba ?

Apple ?

AT&T

CATT ?

China Mobile ?

China Telecom ?

China Unicom ?

Convida Wireless LLC ?

Deutsche Telekom AG

DISH Network ?

Ericsson

Huawei ?

Intel ?

InterDigital ?

KDDI

KPN ?

KT Corp. ?

Lenovo ?

LG Electronics

LG Uplus ?

MATRIXX Software

MITRE ?

Motorola Mobility ?

NEC ?

Nokia ?

Nokia Shanghai Bell ?

NTT Docomo ?

OPPO ?

Oracle ?

Orange ?

Qualcomm ?

Samsung

Sanechips ?

Sharp ?

SK Telecom ?

T-Mobile USA ?

Spreadtrum ?

Tencent ?

TELUS ?

Verizon UK Ltd ?

Xiaomi ?

ZTE