3GPP TSG-SA WG2 Meeting #154	S2-2211141

# November 14 – 18, 2022, Toulouse, France							      (revision of S2-2210396)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0132 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer-3 UE-to-UE Relay Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Layer-3 UE-to-UE Relay Communication has been studied and concluded to be supported in Rel-18 per TR 23.700-33 clasue 8.1, so this feature needs to be specified in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions and procedures for Layer-3 UE-to-UE Relay Communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Layer-3 UE-to-UE Relay Communication not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 5.X.1 (new), 6.X (new), 6.X.1 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

5.X	5G ProSe UE-to-UE Relay Communication

5.X.1	5G ProSe Layer-3 UE-to-UE Relay Communication

The 5G ProSe Layer-3 UE-to-UE Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic.

The type of traffic supported over PC5 reference point is indicated by the 5G ProSe Layer-3 UE-to-UE Relay e.g. using the corresponding RSC.

* * * * 2nd Change * * * *

6.X	5G ProSe UE-to-UE Relay Communication

6.X.1	5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.X.1-1 shows the procedure for 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay.

Figure 6.X.1-1: 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

1.	Service authorization and provisioning are performed for Source 5G ProSe Layer-3 End UE, Target 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.2.

2.	The Source 5G ProSe Layer-3 End UE performs discovery of a 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.3.2.X.

3.	The Source 5G ProSe Layer-3 End UE sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.X.

The Source Layer-2 ID of the DCR message is self-assigned by the Source 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of the 5G ProSe Layer-3 UE-to-UE Relay.

4.	If the User Info ID of Target 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay and RSC are included in the Direct Communication Request, the 5G ProSe Layer-3 UE-to-UE Relay responds by establishing the security with the Source 5G ProSe Layer-3 End UE. When the security protection is enabled, the Source 5G ProSe Layer-3 End UE sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the 5G ProSe Layer-3 UE-to-UE Relay.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the Source 5G ProSe Layer-3 End UE obtains the 5G ProSe Layer-3 UE-to-UE Relay's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	After the Security Establishment procedure in step 4 is completed, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the Target 5G ProSe Layer-3 End UE. The parameters included in the Direct Communication Request message are described in clause 6.4.3.X.

The Source Layer-2 ID of the DCR message is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is the unicast Layer-2 ID of Target 5G ProSe Layer-3 End UE associated with the User Info ID of Target 5G ProSe Layer-3 End UE.

6.	If the User Info ID of Target 5G ProSe Layer-3 End UE and RSC are included in the Direct Communication Request, the Target 5G ProSe Layer-3 End UE responds by establishing the security with the 5G ProSe Layer-3 UE-to-UE Relay. When the security protection is enabled, the 5G ProSe Layer-3 UE-to-UE Relay sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the Target 5G ProSe Layer-3 End UE.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the Target 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the 5G ProSe Layer-3 UE-to-UE Relay obtains the Target 5G ProSe Layer-3 End UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

7.	The Target 5G ProSe Layer-3 End UE sends a Direct Communication Accept message to the 5G ProSe Layer-3 UE-to-UE Relay that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.X.

8.  For IP traffic, IPv6 prefix or IPv4 address is allocated for the Target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.X.

9.	After receiving the Direct Communication Accept message from the Target 5G ProSe Layer-3 End UE, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Accept message to the Source 5G ProSe Layer-3 End UE that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.X.

10. For IP traffic, IPv6 prefix or IPv4 address is allocated for the Source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.X.

11.	For IP communication, the 5G ProSe Layer-3 UE-to-UE Relay may store an association of User Info ID and the IP address of Target 5G ProSe Layer-3 End UE into its DNS entries, and the 5G ProSe Layer-3 UE-to-UE Relay may act as a DNS server to other UEs. The Source 5G ProSe Layer-3 End UE may send a DNS query to the 5G ProSe Layer-3 UE-to-UE Relay to request IP address of Target 5G ProSe Layer-3 End UE after step 10 if the IP address of Target 5G ProSe Layer-3 End UE in not received in step 9, and the 5G ProSe Layer-3 UE-to-UE Relay returns the IP address of the Target 5G ProSe Layer-3 End UE to the 5G ProSe Layer-3 End Source UE.

12.	The Source 5G ProSe Layer-3 End UE communicates with the Target 5G ProSe Layer-3 End UE via the 5G ProSe Layer UE-to-UE Relay.

In the case of one Source 5G ProSe Layer-3 End UE communicates with multiple Target 5G ProSe Layer-3 End UEs, the PC5 link between the Source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay can be shared for multiple Target 5G ProSe Layer-3 End UEs per RSC while the PC5 links may be established individually between the 5G ProSe Layer-3 UE-to-UE Relay and Target 5G ProSe Layer-3 End UEs per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

In the case of multiple Source 5G ProSe Layer-3 End UEs communicate with one 5G ProSe Layer-3 End Target UE, the PC5 link between the 5G ProSe Layer-3 UE-to-UE Relay and the Target 5G ProSe Layer-3 End UE can be shared per RSC while the PC5 links may be established individually between the Source 5G ProSe Layer-3 End UEs and the 5G ProSe Layer-3 UE-to-UE Relay per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

* * * * End of Changes * * * *