3GPP TSG-SA2 Meeting #154	S2-2211276

# Toulouse, France, Nov 14 - 18, 2022                                         revision of S2-2210368

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0125 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Communication via U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusion of U2U relay has been agreed in the TR 23.700-33. Both the communication of peer End UEs after Model A/B discovery or without Model A/B discovery (i.e. Discovery integrated into PC5 unicast link establishment procedure) are supported. And both Layer-2 UE-to-UE Relay and Layer-3 UE-to-UE Relay are supported. This CR adds the 5G ProSe L2 UE-to-UE Relay Communication procedure based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the 5G ProSe L2 UE-to-UE Relay Communication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose communication for L2 U2U relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 6.4.3.x is introduced in CR 0124. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st  Change * * * *

6.x	5G ProSe UE-to-UE Relay Communication

6.x.y	5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

This procedure applies to 5G ProSe Layer-2 UE-to-UE Relay.

Figure 6.x.y-1: 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

Service authorization and provisioning has been performed for the 5G ProSe Layer-2 UE-to-UE Relay and the 5G ProSe End UEs as described in clause 6.2 before this procedure.

1.	Model A or Model B 5G ProSe UE-to-UE Relay Discovery as described in clause 6.3.2.x is performed and a source 5G ProSe End UE selects a suitable 5G ProSe Layer-2 UE-to-UE Relay for the communication with a target 5G ProSe End UE.

2.	The source 5G ProSe End UE decides whether to use an existing PC5 link with the 5G ProSe UE-to-UE Relay for the required service. If an existing PC5 link is used then the Layer-2 link modification procedure as specified in clause 6.4.3.x is used towards a 5G ProSe UE-to-UE Relay, otherwise a Layer-2 link establishment procedure is used towards a 5G ProSe UE-to-UE Relay.

This procedure is towards the selected 5G ProSe UE-to-UE Relay, and for Layer-2 link establishment, the security establishment is performed before step 3 is initiated.

3.	The 5G ProSe Layer-2 UE-to-UE Relay decides whether to use an existing PC5 link between the 5G ProSe UE-to-UE Relay and the target 5G ProSe End UE for the required service, and initiates Layer-2 link establishment procedure or Layer-2 link modification procedure as specified in clause 6.4.3.x with the target 5G ProSe End UE.

This procedure is performed towards the target 5G ProSe End UE using the unicast Layer-2 ID.

The 5G ProSe Layer-2 UE-to-UE Relay sends a Direct Communication Accept message or Link Modification Accept message to the the source 5G ProSe End UE after step 3 is completed.

4.	The source 5G ProSe End UE establishes an end-to-end connection for unicast mode communication with the target 5G ProSe End UE as described in clause 6.4.3.x.

The data is transferred between the source 5G ProSe End UE and the target 5G ProSe End UE via the 5G ProSe Layer-2 UE-to-UE Relay. The 5G ProSe Layer-2 UE-to-UE Relay forwards all the data messages between the source 5G ProSe End UE and the target 5G ProSe End UE, as specified in TS 38.300 [12].

* * * * End of Changes * * * *