3GPP TSG-WG SA2 Meeting #154	S2-2211316

Toulouse, Nov 14 - Nov 18, 2022	rev. S2-2210917, 11030

Source:	Google Inc., Samsung

Title:	KI #4: Update Conclusion on clause 8.4.X

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: This paper proposes to resolve some open issues and editor’s note, and update conclusion on clause 8.4.x.

# Discussion

This paper proposes to update the conclusions in Key Issue#4 for the following aspects:

- For the localized services information, the UE can connect to localized services portal website via established PDU session and choose available localized services that can be accessed via the current hosting network. In this case, the UE does not require localized services information beforehand to match desired localized services provided by the hosting network in the lists.

Proposal#1: it is proposed to update the conclusion in clause 8.4.2, clause 8.4.3, and clause 8.4.5.

- For clause 8.4.5: Conclusion for what credentials are used to access hosting network and how to obtain them. In bullet 2: If new credentials for accessing hosting network are needed, the UE may use the existing credential onboarding mechanism as per Rel-17.

Proposal#5: it is proposed to add the following information as additional provisioning information:

- 	Localized services portal URL (which may be the same or different for different hosting networks).

## 3.	Conclusion and Proposal

It is proposed to progress the study for key issue#4 based on the proposals described in clause 2, and agree to update conclusion as proposed.

# Proposal

Add the following conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

## 8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

Editor's note:	The following are interim conclusions for KI#4, and to be taken as basis for further work i.e. also what has been agreed as to be included as a conclusion is candidate for being changed.

### 8.4.1	General

The conclusion for KI #4 is made for each component that is evaluated in clause 7.4.

When UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If Home network is SNPN, the Hosting network can be only SNPN

If the UE accesses the Hosting network using the other credentials rather than the subscription/credentials from the UE Home network, the determination of the subscription used to access the Hosting network is by implementation specific prior to automatic network selection as described in NOTE 1 of clause 5.30.2.4.2 of TS 23.501 [3].

### 8.4.2	Conclusion for the content of the localized service information

The following interim conclusions are reached. Final conclusions for normative work are expected for SA2#154 i.e. whether to proceed with Alt.1-3.

Alt.1.	The localized service information is provided to UE for UE to discover and select hosting network to receive desired localized service. The content of the localized service information can include the following elements:

a.	Identifier/name of the localized service.

b.	Validity conditions for the localized service, e.g. duration of remaining service operation ,time and location.

Editor's note 2-1 a:	It is FFS whether a PVS address / captive portal information need to be provided as an optional information to the UE. In the Localized service information. If not How will UE be provided this address is FFS.

d.	Hosting network related information, per hosting network in the case of SNPN as hosting network:

i.	hosting network identifier, e.g. SNPN ID, GIN;

ii.	Validity condition, e.g. time and/or location (optional)

Editor's note 2-1b:	It is FFS whether Validity condition is to be possible to be set per localized service, hosting network or both.

e.	Hosting network related information, per hosting network in the case of PNI-NPN as hosting network:

i.	PLMN ID of the PNI-NPN.

ii.	Optionally, if hosting network is associated with CAG IDs, a list of CAG IDs corresponding to the localized service.

Editor's note 2-2:	It is FFS whether it is beneficial to provide CAG information within the localized service information, given that there is existing procedure to update UE with CAG information, and whether CAG ID is used to identify hosting network. It is also FFS whether CAG IDs needs to be associated with localized service when sent to the UE.

Editor's note 2-3:	It is FFS whether more hosting network related information can be sent as to better enable a user selection of hosting network, e.g. balance/service rate information to access the hosting network for the localized service, quality of the service.

Alt.2.	The UE is provided with:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services, in the case of SNPN as hosting network.

b.	allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID.

NOTE:	The home network / hosting network map localized services to hosting network ID (e.g. SNPN ID, GIN, CAG ID).

Alt.3.	After the UE selects a hosting network via automatic hosting network selection procedure and then registers to the selected hosting network, the UE can know about localized services provided by the hosting network via localized service portal URL, which can be obtained from ePCO IE during PDU Session Establishment procedure. The user can then select desired localized services from the localized service portal based on its interests. If the localized service portal URL is provided, it takes precedence over the configured Localized services portal URL at the UE.

NOTE:	It is UE implementation for the UE to establish PDU Session after registration to the hosting network.

Editor's note 2-4:	It is FFS whether the UE receives list a/list b per localized service ID, or the lists are used for localized services in general.

Editor's note 2-5:	It is FFS whether necessary to associate validity conditions with the list in a and b above, and what are the conditions, e.g. time and/or location.

Editor's note 2-6:	It is FFS what localized service information needs to be provided to UE in b above, in the case PNI-NPN as hosting network does not support CAG.

Editor's note 2-7:	Alt. 1 provides UE with necessary information as a first step, and then the UE based on the received information derives how to perform other activities in later steps, e.g. network selection, credential determination and provisioning, etc. Alt.2 provides UE with information directly used for network selection, but may requires that UE/network knows beforehand whether the desired localized service is provided by the hosting networks in the lists.

*** 2nd CHANGE ***

### 8.4.3	Conclusion for from where and how UE obtains the localized service information

The information for localized service and hosting network discovery, selection and access can also be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope.

The following interim conclusions are reached. Final conclusions for normative work are expected for SA2#154 i.e. whether to proceed with Alt.1 and/or Alt.2.

Alt.1 (complementary to Alt.1 in clause 8.4.2):

1.	The localized service information is formulated as application data that can be stored in the home network, or hosting network.

2.	The external parameter provisioning procedure in clause 4.15.6 of TS 23.502 [4] is extended to support the provisioning of localized service information to home, or hosting network, or the OAM method is used to provision the data to home network.

3.	The localized service information can be preconfigured in the UE or dynamically provisioned via signalling. For dynamic provisioning, the AMF could provide the UE requests the desired application data (e.g. by specifying the identifier or name of the localized service) from either home network, hosting network or serving network e.g. via new UE policy.

NOTE:	How UE obtains localized service identifier or name to make the request for dynamic provisioning is out of 3GPP scope.

Editor's note 3-1:	It is FFS the benefit of UE requesting desired localized service information.

4.	If PNI-NPN as hosting network is associated with CAG ID, separated from other localized service information, the UE obtains the Allowed CAG ID list from the home network according to clause 5.30.3.3 TS 23.501 [3].

a.	The home network may send the updated Allowed CAG ID list to the UE

Editor's note 3-2b:	It is FFS how UE can associate the Allowed CAG IDs to the localized service if sent without any association to the localized service.

Editor's note 3-2c:	The trigger for the home network to send the updated Allowed CAG ID list to the UE is FFS.

Alt.2 (complementary to Alt.2 in clause 8.4.2):

5.	The information for localized service and hosting network discovery, selection and access can be preconfigured in the UE or dynamically provisioned by the hosting network or home network (via the VPLMN when roaming).

6.	In the case of SNPN as hosting network, the dynamic provisioning of prioritized list of hosting network information can be done via SoR.

NOTE:	How SOR-AF and/or UDM acquires hosting network information is outside the scope for 3GPP.

i.	The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply:

-	UE location as part of Registration procedure.

-	UE subscription data change, e.g. via external parameter provisioning.

Editor's note:	It is FFS whether home network, may broadcast in a SIB an indication that localised service(s) are available in the geographic locality of the cell that is broadcasting.

Editor's note:	Whether SIB can include localized service information for hosting network selection is FFS and it shall be evaluated by SA3 on if it is feasible from security aspect.

NOTE:	Given the potential large content of SIB information for manual selection, on demand SIB solution as already defined in TS 38.331 [14] can be used. Details of this option are to be determined by RAN WG2.

Editor's note:	It is FFS whether, in the case of SNPN as hosting network, the hosting network may broadcast the supported localized service information (e.g. service identifiers and/or human readable service information) to help UEs discover the service and the hosting network. The localized service identifier may be pre-configured in the UE as per clause 8.4.2. The human readable service information is used for manual hosting network selection. UE may acquire this information from hosting network also via on-demand SIB while UE is RRC_IDLE or RRC_INACTIVE state in serving network.

7. When UE has accessed with the hosting network, the following options can be supported:

- 	the AMF can provide the latest localized service information to UE as part of Registration procedure (Mobility Update) or UE Configuration Update procedure.

- 	the SMF can provide the localized services portal URL with latest localized services information in ePCO IE to UE during PDU Session Establishment/Modification procedure.

8.	In the case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3].

*** END CHANGES ***