3GPP TSG-SA2 Meeting #155	S2-2302364

Feb 20 – 24, 2023, Athens, Greece			                                      (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4130 | Rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on N3IWF selection for onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | S3i230106 was approved with the following LI requirements for UE accessing to SNPN for onboarding using untrusted non-3GPP access network: SA3-LI have concluded that the LI requirements for UE access to SNPN for UE Onboarding are identical to the requirements for UE access to SNPN for service, a.k.a. “general access”. SA3-LI expect the procedure described in TS 23.501, clause 6.3.6.2a, for “general access” would apply to the UE Onboarding case, as well. Following the mechanism defined in clause 6.3.6.2a, the UE is pre-configured with ON-SNPN selection information including SNPN ID and corresponding N3IWF FQDN used for Onboarding. The N3IWF selection for onboarding is performed as per clause 6.3.6.2a with the following differences: - The procedure applies when the UE is accessing the ON-SNPN N3IWF directly via untrusted non-3GPP access. - The configured N3IWF is replaced by the pre-configured N3IWF FQDN used for Onboarding. - The subscribed SNPN is replaced by the ON-SNPN. - If the UE determines that it is located in a country (called the visited country) different from the country where the pre-configured N3IWF FQDN used for Onboarding is located, then the UE shall construct an FQDN consisting of the SNPN ID of the ON-SNPN selection information instead of the subscribed SNPN. With the above proposal, the UE does not need to be preconfigured with N3IWF of every country. When one country does not mandate N3IWF selection in the same country, the UE can select the preconfigured N3IWF for onboarding. Moreover, the UE logic is the same for SNPN N3IWF selection, while using different input (SNPN ID and preconfigured N3IWF FQDN). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the N3IWF selection for onboarding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications not completed and does not follow SA3-Li guidance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.12	Access to SNPN services via Untrusted non-3GPP access

Access to SNPN services via Untrusted non-3GPP access network follows the specification in the previous 5.30.2 clauses with the differences as specified in this clause.

N3IWF selection is supported as follows:

-	When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a.

Emergency services are supported as follows:

-	UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following:

-	If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with valid SNPN credentials, in the same way as defined in clause 6.3.6.4.2 for a UE not equipped with a UICC.

UE onboarding is supported as follows:

-	When UE registers to SNPN over Untrusted non-3GPP access for UE Onboarding, the UE is pre-configured with ON-SNPN selection information including SNPN ID and corresponding N3IWF FQDN used for Onboarding. The N3IWF selection for onboarding is performed as per clause 6.3.6.2a with the following differences:

-	The procedure applies when the UE is accessing the ON-SNPN N3IWF directly via untrusted non-3GPP access.

-	The configured N3IWF is replaced by the pre-configured N3IWF FQDN used for Onboarding.

-	The subscribed SNPN is replaced by the ON-SNPN.

-	If the UE determines that it is located in a country (called the visited country) different from the country where the pre-configured N3IWF FQDN used for Onboarding is located, then	the UE shall construct an FQDN consisting of the SNPN ID of the ON-SNPN selection information instead of the subscribed SNPN.

-	If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without being connected to an N3WIF) with a PVS to obtain the SNPN credentials.

END CHANGE