SA WG2 Meeting #155	S2-2302510

# 20-24 February 2023, Athens, Greece	(was S2- S2-2301371)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3899 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN identifiers |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [vivo, Interdigital], Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-02-20 |  |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: |  | Following conclusion was capatured in clause 8.7 of TR 23.700-88: 2) PIN ID is uniquely identifiable within the 5G network, NOTE 2: It will be determined during the normative phase whether the PIN ID is defined as an external PIN identifier in similar way of the External Group Identifier and GPSI, and an 5G internal PIN identifier similar as Internal Group Identifier. It has been agreed in SA2 #154AHE to use External PIN ID and Internal PIN ID. But it is still not clear why the definition of Internal PIN ID is required, and what’s the difference between the newly defined Internal PIN ID and the existing Internal Group ID as specified in clause 5.29.2 of TS 23.501. The impact to UE, PCF, SMF, UPF in handling the Internal PIN ID is not clear. It has been concluded that PIN Service servie subscription is not required for PEMC. Whether PEMC can be configured with an Internal PIN ID is not clear. The above issues and the usage of Internal PIN ID is not specified in any existing solutions in TR 23.700-88. More discussion is required to clarify the above issues on the usage of Internal PIN ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the description on Internal PIN ID and add an EN for further study. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear the necessarity of Internal PIN ID and how to handle the Internal PIN ID in UE and 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.X.d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 5.X	Support for Personal IoT Networks

### 5.X.d	Identifiers

A PIN pre-configured or created by an AF is identified by an External PIN ID, which is unique in the PLMN.

Editor’s note:	Whether an internal PIN ID is needed and whether existing information can be reused to represent an internal PIN ID is FFS. In case a new Internal PIN ID is concluded to be needed, how NFs (PCF, SMF, UPF, UDM etc.) handle the proposed Internal PIN ID is FFS.

# >>>>END OF CHANGES<<<<