3GPP TSG-SA WG2#155	S2-2302569

20 - 24 Feb 2023, Athens, Greece

Source:	CATT

Title:	Discussion on Differentiation of Layer2 ID and Coexistence of U2N/U2U

Document for:	Discussion

Agenda Item:	9.7.2

Work Item / Release:	5G_ProSe_Ph2 / Rel-18

Abstract of the contribution: This contribution discusses the questions raised by RAN2 in the incoming LS (S2-2302187/R2-2213328), and gives the proposed answers.

- Dicussion

In the incoming LS (S2-2302187/R2-2213328), RAN2 asked some questions on differentiation of Layer2 ID and Coexistence of U2N/U2U. In this section, these questions are discussed and the answers are proposed.

Question1:

Can a UE be involved in acting as both U2N (as U2N Relay or Remote UE) and U2U (as U2U Relay or Remote UE) at the same time?

Regarding on whether a UE can act as a U2N Relay/Remote UE or U2U Relay/End UE, it depends on the UE’s 5G ProSe Capability as well as the configured/provisioned 5G ProSe policy/parameters. There is no restriction from specification point of view to prohibit a UE to act as the above roles.

Proposed Answer 1:

Yes. A UE can take any role as above based on its 5G ProSe Capability and configured/provisioned 5G ProSe policy/parameters.

Question2:

Can the L2ID used for U2U communication be the same as the L2ID for U2N services?

The agreed TS 23.304 CR0174 (S2-2301565) includes the following NOTE:

NOTE:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication) in clause 6.4, 6.5 and 6.7, and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) with a different discovery model.

It can be seen from the above NOTE that, the self-selected Source Layer-2 IDs are different between U2U communication and U2N discovery/communication.

Proposed Answer 2:

No. The Layer-2 IDs used for 5G ProSe UE-to-UE Relay Communication would be different from the Layer-2 IDs used for 5G ProSe UE-to-Network Relay Discovery and Communication.

Question3a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay service and Non-relay service?

According to the referred NOTE in the Question2 discussion, it can also been seen that the self-selected Source Layer-2 ID for U2U Relay UE discovery/communication are different from the non-relay discovery/communication (i.e. non-relay service).

Proposed Answer 3a:

No. The Layer-2 ID of 5G ProSe UE-to-UE Relay used for 5G ProSe UE-to-UE Relay Discovery and Communication would be different from the Layer-2 IDs used for Non-relay discovery and communication.

Question3b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay service and Non-relay service?

According to the referred NOTE in the Question2 discussion, it can also been seen that the self-selected Source Layer-2 ID for U2U End UE discovery/communication are different from the non-relay discovery/communication (i.e. non-relay service).

Proposed Answer 3b:

No. The Layer-2 IDs of 5G ProSe End UE used for 5G ProSe UE-to-UE Relay Discovery and Communication would be different from the Layer-2 IDs used for Non-relay discovery and Communication.

Question4a:

Whether the Layer2 ID of the relay UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

According to the referred NOTE in the Question2 discussion, it can also been seen that the self-selected Source Layer-2 ID for U2U Relay UE discovery are different from U2U Relay UE communication.

Proposed Answer 4a:

No. The Layer-2 ID used for 5G ProSe UE-to-UE Relay Discovery would be different from the Layer-2 ID used for 5G ProSe UE-to-UE Relay Communication for a 5G ProSe UE-to-UE Relay.

Question4b:

Whether the Layer2 ID of the remote UE can be the same for U2U Relay communication and U2U Relay discovery transmission?

According to the referred NOTE in the Question2 discussion, it can also been seen that the self-selected Source Layer-2 ID for U2U End UE discovery are different from U2U End UE communication.

Proposed Answer 4b:

No. The Layer-2 ID used for 5G ProSe UE-to-UE Relay Discovery would be different from the Layer-2 ID used for 5G ProSe UE-to-UE Relay Communication for a 5G ProSe End UE.

- Proposal

It is proposed agree the above proposed answers as captured in the reply LS S2-2302568.