3GPP TSG-SA2 Meeting #155	S2-2302585

# Athens, Greece, 20th Feb 2023 - 24th Feb 2023   					 (revision of S2-2302002)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0604 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support the Maintenance of Federated Learning Process in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson], OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principle 5 and NOTE 2 listed in the conclusion in clause 8.8 of TR 23.700-81 as follows: “During the FL procedure, the NWDAF containing MTLF as FL server may trigger reselection, addition or removal of FL clients and may issue a new FL client discovery via NRF, based on local policy or status of FL clients, e.g. load, availability, capability, latency, accuracy, etc. FL clients can join or quit FL operation dynamically in the execution phase.” “NWDAF containing MTLF as FL Server can monitor the accuracy level of the trained model and exclude a Client NWDAF from the FL group …” Besed on these conclusions, the procedures to support maintenance of Federated Learning in FL execution phase are added to TS 23.288, including FL server triggers reselection, addition or removal of FL client(s), discovers FL client(s) via NRF, and FL client joins or quits FL operation dynamically. To address the following key issues left in clause 6.2C.2.Y Editor’s Note: The details of the judgement process is FFS. In the conclusion of KI#8 in TR 23.700-81, principle 5 stated that “During the FL procedure, the NWDAF containing MTLF as FL server may trigger reselection, addition or removal of FL clients and may issue a new FL client discovery via NRF, based on local policy or status of FL clients, e.g. load, availability, capability, latency, accuracy, etc. FL clients can join or quit FL operation dynamically in the execution phase.” Therefore, the judgement should be carried out based on the following criteras: FL client NWDAF working load, data set availability, capability (i.e. whether it have enough available computation resource to work as a FL client), accuracy of local model. Notice that the working load and computation capability can be indicated as computation capability. Proposal-1: FL server NWDAF should register to the NRF about its FL capablity in FL process fo the current analytic ID as a FL server NWDAF. Proposal-2: FL client NWDAF may report status of FL training including accuracy of local model, availability of local dataset, computation capability, etc to the FL server NWDAF for judgement. Proposal-3: The terminology of FL client NWDAF and FL server NWDAF is updated to align with the description part of clause 5.X (S2-2301460) agreed in #154AH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure to TS 23.288 to support FL server to reselect FL client(s), discover FL client(s) via NRF, and to support FL client to join or quit Federated Learning operation dynamically. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for support maintenance of Federated Learning process in FL execution phase will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new) * * *

6.2C.2.Y Procedures for Maintenance of Federated Learning Process

This clause specifies how to maintain a Federation Learning process in FL execution phase, including FL Server NWDAF triggers reselection, addition or removal of FL Client NWDAF(s), discovers new FL Client NWDAF(s) via NRF, and FL Client NWDAF(s) joins or quits Federated Learning operation dynamically.

In Federated Learning execution phase, FL Server NWDAF monitors the status changes of FL Client NWDAF(s), and may reselects FL Client NWDAF(s) based on the updated status, availability, and/or capability, etc.

NOTE 1: FL Server NWDAF checks if there is a need to carry on the FL execution phase and then reselects FL members for the next iteration if needed.

Figure 6.2C.2.Y-1: Procedure of FL Server NWDAF reselects FL Client NWDAF(s), FL Client NWDAF(s) Join or Quit Federated Learning Operation Dynamically in Federated Learning execution phase

The procedure for FL Server NWDAF reselecting FL Client NWDAF(s), FL Client NWDAF(s) joining or quiting Federated Learning operation dynamically is as follows:

0.	FL Server NWDAF registers into NRF about the Federated Learning procedure with the following parameters:

-	Analytics ID.

- 	FL capability type (i.e. FL server)

1a.	FL Server NWDAF may get the updated status of current FL Client NWDAF(s) via NRF by using Nnrf_NFManagement service (as in clause 5.2.7.2 of TS 23.502) in the Federated Learning execution phase.

FL Server NWDAF may subscribe to NRF for notifications of status changes of the current NWDAF(s) (FL Client NWDAFs 1…N) by invoking an Nnrf_NFManagement_NFStatusSubscribe service operation. NRF notifies the FL Server NWDAF the status changes of the current FL Client NWDAF(s) by invoking Nnrf_NFManagement_NFStatusNotify service operation(s).

The status of a current FL Client NWDAF could be availability changes, capability changes (e.g., it will not support FL anymore, etc.).

1b. The current FL Client NWDAF(s) may inform FL Server NWDAF to quit the Federated Learning operation.

1c. FL Server NWDAF may get the information of the new FL Client NWDAF(s) dynamically via NRF by subscribing to the event that a new FL Client NWDAF registers (Nnrf_NFManagement _NFStatusSubscribe service as in clause 5.2.7.2 of TS 23.502).

1d. NWDAF may subscribe for NF load analytics of the FL Client NWDAF(s).

1e. FL Client NWDAF(s) may report status of FL training including accuracy of local model, availablility of local dataset, computation capability, etc.

2.	FL Server NWDAF checks FL Client NWDAF(s) status based on the received information, judges whether reselection of FL Client NWDAF(s) for the next round(s) of Federated Learning is needed. The judgement is based on the updated status of the FL Client NWDAF(s), including the availability, capability, status of FL training, and etc.

3.	[If re-selection is needed as judged in step 2.] If step 1c is not performed, FL Server NWDAF may discover new candidate FL Client NWDAF(s) via NRF by using Nnrf_NFDiscovery services as in clause 5.2.7.3 of TS 23.502. FL Server NWDAF reselects FL Client NWDAF(s) from the current FL Client NWDAF(s) and the new candidate FL Client NWDAF(s) (found in steps 1c or 3). For the new candidate FL Client NWDAF(s), the interaction between FL Server NWDAF and FL Client NWDAF(s) is same as the selection procedure described in clause 6.2C.2.1.

4.	FL Server NWDAF sends termination request to the FL Client NWDAF(s) which will be removed from the Federated Learning process optionally indicating the reason. FL Client NWDAF(s) terminates Federated Learning operations if receive termination request from the FL Server NWDAF and may performe further action to be qualitifed in participation of FL training in the next cycles.

Editor’s Note: The further actions by Client NWDAF is FFS.

Editor’s Note: What service will be used by Server and Client NWDAFs to terminate Federated Learning operation is FFS.

* * * End of Changes * * *