3GPP TSG-WG SA2 Meeting #155	S2-2302888

# Athens, Greece, February 20 – 24, 2023		(revision of S2-2300988)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0225 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Principles for applying parameters for 5G ProSe UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution proposes to add the principles for applying parameters for 5G ProSe UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Principles for applying parameters for 5G ProSe UE-to-UE Relay is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Principles for applying parameters for 5G ProSe UE-to-UE Relay is not supportted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.x.2 (new), 5.1.x.2.1 (new), 5.1.x.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

#### 5.1.x.2	Principles for applying parameters for 5G ProSe UE-to-UE Relay

##### 5.1.x.2.1	Principles for applying parameters for ProSe UE-to-UE Relay discovery

For 5G ProSe UE-to-UE Relay discovery over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for 5G ProSe UE-to-UE Relay discovery, without the need for the UEs to connect to the 5GC to get this initial configuration. The following applies:

-	The provisioning parameters for 5G ProSe UE-to-UE Relay discovery could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

The UE shall use radio resources for 5G ProSe UE-to-UE Relay discovery as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe UE-to-UE Relay discovery the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for 5G ProSe UE-to-UE Relay discovery, the UE shall not perform 5G ProSe UE-to-UE Relay discovery message transmission and reception on radio resources operated by this cell;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe UE-to-UE Relay discovery that are not operated by the UE's serving cell, as specified in clause 5.1.x.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for 5G ProSe UE-to-UE Relay discovery to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for 5G ProSe UE-to-UE Relay discovery, the UE shall not perform 5G ProSe UE-to-UE Relay discovery message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for 5G ProSe UE-to-UE Relay discovery and provides radio resources for 5G ProSe UE-to-UE Relay discovery then the UE shall perform PLMN selection triggered by 5G ProSe UE-to-UE Relay discovery as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for 5G ProSe UE-to-UE Relay discovery the UE shall not use 5G ProSe UE-to-UE Relay discovery;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize 5G ProSe UE-to-UE Relay discovery, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for 5G ProSe UE-to-UE Relay discovery if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe UE-to-UE Relay discovery, according to TS 38.331 [16] and as specified in clause 5.1.x.1, then the UE shall perform 5G ProSe UE-to-UE Relay discovery using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize 5G ProSe UE-to-UE Relay discovery, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe UE-to-UE Relay discovery when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for 5G ProSe UE-to-UE Relay discovery based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (5G ProSe UE-to-UE Relay discovery) is available. This is to cover the scenario when e.g. the radio resources used for 5G ProSe UE-to-UE Relay discovery are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 38.331 [16], a UE can be instructed by its serving cell to perform 5G ProSe UE-to-UE Relay discovery over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for 5G ProSe UE-to-UE Relay discovery when the UE attempts to use a carrier frequency configured for 5G ProSe UE-to-UE Relay discovery, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The 5G ProSe UE-to-UE Relay discovery is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

##### 5.1.x.2.2	Principles for applying parameters for 5G ProSe UE-to-UE Relay communication

For 5G ProSe UE-to-UE Relay communication over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for 5G ProSe UE-to-UE Relay communication, without the need for the UEs to connect to the 5GC to get this initial configuration. The following applies:

-	The provisioning parameters for 5G ProSe UE-to-UE Relay communication could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

The UE shall use radio resources for 5G ProSe UE-to-UE Relay communication as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe UE-to-UE Relay communication the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for 5G ProSe UE-to-UE Relay communication, the UE shall not perform 5G ProSe UE-to-UE Relay communication message transmission and reception on radio resources operated by this cell;

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe UE-to-UE Relay communication that are not operated by the UE's serving cell, as specified in clause 5.1.4.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13]; and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for 5G ProSe UE-to-UE Relay communication to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for 5G ProSe UE-to-UE Relay communication, the UE shall not perform 5G ProSe UE-to-UE Relay communication message transmission and reception on those radio resources;

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for 5G ProSe UE-to-UE Relay communication and provides radio resources for 5G ProSe UE-to-UE Relay communication then the UE shall perform PLMN selection triggered by 5G ProSe UE-to-UE Relay communication as defined in TS 23.122 [14];

-	If the UE finds such cell but not in a PLMN authorized for 5G ProSe UE-to-UE Relay communication the UE shall not use 5G ProSe UE-to-UE Relay communication;

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize 5G ProSe UE-to-UE Relay communication, then the UE is not authorized to transmit;

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) provisioned in the ME or the UICC for 5G ProSe UE-to-UE Relay communication if the UICC indicates it is authorized;

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe UE-to-UE Relay communication, according to TS 38.331 [16] and as specified in clause 5.1.4.1, then the UE shall perform 5G ProSe UE-to-UE Relay communication using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize 5G ProSe UE-to-UE Relay communication, then the UE is not authorized to transmit;

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for 5G ProSe UE-to-UE Relay communication when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas;

NOTE 2:	It is possible for a UE to use other radio resources for 5G ProSe UE-to-UE Relay communication based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (5G ProSe UE-to-UE Relay communication) is available. This is to cover the scenario when e.g. the radio resources used for 5G ProSe UE-to-UE Relay communication are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 38.331 [16], a UE can be instructed by its serving cell to perform 5G ProSe UE-to-UE Relay communication over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for 5G ProSe UE-to-UE Relay communication when the UE attempts to use a carrier frequency configured for 5G ProSe UE-to-UE Relay communication, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The 5G ProSe UE-to-UE Relay communication is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

# * * * * End of changes * * * *