3GPP TSG-SA2 Meeting #155	S2-2302910

# Athens, Greece, 20th Feb 2023 - 24th Feb 2023								Revision of S2-2301587

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0616 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#7 - 23.288 CR for adding UE Transmission Latency Performance Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, Nokia Shanghai Bell, OPPO, SK Telecom, Samsung], CATT, [ETRI] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposal to update TS 23.288 to add a new analytic as concluded in TR 23.700-80 KI#7 5GS Assistance to Federated Learning Operation, where the conclusions state the following: AF can request NWDAF (via NEF if needed) to provide existing, enhanced or new analytics to assist with federated learning operation. NWDAF Analytics services will be enhanced as follows: - New latency performance analytics are introduced for NWDAF. NOTE 1: The specific input and output parameters for the latency performance analytics among the group of UEs to assist the Application AI/ML FL operation will be determined during the normative phase. It is concluded that the AF as a consumer can request NWDAF to get a new Analytics ID on UE latency performance analytics. Updates at SA2#155: The following EN was resolved: Editor's note: It is FFS how NWDAF will use those parameters: expected number of repeating data transmissions and expected time interval between data transmissions. Editor's note: It is FFS if this analytics needs more input data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new clause under 6.7 to define a new analytic on UE latency performance analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-88 (AIMLsys) KI#7 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1 , 6.7.x (new), 6.7.x.1 (new), 6.7.x.2 (new), 6.7.x.3 (new), 6.7.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_AIMLsys |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** Start of Changes ***************

### 6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics;

-	Abnormal behaviour related network data analytics;

-	UE latency performance analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3] for a group of UEs or a specific UE, by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

Depending on local regulations, the NWDAF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the NWDAF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training.

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

********** Next Change (all new text) ***************

6.7.x UE latency performance analytics

6.7.x.1 General

The clause 6.7.x describes how NWDAF can provide UE latency performance analytics, in the form of statistics or predictions or both, to a service consumer. NWDAF collects UE latency performance related input data from 5GC NFs, OAM and AF. The consumer can either subscribe to analytics notifications (i.e., a Subscribe-Notify model) or request a single notification (i.e., a Request-Response model).

The UE latency performance refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE.  If an expected number of repeating data transmissions or an expected time interval between data transmissions is given, the UE latency performance can be provided as an average value of every data packet transmission latency performance within the Analytics target period. The UE latency performance analytics may be used to assist an AF hosting AI/ML-based services, e.g., for member selection of federated learning.

The UE latency performance analytics may be provided as defined in clause 6.7.x.3 for a UE individually or a list of UEs.

The service consumer may be an NF (e.g. AF, or NEF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Latency Performance".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a group of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the UE latency performance analytics to the provided area;

-	An optional list of analytics subsets that are requested (see clause 6.7.x.3);

-	Data Volume UL/DL: indicates a specific data volume transmitted once from UE to AF, or from AF to UE;

-	QoS requirements (e.g., 5QI, QoS Characteristics);

-	Optionally, an expected number of repeating data transmissions within the Analytics target period;

-	Optionally, an expected time interval between data transmissions

-	Optionally, a request for geographical distribution (i.e., the AoIs) of the UEs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optional preferred order of results for the list of UE latency performance:

-	ordering criterion: "UE latency performance",

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which indicate conditions on the level to be reached for respective analytics subsets (see clause 6.7.X.3) in order to be notified by the NWDAF; e.g., NWDAF may provide the percentage of UEs that have reached certain Reporting Threshold(s).

-	Optionally, maximum number of UEs;

6.7.x.2 Input Data

The NWDAF supporting analytics on UE latency performance shall be able to collect UE performance information from AF, OAM and 5GC NFs.

More detailed information collected by the NWDAF from the OAM is defined in the Table 6.7.x.2-1, and from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.7.x.2-2.

Table 6.7.x.2-1: Input data from OAM related to UE latency performance

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM (see NOTE 1) | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM (see NOTE 1) | The per UE measurement of the received radio quality in a network cell the specified area where UE(s) will be located, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM (see NOTE 1) | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20] |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clauses 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clauses 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clauses 5.4.1.1 of TS 37.320 [20] |
| Distribution of UL/DL/round trip packed delay between PSA UPF and UE | OAM | The distribution of UL/DL/round trip packet delay between PSA UPF and UE as captured in TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.7.X.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s))

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.7.x.2-2: Service Data from 5GC NFs for UE latency performance analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be selected via AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be selected via GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions; |
| Transmission start Timestamp | UPF | The start timestamp of the collected information. |
| Transmission end Timestamp | UPF | The end timestamp of the collected information. |
| Transmitted data size | UPF | The size of the transmitted data. |

6.7.x.3 Output Analytics

The NWDAF supporting UE latency performance analytics provides the analytics results to consumer NFs, e.g., AF, or NEF. The analytics results provided by the NWDAF could be UE latency performance statistics as defined in Table 6.7.X.3-1 or predictions as defined in table 6.7.x.3-2.

Table 6.7.x.3-1: UE Latency performance statistics

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| UE latency performances (1…max) | List of latency performance per UE Max. is the number of UEs, if applicable. |
| > Application ID | Identifies the application in use during the time slot |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > UE location | Indicating the UE location information when the UE service is delivered. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies |
| > Latency Performance | Latency Performance indicators Statistics of UE latency performance over the Analytics target period (e.g. average, variance). This information refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If an expected number of repeating data transmissions or an expected time interval between data transmissions is given in the request, the UE transmission latency performance can be provided as an average value of every data transmission latency performance within the Analytics target period, which is also provided in the request. |
| >> UL packet delay (NOTE 2) | The UL packet delay for the UE communicating with the application (e.g. average, variance). |
| >> DL packet delay (NOTE 2) | The DL packet delay for the UE communicating with the application (e.g. average, variance). |
| >> Round trip packet delay (NOTE 2) | The round trip packet delay for the UE communicating with the application (e.g. average, variance). |
| Aggregate latency performances (NOTE 2) | Aggregated latency performance statistics for multiple UEs |
| > Latency classes (1…max) (NOTE 3) | List with group of UEs classified by ranges of latency performance |
| >> Ratio/percentage of UEs per latency class | Percentage or ratio of UEs |
| >> Aggregate UL/DL/round trip packet delay per latency class | Aggregated statistical values of the corresponding UE latency statistics in the latency class (e.g. average, variance). |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information |
| > Maximum Packet Delay (NOTE 2) | Maximum packet delay for observed for UEs communicating with the application. |
| > Spatial Validity Condition | Area where the UE latency performance analytics applies. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: The number of latency classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.7.x.3-2: UE latency performance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| UE latency performances (1…max) | List of transmission latency performance per UE. Max. is the number of UEs, if applicable. |
| > Application ID | Identifies the application in use during the time slot |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info (NOTE 1) | The UPF ID/address/FQDN information for the involved anchor UPF. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > UE location | Indicating the UE location information when the UE service is delivered. |
| > DNN | DNN for the PDU Session which contains the QoS flow. |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Spatial validity | Area where the UE latency performance statistics applies |
| > Latency performance | Latency Performance indicators Predictions of UE transmission latency performance over the Analytics target period (e.g. average, variance). This information refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If an expected number of repeating data transmissions or an expected time interval between data transmissions is given in the request, the UE transmission latency performance can be provided as an average value of every data transmission latency performance within the Analytics target period, which is also provided in the request. |
| >> UL packet delay (NOTE 2) | The UL packet delay for the UE communicating with the application (e.g., average, variance). |
| >> DL packet delay (NOTE 2) | The DL packet delay for the UE communicating with the application (e.g., average, variance). |
| >> Round trip packet delay (NOTE 2) | The round trip packet delay for the UE communicating with the application (e.g. average, variance). |
| Aggregate latency performances (NOTE 2) | Aggregated transmission latency performance statistics for multiple UEs |
| > Latency classes (1…max) (NOTE 3) | List with group of UEs classified by ranges of latency performance |
| >> Ratio/percentage of UEs per latency class | Percentage or ratio of UEs |
| >> Aggregate UL/DL/round trip packet delay per latency class | Aggregated statistical values of the corresponding UE latency statistics in the latency class (e.g. average, variance). |
| > Validity period | The validity period for the UE latency performance statistics as defined in clause 6.1.3. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information |
| > Maximum Packet Delay (NOTE 2) | Maximum packet delay for observed for UEs communicating with the application. |
| > Spatial Validity Condition | Area where the UE latency performance analytics applies. |
| Confidence | Confidence of this prediction. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: The number of latency classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Editor's note:	It is FFS if Aggregate latency performances can be output of this analytics.

Editor's note: The relation between the latency performance outputs and the Time slot entry is FFS

6.7.x.4 Procedures

The NWDAF can provide UE latency performance analytics to a 5GC NF (e.g., AF, or NEF).

Figure 6.7.x.4-1: Procedure for UE latency performance analytics

1.	The Consumer NF, e.g., AF, or NEF, requests or subscribes to analytics for UE latency performance analytics from NWDAF (possibly via NEF in case the consumer NF is AF) and provides the input information as specified in 6.7.x.1 to 5GC.

2a-b.	The NWDAF subscribes the service data from AMF in Table 6.7.x.2-2 using Namf_EventExposure_Subscribe service for collecting UE location(s) for a UE or a group of UEs.

NOTE:	If NWDAF requires UE location information with finer granularity than TA/cell, then NWDAF collects the location data from GMLC instead of AMF.

2c.	NWDAF subscribes to service data from SMF in Table 6.7.x.2-2 by invoking Nsmf_EventExposure_Subscribe (Event ID, SUPI(s) or Application ID).

In order to provide the requested analytics, the NWDAF subscribes to information of the UE and may subscribe to N4 Session related input data from SMFs as defined in Table 6.7.x.2-2.

2d-e.	N4 related input data is provided by UPF to SMF.

2f.	SMF provides the requested input data to NWDAF.

2g-h.	The NWDAF may subscribe to input data in Table 6.7.x.2-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

3.	The NWDAF derives requested analytics, in the form of latency performance statistics or predictions or both.

4.	The NWDAF provides requested latency performance analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the NF subscribed to UE latency performance analytics at step 1, when the NWDAF generates new analytics, it notifies the new generated analytics to the consumer.

********** Next Change ***************

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| UE Latency Performance | Analytics ID: UE Latency Performance | Analytics on UE latency performance |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

********** End of Changes ***************